3GPP TSG-RAN WG2 Meeting #119e	R2-2207929

Electronic, 15th Aug– 26th Aug, 2022

Title:		Text Proposal for RSRP-based TA validation

Source:	Huawei, HiSilicon

Agenda item:	8.2.4

Document for:	Discussion and Decision

- Introduction

In the previous R2 meeting R2#118, the following has been agreed:

12 For RSRP-based TA validation that there is no need for a condition for “if measObject is configured”

This paper discusses the issues related to this agreement and propose a text proposal for resolving the issue.

- Discussion

2.1 R4 discussion

During R2#117, the LS R2-2204269 has been sent to R4 in order to trigger the R4 discussion for TA update for CG-SDT.

During the last R4 meeting, there has been discussion on this and the update for the TS 38.133 for the requirements for CG-SDT has been captured as follows as in the version h60:

### 5.5.3 TA validation requirements When cg-SDT-RSRP-ChangeThreshold [TS 38.331] is configured for TA validation based on the RSRP change criterion according to clause 5.8.2.x in [TS 38.321], the UE is allowed to transmit using CG-SDT using the timing derived using the latest available NTA value as specified in subclause 7.1 provided that - the first RSRP (RSRP1) measurement and the second RSRP (RSRP2) measurements used in the TA validation are valid measurements and, - timing alignment validation for transmission using CG-SDT is valid according to the validation criteria in clause 5.8.2 in [TS 38.321]. RSRP1 and RSRP2 are considered valid provided that the conditions in Table 5.5.3-1 and Table 5.5.3-2 are met for FR1 and FR2-1. Table 5.5.3-1 Valid measurement for FR1 Measurement; FR1. RSRP1; (T1 – min(640ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(640ms, M1*TDRX)). RSRP2; (T2 – min(640ms, M1*TDRX)) ≤ T2’ ≤ T2. Table 5.5.3-2 Valid measurement for FR2-1 Measurement; FR2-1. RSRP1; (T1 – [max{480ms, 8*SMTC periodicity}]) ≤ T1’ ≤ (T1 + [max{480ms, 8*SMTC periodicity}]). RSRP2; (T2 – [max{480ms, 8*SMTC periodicity}]) ≤ T2’ ≤ T2. If at least one of RSRP1 and RSRP2 is considered to be invalid based on the above conditions, then the UE shall not validate the CG-SDT using RSRP1 and RSRP2 and shall not transmit using CG-SDT. Additionally, the UE shall not transmit in an CG-SDT occasion that occurs more than [640 ms] after T2. Where: - T1 is the time when - RRCRelease with CG-SDT configuration (TS 38.331 [2]) is received when changing from RRC_CONNECTED to RRC_INACTIVE state - the latest MAC CE TA command is received if TA command is received while in RRC_INACITVE state. - T1’ is the time when the UE has completed RSRP1. - T2 is the time when the UE performs TA validation as defined in clause 5.27.2 in (TS 38.321 [7]) for transmission using CG-SDT. - T2’ is the time when the UE has completed RSRP2. - TDRX is the DRX cycle length in ms. - M1 the scaling factor as defined in clause 4.2.2.2.

Take the case for FR1 as example, the condition in T1’ means that the UE has to finish measurement for RSRP1 within the time define as (T1 – min(640ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(640ms, M1*TDRX)). The time T1 is denoted as the time when (a) RRCRelease message is received when the UE moves from RRC_CONNECTED to RRC_INACTIVE. or (b) the time when TAC in MAC CE is received. It should be noted that R4 is also discussing whether to support the RSRP update when the UE receives RRCRelease message between CG-SDT procedures, i.e., when the UE transits from RRC_INACTIVE with on-going CG-SDT procedure to RRC_INACTIVE without CG-SDT procedure as can be seen in the following.

2.3 R2 aspects

During R2#118, the following agreement in R2 has been agreed on the use of measObject for TA derivation when RRCRelease message is received. The opinion is that measObject is always configured when the UE is released from RRC_CONNECTED to RRC_INACTIVE.

- For RSRP-based TA validation that there is no need for a condition for “if measObject is configured”

With the above agreement, the text for TA validation has been updated as follows. The text means that the UE needs to update the pathloss reference with the measObject configuration for the serving cell, whenever CG-SDT configuration is received.

### 5.27.2 TA Validation for CG-SDT RRC configures the following parameters for validation for CG-SDT: - cg-SDT-RSRP-ChangeThreshold: RSRP threshold for the increase/decrease of RSRP for time alignment validation. The MAC entity shall, upon the reception of CG-SDT configuration: 1> store the RSRP of the downlink pathloss reference derived based on the measObject configured for the Serving Cell as in TS 38.331 [5]. The MAC entity shall consider the TA of the initial CG-SDT transmission with CCCH message to be valid when the following conditions are fulfilled: 1> The RSRP values for the stored downlink pathloss reference and the current downlink pathloss reference are valid according to TS 38.133 [11]; and 1> Compared to the stored downlink pathloss reference RSRP value, the current RSRP value of the downlink pathloss reference calculated as specified in TS 38.133 [11] has not increased/decreased by more than cg-SDT-RSRP-ChangeThreshold, if configured; and 1> cg-SDT-TimeAlignmentTimer is running.

But the issue with the above agreement is that it does not consider the case when the UE receives RRCRelease with CG-SDT configuration during an on-going SDT procedure. In this case, there is no measObject configured for the serving cell.

With the discussion in Section 2.2 above on the current R4 discussion, there are two options

- Option1: RSRP is not updated when RRCRelease message is received without TAC MAC CE during SDT procedure and CG-SDT is configured.

- Option2: RSRP is updated whenever the UE receives RRCRelease in RRC_INACTIVE and CG-SDT is configured.

For Option1, we think the agreement above should be reverted that the measObject is only applicable when the UE moves from RRC_CONNECTED to RRC_INACTIVE with CG-SDT configuration. Then, for the case when UE receives RRCRelease during an on-going SDT procedure to, there is no measObject configuration, and RSRP will not be updated. However, there is another possibility that the CG-SDT configuration can be initially received in RRC_INACTIVE. For this case, the UE’s pathloss reference also needs to be derived. The measObject configuration should be obtained from the UE’s AS context.

For Option2, in order to obtain the RSRP, the UE also requires measObject to obtain the RSRP for the cell. We think that since for CG-SDT, the configuration only applies to the cell where it is received, hence the previous measObject configuration still can be used. Then, again, the UE should restore the measObject configuration from the Inactive AS context.

Based the discussion on the two possibilities for R4’s future progress, we propose the following:

Proposed 1: For the condition for updating pathloss reference for TA validation of CG-SDT,

- In RRC spec, UE should restore the measObject from the Inactive AS context. Adopt the TP in Clause 5.1

- In MAC spec, cite a reference to the spec TS 38.133 for when the pathloss reference should be updated

Furthermore, according to the current RRC spec, whenever sdt-MAC-PHY-CG-Config is received, the UE shall trigger the lower layer to start or restart the cg-SDT-TimeAlignmentTimer. This is needed for two cases: (a) when the UE is released from RRC_CONNECTED to RRC_INACTIVE; (b) when the UE is in RRC_INACTIVE and it receives CG-SDT configuration by RRCRelease message. Hence, the UE should start the CG-SDT TAT when the CG-SDT configuration is initially received

Proposal 2: Start/Restart the CG-SDT TAT from RRC’s perspective only when the UE initially receives the CG-SDT configuration in RRCRelease message. Adopt the TP in Clause 5.3.

- Summary

Based on the discussion above, it is proposed that:

Proposed 1: For the condition for updating pathloss reference for TA validation of CG-SDT,

- In MAC spec, cite a reference to the spec TS 38.133 for when the pathloss reference should be updated. Adopt the TP in Clause 5.1

- In RRC spec, UE should restore the measObject from the Inactive AS context. Adopt the TP in Clause 5.2

Proposal 2: Start/Restart the CG-SDT TAT from RRC’s perspective only when the UE initially receives the CG-SDT configuration in RRCRelease message. Adopt the TP in Clause 5.2.

- References

- TS 38.331, Radio Resource Control, 3GPP

- Text proposal

5.1 Text proposal 2 for TS 38.321

### 5.27.2 TA Validation for CG-SDT RRC configures the following parameters for validation for CG-SDT: - cg-SDT-RSRP-ChangeThreshold: RSRP threshold for the increase/decrease of RSRP for time alignment validation. The MAC entity shall, upon the reception of CG-SDT configuration, according to the conditions in TS 38.133 [11] for RSRP measurement: 1> store the RSRP of the downlink pathloss reference derived based on the measObject configured for the Serving Cell as in TS 38.331 [5]. The MAC entity shall consider the TA of the initial CG-SDT transmission with CCCH message to be valid when the following conditions are fulfilled: 1> The RSRP values for the stored downlink pathloss reference and the current downlink pathloss reference are valid according to TS 38.133 [11]; and 1> Compared to the stored downlink pathloss reference RSRP value, the current RSRP value of the downlink pathloss reference calculated as specified in TS 38.133 [11] has not increased/decreased by more than cg-SDT-RSRP-ChangeThreshold, if configured; and 1> cg-SDT-TimeAlignmentTimer is running.

5.2	Text proposal 3 for TS 38.331

#### 5.3.8.3 Reception of the RRCRelease by the UE The UE shall: 1> delay the following actions defined in this clause 60 ms from the moment the RRCRelease message was received or optionally when lower layers indicate that the receipt of the RRCRelease message has been successfully acknowledged, whichever is earlier; 1> stop timer T380, if running; 1> stop timer T320, if running; 1> if timer T316 is running; 2> stop timer T316; 2> clear the information included in VarRLF-Report, if any; 1> stop timer T350, if running; 1> stop timer T346g, if running; 1> if the AS security is not activated: 2> ignore any field included in RRCRelease message except waitTime; 2> perform the actions upon going to RRC_IDLE as specified in 5.3.11 with the release cause 'other' upon which the procedure ends; 1> if the RRCRelease message includes redirectedCarrierInfo indicating redirection to eutra: 2> if cnType is included: 3> after the cell selection, indicate the available CN Type(s) and the received cnType to upper layers; NOTE 1: Handling the case if the E-UTRA cell selected after the redirection does not support the core network type specified by the cnType, is up to UE implementation. 2> if voiceFallbackIndication is included: 3> consider the RRC connection release was for EPS fallback for IMS voice (see TS 23.502 [43]); 1> if the RRCRelease message includes the cellReselectionPriorities: 2> store the cell reselection priority information provided by the cellReselectionPriorities; 2> if the t320 is included: 3> start timer T320, with the timer value set according to the value of t320; 1> else: 2> apply the cell reselection priority information broadcast in the system information; 1> if deprioritisationReq is included and the UE supports RRC connection release with deprioritisation: 2> start or restart timer T325 with the timer value set to the deprioritisationTimer signalled; 2> store the deprioritisationReq until T325 expiry; NOTE 1a: The UE stores the deprioritisation request irrespective of any cell reselection absolute priority assignments (by dedicated or common signalling) and regardless of RRC connections in NR or other RATs unless specified otherwise. 1> if the RRCRelease includes the measIdleConfig: 2> if T331 is running: 3> stop timer T331; 3> perform the actions as specified in 5.7.8.3; 2> if the measIdleConfig is set to setup: 3> store the received measIdleDuration in VarMeasIdleConfig; 3> start timer T331 with the value set to measIdleDuration; 3> if the measIdleConfig contains measIdleCarrierListNR: 4> store the received measIdleCarrierListNR in VarMeasIdleConfig; 3> if the measIdleConfig contains measIdleCarrierListEUTRA: 4> store the received measIdleCarrierListEUTRA in VarMeasIdleConfig; 3> if the measIdleConfig contains validityAreaList: 4> store the received validityAreaList in VarMeasIdleConfig; 1> if the RRCRelease includes suspendConfig: 2> reset MAC and release the default MAC Cell Group configuration, if any; 2> apply the received suspendConfig except the received nextHopChainingCount; 2> if the sdt-Config is configured: 3> for each of the DRB in the sdt-DRB-List: 4> consider the DRB to be configured for SDT; 3> if sdt-SRB2-Indication is configured: 4> consider the SRB2 to be configured for SDT; 3> for each RLC bearer that is not suspended: 4> re-establish the RLC entity as specified in TS 38.322 [4]; 3> for SRB2 (if it is resumed) and for SRB1: 4> trigger the PDCP entity to perform SDU discard as specified in TS 38.323 [5]; 3> if sdt-MAC-PHY-CG-Config is configured: 4> restore the measObject configuration for the serving cell within the UE's Inactive AS context; 4> if sdt-MAC-PHY-CG-Config was not configured before the reception of the RRCRelease message: 5> configure the PCell with the configured grant resources for SDT and instruct the MAC entity to start the cg-SDT-TimeAlignmentTimer; 2> if srs-PosRRC-Inactive is configured: 3> apply the configuration and instruct MAC to start the inactivePosSRS-TimeAlignmentTimer; 2> remove all the entries within VarConditionalReconfig, if any; 2> for each measId of the MCG measConfig and for each measId of the SCG measConfig, if configured, if the associated reportConfig has a reportType set to condTriggerConfig: 3> for the associated reportConfigId: 4> remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig; 3> if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig: 4> remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig; 3> remove the entry with the matching measId from the measIdList within the VarMeasConfig; 2> re-establish RLC entities for SRB1; 2> if the RRCRelease message with suspendConfig was received in response to an RRCResumeRequest or an RRCResumeRequest1: 3> stop the timer T319 if running; 3> in the stored UE Inactive AS context: 4> if timer T319a is running: 5> replace the stored sdt-Config with the one received in the RRCRelease message; 4> replace the KgNB and KRRCint keys with the current KgNB and KRRCint keys; 4> replace the nextHopChainingCount with the value of nextHopChainingCount received in the RRCRelease message; 4> replace the cellIdentity with the cellIdentity of the cell the UE has received the RRCRelease message; 4> if the suspendConfig contains the sl-UEIdentityRemote (i.e. the UE is a L2 U2N Remote UE): 5> replace the C-RNTI with the value of the sl-UEIdentityRemote; 5> replace the physical cell identity with the value of the sl-PhysCellId in sl-ServingCellInfo contained in the discovery message received from the connected L2 U2N Relay UE; 4> else: 5> replace the C-RNTI with the C-RNTI used in the cell (see TS 38.321 [3]) the UE has received the RRCRelease message; 5> replace the physical cell identity with the physical cell identity of the cell the UE has received the RRCRelease message; 3> replace the nextHopChainingCount with the value associated with the current KgNB; 3> stop the timer T319a if running; 2> else: 3> store in the UE Inactive AS Context the nextHopChainingCount received in the RRCRelease message, the current KgNB and KRRCint keys, the ROHC state, the stored QoS flow to DRB mapping rules, the application layer measurement configuration, the C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell, the spCellConfigCommon within ReconfigurationWithSync of the NR PSCell (if configured) and all other parameters configured except for: - parameters within ReconfigurationWithSync of the PCell; - parameters within ReconfigurationWithSync of the NR PSCell, if configured; - parameters within MobilityControlInfoSCG of the E-UTRA PSCell, if configured; - servingCellConfigCommonSIB; - sl-L2RelayUE-Config, if configured; - sl-L2RemoteUE-Config, if configured; 3> store any previously or subsequently received application layer measurement reports for which no segment, or full message, has been submitted to lower layers for transmission; NOTE 2: NR sidelink communication related configurations and logged measurement configuration are not stored as UE Inactive AS Context, when UE enters RRC_INACTIVE. 2> suspend all SRB(s) and DRB(s) and multicast MRB(s), except SRB0; 2> indicate PDCP suspend to lower layers of all DRBs and multicast MRBs; 2> if the t380 is included: 3> start timer T380, with the timer value set to t380; 2> if the RRCRelease message is including the waitTime: 3> start timer T302 with the value set to the waitTime; 3> inform upper layers that access barring is applicable for all access categories except categories '0' and '2'; 2> if T390 is running: 3> stop timer T390 for all access categories; 3> perform the actions as specified in 5.3.14.4; 2> indicate the suspension of the RRC connection to upper layers; 2> enter RRC_INACTIVE and perform cell selection as specified in TS 38.304 [20]; 1> else 2> perform the actions upon going to RRC_IDLE as specified in 5.3.11, with the release cause 'other'.