3GPP TSG-RAN WG2 Meeting #119 electronic	R2-207949

# Online, 17th – 29th August, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0521 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the application layer measurement configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE-Core |  |  |  |  |  | Date: |  |  | 2022-07-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - RAN3#112-e agreed that: MR-DC support is out of the scope of R17. In cosequence only the MN can configure the QoE measurement in Rel-17 for the UE configured with MR-DC. The current specification does not clarify this. - RAN2 also agreed that the UE retransmits application layer measurement reports in the target node if a handover occurs during the transmission of the report. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In section 21.2.1, it is clarified that only MN can configure QoE for the UE. - In section 21.3, it is clarified that the UE retransmits application layer measurement reports in the target node if a handover occurs during the transmission of the report. Impact analysis Impacted 5G architecture options: MR-DC Impacted functionality: QoE measurement collection Inter-operability: If NW implements according to this CR but UE does not, there is no inter-operability issue. If UE implements according to this CR but NW does not, the UE cannot perform the RRC reconfiguration with QoE measurement provided by for SCG which will cause the SCG failure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The applicability of QOE for MR-DC remains unbclear and configuration failures may happen. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 21.2.1, 21.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Beginning of changes

21.2	QoE Measurement Configuration

21.2.1	QoE Measurement Collection Activation and Reporting

The feature is activated in the gNB either by direct configuration from the OAM system (management-based activation), or by signalling from the OAM via the Core Network (signalling-based activation), containing UE-associated QoE configuration. One or more QoE measurement collection jobs can be activated at a UE per service type, and each QoE measurement configuration is uniquely identified by a QoE Reference. In this release, only the Master Node can activate the QoE measurement when the UE is configured with MR-DC.

For signalling-based QoE measurements, the OAM initiates the QoE measurement activation for a specific UE via the Core Network, and the gNB receives one or more QoE measurement configurations by means of UE-associated signalling. The QoE measurement configuration for signalling-based activation includes an application layer measurement configuration list and the corresponding information for QoE measurement collection, e.g., QoE Reference, service type, MCE IP Address, Slice Scope, Area Scope, MDT Alignment Information and the indication of available RAN visible QoE metrics.

For management-based QoE measurement activation, the OAM sends one or more the QoE measurement configurations directly to the gNB. The QoE measurement configuration for management-based activation also includes an application layer measurement configuration list and the corresponding information for QoE measurement collection. Each application layer measurement configuration is encapsulated in a transparent container. The gNB selects UE(s) that meet the required QoE measurement capability, Area Scope and Slice Scope.

Application layer measurement configuration received by the gNB from OAM or CN is encapsulated in a transparent container, which is forwarded to a UE as Application layer configuration in the RRCReconfiguration message (there can be multiple configurations in the same message). Application layer measurement reports received from UE's application layer are encapsulated in a transparent container and sent to the network in the MeasurementReportAppLayer message, as specified in TS 38.331 [12]. The UE can send multiple application layer measurement reports to the gNB in one MeasurementReportAppLayer message. In order to allow the transmission of application layer measurement reports which exceed the maximum PDCP SDU size, segmentation of the MeasurementReportAppLayer message may be enabled by the gNB. An RRC identifier conveyed in the RRC signalling is used to identify the application layer measurement configuration and report between the gNB and the UE. The RRC identifier is mapped to the QoE Reference in the gNB. and the gNB forwards the application layer measurement report is forwarded to MCE together with the QoE Reference ID. The gNB can release one or multiple application layer measurement configurations from the UE in one RRCReconfiguration message at any time. The UE may additionally be configured by the gNB to report when a QoE measurement session starts or stops for a certain application layer measurement configuration.

Next change

21.3	QoE Measurement Continuity for Mobility

The QoE measurement collection continuity for intra-system intra-RAT mobility is supported, with the Area Scope parameters configured by the OAM, where the network is responsible for keeping track of whether the UE is inside or outside the Area Scope. A UE continues an ongoing measurement even if it leaves the Area Scope, unless the network indicates to the UE to release the QoE configuration.

For RRC_CONNECTED state mobility, the source gNB may transmit the QoE measurement configuration(s) and/or the information related to the configuration(s) of a specific UE to the target gNB via XnAP or NGAP. For signalling-based QoE, QoE Reference, MCE IP Address, Measurement Configuration Application layer ID, MDT Alignment Information, Area Scope, Slice Scope and Measurement Status are passed to the target node. For management-based QoE, Measurement Configuration Application Layer ID, MCE IP Address and Measurement Status are passed to the target node. For RRC_INACTIVE state mobility, QoE measurement configuration(s) of a specific UE can be restored from the node hosting the UE context when it resumes to RRC_CONNECTED state. Multiple sets of QoE measurement configurations should be supported during mobility.

For signalling based QoE, at handover to a target gNB that supports QoE, the target gNB decides which of the application layer measurement configurations should be kept or released, e.g. based on application layer measurement configuration information received from the source gNB in Xn/NG signalling.

If a handover occurs during the transmission of the application layer measurement report or before the reception of the report is confirmed by the network, the UE re-transmits the report to the network after successful handover completion.

When the UE resumes the connection with a gNB that does not support QoE, the UE releases all application layer measurement configurations.