3GPP TSG RAN WG2 #119e												R2-2207963

Electronic meeting, August 15th –26th, 2022

Agenda Item:	8.9.3

Source: 	NEC

Title:	Considerations on Service Continuity Enhancement

Document for:	Discussion

- Introduction

In RAN#94, a work item on enhancements of NR sidelink relay was approved. Objectives for enhancements on NR sidelink relay includes study on service continuity enhancement in this work item as follows [1].

2. Specify mechanisms to enhance service continuity for single-hop Layer-2 UE-to-Network relay for the following scenarios [RAN2, RAN3]: A. Inter-gNB indirect-to-direct path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> gNB Y”) B. Inter-gNB direct-to-indirect path switching (i.e., “remote UE <-> gNB X” to “remote UE <-> relay UE A <-> gNB Y”) C. Intra-gNB indirect-to-indirect path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB X”) D. Inter-gNB indirect-to-indirect path switching (i.e., “remote UE<-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB Y”) Note 2A: Scenario D is to be supported by reusing solutions for the other scenarios without specific optimizations.

This contribution provides our initial considerations on service continuity enhancement.

- Discussion

During the Rel-17 WI stage on NR sidelink relay, the procedures for two scenarios, including intra-gNB direct-to-indirect path switching and intra-gNB indirect-to-direct path switching, are discussed and captured in the corresponding specifications[2] [3] which include:

- the configuration procedures and parameters on the addition and releasing of relay channels, such as releasing indirect path for intra-gNB indirect-to-direct path switching and adding indirect path for intra-gNB direct-to-indirect path switching, and

- parameters for the measurement and report of the path switching, such as trigger events corresponding to intra-gNB direct-to-indirect path switching and intra-gNB indirect-to-direct path switching.

2.1 Considerations on the Measurement of Indirect-to-indirect Path Switching

According to the scenarios of service continuity enhancement for Rel-18, indirect-to-indirect path switching is a scenario not covered by the Rel-17 issues. Therefore, to enable the measurement and report for this scenario, new trigger events need to be defined.

RAN2 to define new trigger events for indirect-to-indirect path switching.

2.2 Basic Procedures of Inter-gNB Scenarios

When considering the procedure of indirect-to-indirect path switching including the releasing of indirect path and the addition of another indirect path, the configuration procedures for these two scenarios of Rel-17 can be reused as much as possible to release one indirect path and to add another indirect path.

For indirect-to-indirect path switching, the configuration procedures on addition and releasing of relay channels defined in Rel-17 can be reused as much as possible.

When considering the procedures of inter-gNB scenarios:

- The source gNB and the target gNB need to exchange information about Relay UE(s), such as the serving cell of Relay UE(s) and the measurement results according to Relay UE(s).

- The legacy Handover Request message and Handover Request Acknowledge message can be adopted to exchange such information. And some indication such as message type specific for Relay UE should be included to indicate that this message is for the sidelink relay case. Alternatively, some new messages via Xn need to be specified to enable the exchange of information about Relay UE(s).

- It is straight forward that the target gNB can be in charge of the selection of the target Relay UE, as what we have approved for the selection of target cell for the legacy HO scenarios.

RAN2 to consider the following technical issues for scenarios involving inter-gNB path switching:

Which information is essential to enable the selection of the target Relay UE?

Request RAN3 to make a decision on whether to reuse legacy Xn messages or specify new Xn messages to exchange such information for the selection of the target Relay UE.

Which gNB is in charge of the selection of the target Relay UE?

2.3 Failure handling for path switching to an indirect path

During D2I or I2I path switching, the RRC layer of Remote UE may trigger the upper layer PC5 unicast link with the target Relay UE. The RRCReconfigurationComplete message can only be sent if this link is successfully established. However, the failure of the PC5 unicast link establishment may be detected. Thus, the RRCReconfigurationComplete message will not be sent, let alone sending it successfully to stop timer T420. Then, the failure of path switching can be determined once the PC5 unicast link is not established successfully, and Remote UE can solve the failure in time by executing the connection re-establishment procedure. It is valuable for inter-gNB scenarios, since more time is needed for the preparation of the target Relay UE between two gNBs and the channel condition between Relay UE and Remote UE may be changed during that long time period. So we propose:

RAN2 to consider the failure of PC5 unicast link establishment during path switching to trigger the RRC Reconfiguration failure procedure.

Conclusion

Observation 1:	For indirect-to-indirect path switching, the configuration procedures on addition and releasing of relay channels defined in Rel-17 can be reused as much as possible.

Proposal 1:	RAN2 to define new trigger events for indirect-to-indirect path switching.

Proposal 2:	RAN2 to consider the following technical issues for scenarios involving inter-gNB path switching:

−	Which information is essential to enable the selection of the target Relay UE?

−	Request RAN3 to make a decision on whether to reuse legacy Xn messages or specify new Xn messages to exchange such information for the selection of the target Relay UE.

−	Which gNB is in charge of the selection of the target Relay UE?

Proposal 3:	RAN2 to consider the failure of PC5 unicast link establishment during path switching to trigger the RRC Reconfiguration failure procedure.

- References

[1] RP-221262, WI on NR sidelink relay enhancements, LG.

[2] TS 38.300 V17.1.0

[3] TS 38.331 V17.1.0