3GPP TSG-RAN2#119-e	   	 					 			R2-2207979

Electronic meeting, August 2022

Source: 			ZTE Corporation, Sanechips

Title: 	Power Saving enhancements for XR

Agenda item:	8.5.3

Document for: Discussion and Decision

# Introduction

In this contribution we discuss power saving enhancements for XR.

# Power consumption and latency trade-off for XR

XR encompasses a variety of traffic profiles and the XR traffic characterised by periodicity with jitter, varying QoS requirements within the traffic and correlated traffic frames with a group of frames being dependent on each other for decoding and rendering purposes. The strict QoS requirements for XR pose a wide variety of challenges for RAN. One of the issues is the UE power consumption. There is always a trade-off between power consumption and latency and given the extreme requirements on QoS for XR, power saving gains will be essential to ensure that the power consumption at the UE is not disproportionately affected by XR traffic.

Observation 1: Power saving gains are essential to ensure that the power consumption at the UE is not disproportionately affected by the tight QoS requirements of XR traffic

# Enhancements for DL power saving for XR

One of the critical issues with XR traffic is the jitter. Whilst there are strict requirements for end-to-end delay for the XR packets, jitter in DL traffic may happen for various reasons:

- Jitter in frame generation at the XR application server

  - As noted in [3], frame rate and frame generation may be triggered by events such as empty uplink buffer at the video codec etc, which can result in jitter in frame generation at the TX side. This jitter will filter down through the transport network and will be visible to the RAN at the DL scheduler.

- Jitter in the transport network

  - Even in case of periodic frame generation at the transmit side, jitter can be introduced due to congestion in the transport network. This jitter typically results in late arrival of the frame at the RAN scheduler in DL.

Figure 1: DL packet arrival outside the DRX ON duration due to jitter

If DL packets arrive at the gNB scheduler at a predictable time, then a DRX cycle with a very small ON duration would be sufficient to satisfy the QoS requirements of the XR traffic. However, due to jitter, such resource allocation scheme is no longer sufficient and hence power consumption enhancements should consider jitter in DL arrival for XR traffic, this is depicted in Figure 1 above. Based on this, the following proposal is made.

Proposal 1: Jitter in arrival of DL packets at the gNB scheduler should be considered in power saving enhancements for XR traffic

# Power saving in case of jitter in DL arrival times

Extension of DRX on duration

In case of periodic XR traffic, jitter can cause both early arrival and delayed arrival of DL data. However, early arrival of packets in general does not have an adverse impact on overall latency budget for XR traffic (i.e the network can simply buffer the early packet and send it at the next ON duration without impacting the power consumption at the UE). However, if the DRX on duration cannot capture the packets that are delayed due to jitter, then this will have significant impact on overall latency since the UE may go into DRX mode and the packet can only be received in the next DRX ON cycle. Periodic XR traffic, is generally characterised by having at least one packet during every DRX on duration, one option could be to extend the DRX on duration dynamically if the DL packet is delayed. With this option, if the DL packet arrives during the DRX ON duration time, then the UE Can terminate the DRX cycle and go back to sleep normally and if the packet is delayed then the DRX on duration can be extended by a specific duration. This is depicted in Figure 2 below.

Figure 2: Dynamic extension of ON Duration to cater for delayed packet arrival due to jitter

Based on the above, the following proposal is made.

Proposal 2: Dynamic extension of DRX ON duration to cater for the delayed downlink XR packet is considered as solution for DL jitter

Multiple DRX cycles

Another alternative is to configure more than one DRX cycles to the same UE. This may be useful particularly in case of non-periodic XR traffic. For instance, three different DRX cycles with different C-DRX configurations (e.g. start Time) can be configured to the UE. e.g. for 60fps service, the periodicity is 16.667ms, which can be transferred to three C-DRX configuration with integer periodicities(50ms) as follows:

|  | PDCCH monitoring start time | Periodicity |
| --- | --- | --- |
| C-DRX Configuration | 0ms | 50ms |
| C-DRX Configuration | 17ms | 50ms |
| C-DRX Configuration | 34ms | 50ms |

Proposal 3: To handle the non-integer periodic XR traffic, the network can configure more than one DRX cycles and change the DRX cycle with non-integer periodicity to multiple DRX cycles with integer periodicity.

Multiple USS

Another alternative is to configure more than one USS to the same UE. This may be useful particularly for UE power saving. For instance, two different UE specific search spaces with different USS periodicities can be configured to the UE. The UE monitors the 1st USS for PDCCH/WUS during the ON duration and upon detecting the indication, the UE monitors the 2nd USS using the second USS periodicity. With this approach, the USS periodicity of the first USS can be kept large and can avoid high power consumption at the UE side. The second USS which is conditionally activated upon receiving the indication on the first USS can have a smaller USS periodicity to allow the network scheduler to transmit the DL data.

Proposal 4: To handle the non-periodic XR traffic or periodic XR traffic with jitter, the network can configure more than one USS and the second USS is activated only upon receiving an indication on the first USS.

# Other enhancements

Finer granularity for DRX cycle

The DRX cycles currently supported do not have a very fine granularity. For instance, XR frame rates typically include 30/60/120 frames per second, and this has a nominal periodicity of 33.33/16.66/8.33 ms. However, the nearest periodicities for DRX cycle to match these periodicities are 32/20/10 ms. As can be seen, there is a large discrepancy between the nominal XR traffic periodicities and the DRX cycle lengths that could be configured for the UE. To better match the actual periodicities for XR traffic, other DRX cycle lengths should be considered. As an example, the following values could be considered as a starting point {33, 34, 16, 17, 8, 9} ms.

Proposal 5: Finer granularity for DRX periodicity should be considered to better match the typical XR traffic periodicities. As a starting point the following additional values are proposed {33, 34, 16, 17, 8, 9} ms - (other values e.g. with granularity of 1ms can also be considered).

DRX Period mismatch at SFN wrap-around

Even if the DRX cycle is perfectly matched with the periodicity of the XR traffic in DL, there can be a mismatch between the DL periodicity and the DRX ON duration cycle because of the SFN wrap around. This issue results in the DL DRX cycle going out of sync with the arrival time of the XR traffic as result of the SFN wrap around. This is illustrated in the Figure 3 below.

Figure 3: Mismatch between DL traffic arrival and DRX cycle after SFN wrap around

Proposal 6: The definition of the CDRX on duration start occasion should be redefined to solve the issue with the mismatch between periodic DL XR traffic arrival and the DRX ON duration periods after SFN wrap around.

# Conclusion

The following observations and proposals are made:

Observation 1: Power saving gains are essential to ensure that the power consumption at the UE is not disproportionately affected by the tight QoS requirements of XR traffic

Proposal 1: Jitter in arrival of DL packets at the gNB scheduler should be considered in power saving enhancements for XR traffic

Proposal 2: Dynamic extension of DRX ON duration to cater for the delayed downlink XR packet is considered as solution for DL jitter

Proposal 3: To handle the non-integer periodic XR traffic, the network can configure more than one DRX cycles and change the DRX cycle with non-integer periodicity to multiple DRX cycles with integer periodicity.

Proposal 4: To handle the non-periodic XR traffic or periodic XR traffic with jitter, the network can configure more than one USS and the second USS is activated only upon receiving an indication on the first USS.

Proposal 5: Finer granularity for DRX periodicity should be considered to better match the typical XR traffic periodicities. As a starting point the following additional values are proposed {33, 34, 16, 17, 8, 9} ms

Proposal 6: The definition of the CDRX on duration start occasion should be redefined to solve the issue with the mismatch between periodic DL XR traffic arrival and the DRX ON duration periods after SFN wrap around.

# References

- S2-2201087, Key issue on traffic characteristics of XR and media service, SA2#149-e

- 3GPP TR 23.700-60, V0.3.0, Study on XR (Extended Reality) and media services

- S4-220505, LS Reply on QoS support with PDU Set granularity, SA4#118e