3GPP TSG-RAN WG2 Meeting #119 Electronic	R2-2208006

17 – 26 August 2022

Agenda item:	8.9.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on service continuity enhancement for L2 U2N relay

WID/SID:	NR_SL_relay_enh- Release 18

Document for:	Discussion and Decision

# 1	Introduction

The approved release 18 WI on enhanced NR SL Relay [RP-221262] includes the following objective on enhancement of service continuity:

- Specify mechanisms to enhance service continuity for single-hop Layer-2 UE-to-Network relay for the following scenarios [RAN2, RAN3]:

  - Inter-gNB indirect-to-direct path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> gNB Y”)

  - Inter-gNB direct-to-indirect path switching (i.e., “remote UE <-> gNB X” to “remote UE <-> relay UE A <-> gNB Y”)

  - Intra-gNB indirect-to-indirect path switching (i.e., “remote UE <-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB X”)

  - Inter-gNB indirect-to-indirect path switching (i.e., “remote UE<-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB Y”)

Note 2A: Scenario D is to be supported by reusing solutions for the other scenarios without specific optimizations.

This contribution proposes several basic issues to be discussed by RAN2.

# 2	Discussion

## 2.1	Clarification of scenario D of inter-gNB indirect-to-indirect path switching

The objective description of scenario D of inter-gNB indirect-to-indirect path switching indicates “remote UE<-> relay UE A <-> gNB X” to “remote UE <-> relay UE B <-> gNB Y” scenario, which shows clearly to support the indirect-to-indirect path switching via different relay UEs. However, it is not fully clear whether relay UE A and relay UE B can be the same relay UE or not. Considering the use cases of V2X, public safety, IoT etc, there are many scenarios that the relay UE and the remote UE may move together and the remote UE may keep connected to the same relay UE during indirect-to-indirect path switching from one gNB to another gNB. Thus, it is better to clarify that inter-gNB indirect-to-indirect path switching via the same relay UE is also in the scope of scenario D.

Proposal 1: RAN2 to confirm inter-gNB indirect-to-indirect path switching via the same relay UE (i.e., the remote UE remains connected to a relay UE when the relay UE performs an inter-gNB handover) is in the scope of scenario D.

If proposal 1 can be confirmed by RAN2, most probably inter-gNB handover (HO) of the relay UE and indirect-to-indirect path switching of the remote UE may be initiated from the source gNB to the target gNB at the same time when the relay and remote UEs move together. This provides the opportunity to enhance the remote UE’s path switching in term of RRC and Xn procedure when the relay UE performs an inter-gNB HO.

Proposal 2: RAN2 to discuss whether enhancements of the indirect-to-indirect path switching when the relay UE performs an inter-gNB HO should be pursued.

## 2.2	Discussion on inter-gNB path switching to indirect path

For inter-gNB path switching to indirect path regardless of whether the remote UE has direct path or indirect path with the serving gNB before path switching is initiated (i.e scenario B or D), the selected target relay UE should be connected to the target gNB as otherwise it will not trigger inter-gNB path switching. On one hand, it follows the existing principle that the source gNB makes the path switching decision and selects the target relay UE for indirect path switching. This also reduce the signaling overhead over Xn as the source gNB only needs to indicate to the target gNB the ID of selected target relay UE. But on the other hand, if multiple candidate relay UEs are connected to the target gNB, the target gNB have better information about the candidate relay UEs including Uu link condition, relay UE’s own traffic and the number of connected remote UEs etc. Therefore, the target gNB is more suitable to select the target relay UEs among the candidate relay UEs connected to the target gNB.

Proposal 3: RAN2 to discuss whether the source gNB or the target gNB makes the final selection of the target relay UE in case of inter-gNB path switching to indirect path.

Regardless of whether the source gNB or the target gNB makes the final selection of the target relay UE, when inter-gNB path switching is initiated, Xn procedure similar as HO preparation procedure will be initiated from the source gNB by sending HO Request to the target gNB. As it is indirect path switching, the target gNB needs to re-configure the target relay UE for preparing remote UE’s indirect path switching before it can send the HO Response to the source gNB over Xn interface. Comparing with normal HO preparation procedure, the additional RRC reconfiguration procedure between the target gNB and the selected target relay UE will prolong the HO preparation time before the source gNB can send path switching command to the remote UE. During the prolonged HO preparation time, the probability of missing the path switching command transmitted from the source gNB for configuration of path switching becomes higher than in normal HO scenarios; e.g., the remote UE may already move out of coverage of the current serving cell if the remote UE has direct path with the source gNB, or if the remote UE has indirect path with the source gNB, it may happen that the relay UE moves out of coverage of source cell or the SL connection between the currently connected relay UE and the remote UE. Missing of path switching command by remote UE will cause path switching failure, which will in turn impact the remote UE’s service continuity.

Proposal 4: RAN2 to confirm the issue of prolonged HO preparation time due to reconfiguration of target relay UE in target gNB during the preparation of inter-gNB path switching to indirect path and study the solution on how to solve the issue.

## 2.3	Discussion on relay UE in RRC Idle/Inactive state for path switching to indirect path

In Release 17 intra-gNB path switching from direct to indirect path, relay UE in RRC Idle/Inactive state is supported. It has been agreed that the remote UE establishes a PC5 link with the selected relay UE and sends the RRCReconfigurationComplete message via the relay UE after receiving the path switching command. This will trigger the relay UE to enter RRC_CONNECTED state. The procedure specified in Release 17 is not optimal from remote UE’s service continuity point of view as remote UE’s traffic transmission can be resumed after the selected relay UE establishes its own RRC connection that is triggered by establishment of PC5 link with the remote UE and reception of the RRCReconfigurationComplete message from the remote UE. To better support service continuity for remote UEs during path switching to indirect path via a relay UE in RRC Idle/Inactive state, the enhancement on support of relay UE in RRC Idle/Inactive state should be further considered.

Proposal 5: RAN2 to confirm that the enhancement on service continuity in case of relay UE in RRC Idle/Inactive state for path switching to indirect path is in the scope of Release 18 objectives.

The path switching decision on remote UE switching to indirect path is made by the serving gNB based on measurement report from the remote UE. In the measurement report, the remote UE reports the candidate relay UEs with the relay UE’s ID, relay UE’s serving cell ID and sidelink channel quality information such as SL-RSRP. It is rather straightforward for gNB to select the best relay UE in RRC_CONNECTED state as relay UE’s context including relay UE’s capability information, Uu link condition, connected remote UEs and traffic load etc is available in the gNB. However, for a relay UE in RRC_IDLE/INACTIVE state, the gNB doesn’t have any UE context information listed above. The only criteria that the gNB can use to select the target relay UE is the reported SL RSRP from the remote UE, which is the SL channel quality at the time when SL-RSRP measured and may not reflect the latest SL condition especially in very dynamic SL channel environment. Therefore, enabling gNB to select the best possible relay UE in RRC Idle/Inactive state should be studied in RAN2 to enhance the service continuity of remote UE during path switching to indirect path.

Proposal 6: RAN2 to discuss enhancements in selecting a relay UE in RRC Idle/Inactive state for service continuity during path switching to indirect path.

# 3	Conclusion

This contribution includes the following proposals:

Proposal 1: RAN2 to confirm inter-gNB indirect-to-indirect path switching via the same relay UE (i.e., the remote UE remains connected to a relay UE when the relay UE performs an inter-gNB handover) is in the scope of scenario D.

Proposal 2: RAN2 to discuss whether enhancements of the indirect-to-indirect path switching when the relay UE performs an inter-gNB HO should be pursued.

Proposal 3: RAN2 to discuss whether the source gNB or the target gNB makes the final selection of the target relay UE in case of inter-gNB path switching to indirect path.

Proposal 4: RAN2 to confirm the issue of prolonged HO preparation time due to reconfiguration of target relay UE in target gNB during the preparation of inter-gNB path switching to indirect path and study the solution on how to solve the issue.

Proposal 5: RAN2 to confirm that the enhancement on service continuity in case of relay UE in RRC Idle/Inactive state for path switching to indirect path is in the scope of Release 18 objectives.

Proposal 6: RAN2 to discuss enhancements in selecting a relay UE in RRC Idle/Inactive state for service continuity during path switching to indirect path.