3GPP TSG-RAN WG2 Meeting #119 Electronic	R2-2208019

Elbonia, 17 – 26 August 2022

Agenda item:	8.5.3

Source:	Nokia, Nokia Shanghai Bell (Rapporteur)

Title:	XR power saving study overview and suggestions for RAN2 focus

WID/SID:	FS_NR_XR_enh- Release 18

Document for:	Discussion and Decision

# 1	Introduction

RAN#95 approved a Study on XR Enhancements for NR [1] with the following objectives:

### 4.1 Objective of SI or Core part WI or Testing part WI The study is to be based on Release 17 TR 38.838, on corresponding Release 17 work from SA4 (as per SP-210043) and on Release 18 work from SA2 (as per SP-211166). Objectives on XR-awareness in RAN (RAN2): - Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of. - Study how the above information aids XR-specific traffic handling. Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement. Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements. - Dynamic scheduling/grant enhancements.

In this contribution we give an overview of XR power saving study that has been done in RAN1 and provide suggestions on key issues for RAN2 to focus from rapporteur point of view. Note that we only have three RAN2 meetings to conclude the study, with TR already to be submitted to RAN for information after this meeting.

# 2	Discussion

## 2.1	Rel-17 RAN1 XR SI conclusions

During the Rel-17 RAN1 XR SI [2], traffic models and characteristics of AR, VR, and CG applications were developed, and performance has been evaluated in terms of capacity, UE power consumption, coverage and mobility. It was recommended to further study and enhance at least the capacity and UE power consumption performance of 5G NR for XR and CG applications, as captured in TR 38.838 [3] and reflected in the Rel-18 SID.

## 2.2	Rel-18 XR study RAN1 #109e agreements

In the previous RAN1 meeting, the following agreements were made with several issues identified as higher/medium/low priority [4][5]:

Agreement: Rel-17 evaluation methodology for XR power saving captured in TR 38.838 is used as the baseline evaluation methodology for UE power evaluation of Rel-18 SI on XR enhancements. Agreement: Companies are encouraged to compare performance of the following Rel-15/16/17 features with the proposed enhancements for Rel-18 XR power saving evaluations. Power saving gain is calculated w.r.t. the AlwaysOn baseline. - Rel-15/16 CDRX including long DRX cycle, short DRX cycle and DRX command MAC CE and DCP - Rel-17 PDCCH adaptation including PDCCH skipping and SSSG switching Note: up to companies to report the configuration of the Rel-15/16/17 features Agreement: For power saving study of Rel-18 XR SI, CDRX enhancements to evaluate in this study item are to be selected from the following: - High priority Issue 1-1: Alignment between CDRX and XR traffic for resolving the mismatch between CDRX cycle and XR traffic periodicity for each flow - High priority Issue 1-2: C-DRX enhancements to handle jitter - Medium priority Issue 1-3: CDRX enhancements for multiple XR traffic flows [Note 2] - Low priority Issue 1-4: CDRX enhancements to adjust to variable burst sizes and frame rate - Note: Some companies think the adjustment for variable burst sizes can be realized by existing spec already - Low priority Issue 1-5: low latency handling - Low priority Issue 1-6: SFN wraparound mismatch (if handled in RAN1) FFS: how the solutions or the combination of the solutions can handle all the identified issues. Note 1: Other considerations are not precluded Note 2: It can also be adopted for addressing issue 1-1 Note 3: Companies are encouraged to clarify or provide more details of the proposed solutions, for addressing concerns from the group. Additional details can be found in R1-2205411. Agreement: For power saving study of Rel-18 XR SI, PDCCH monitoring enhancements to evaluate in this study item are to be selected from the following - Low priority Issue 2-1: Alignment between PDCCH monitoring and XR traffic to resolve the mismatch between PDCCH monitoring periodicity and XR traffic periodicity. - Note: some companies think Rel-17 PDCCH monitoring adaptation can solve issue 2-1 or achieve similar intended outcome - Note: Solutions proposed for Issue 2-1 and those proposed for Issue 1-1 are motivated by the same issue, namely non-integer XR traffic periodicity. It is to be studied how they compare in in terms of power saving gain and capacity, (a) solutions proposed for Issue 1-1; (b) solutions proposed for Issue 2-1. - Low priority Issue 2-2: XR-dedicated PDCCH monitoring window to supplement CDRX for multi-flow traffic. - Note: some companies think Rel-17 PDCCH monitoring adaptation can solve issue 2-2 or achieve similar intended outcome - Note: Solutions proposed for Issue 2-2 and those proposed for Issue 1-3 are motivated by the same issue, namely multiple XR traffic flows. It is to be studied how they compare in in terms of power saving gain and capacity, (a) solutions proposed for Issue 1-3; (b) solutions proposed for Issue 2-2. - High priority Issue 2-3: Enhancements to Rel-17 PDCCH monitoring adaptation. - Note: Discussion on some enhancements may depend on the outcome of Rel-17 PDCCH monitoring adaptation maintenance - Note: The study on enhancement to R17 PDCCH monitoring adaptation should focus on the techniques that are used for addressing XR-specific issues, e.g., jitter Note 1: Other considerations are not precluded Note 2: Companies are encouraged to clarify or provide more details of the proposed solutions, for addressing concerns from the group. Agreement: For Rel-18 XR power saving enhancements, RAN1 further discusses by RAN1 #110 whether the issues below are to be addressed, and if so, which solutions should be selected for evaluation in this study item. These issues are low priority. - Issue 3-1: Misaligned UE transmission and reception. - Issue 3-2: Power saving by XR-aware scheduling. - Note 1b: XR SI objective has XR-awareness in RAN listed as a specific topic of RAN2 study - Issue 3-3: Unnecessary data transmission in allocated resources. Note 1: Rel-18 XR SI objective only has CDRX enhancements and PDCCH monitoring enhancements explicitly listed as focus of RAN1 study Note 2: Other considerations are not precluded

## 2.3	Suggestions for RAN2 focus

Since RAN1 has been discussing power saving for XR since last year, following those conclusions/agreements, we hope RAN2 can conclude in this meeting on the key issues to focus and to be captured in the TR, other than repeating the same discussions. In the coming meetings, we can then focus more on details of potential solutions for those key issues.

For the RAN1 identified high priority issues, RAN2 would obviously need to discuss solutions for those as well. In the following we discuss if any attention from RAN2 point of view is needed for the listed medium/low priority issues.

For DRX enhancement:

- Medium priority Issue 1-3: CDRX enhancements for multiple XR traffic flows, Note 2 indicates “It can also be adopted for addressing issue 1-1”. One can say also the other way around that the solutions for issue 1-1/1-2 could be applicable to 1-3 as well. Whether anything specific for multiple flows can be discussed later when solutions for single flow becomes clearer.

- Low priority Issue Issue 1-4: CDRX enhancements to adjust to variable burst sizes and frame rate, variable burst size does not really represent a problem as it suffices that the UE is in Active Time when the burst arrives and the scheduler can schedule very large bursts using the multi-slot scheduling feature introduced in Rel-17 and it does not require to clear the buffer within one shot either. Variable framerate affects the arrival period of bursts, but enhancements discussed for solving Issue 1-1 and Issue 1-2 like fast adaptation of CDRX parameters can be used to solve variable frame-rate identified in Issue 1-4. It seems to be more relevant to capacity enhancement other than power saving.

- Low priority Issue Issue 1-5: Low latency, it is considered in Issue 1-2 since the objective of all proposed enhancements is to handle jitter in order to meet the PDB of the XR traffic.

- Low priority Issue Issue 1-6: SFN wraparound mismatch, it is caused by the time drift between XR traffic and CDRX cycle due to the mismatch of XR and CDRX periodicities and related problems are evaluated in Issue 1-1.

For PDCCH monitoring enhancements, since the feature has been mainly discussed in RAN1 in Rel-17, we can also leave it to RAN1 for now and check the details later if RAN1 identify any RAN2 impact.

Based on above analysis, we think it is enough for RAN2 to focus on the high priority issues as identified by RAN1 for power saving, as well identifying necessary parameters for XR-awareness to support such enhancements.

Proposal 1: suggest RAN2 to focus on the high priority issues as identified by RAN1 for power saving, as well necessary parameters for XR-awareness to support such enhancements, i.e.:

- DRX enhancements to address the issues of DRX cycle mismatch and jitter

- Enhancements to Rel-17 PDCCH adaptation if RAN1 identify any RAN2 impact

- Identify necessary parameters from CN for XR-awareness for power saving

Proposal 2: RAN2 will not study further the medium and low priority issues listed in RAN1 unless otherwise requested by RAN1.

# 3	Conclusion

In this contribution, we briefly recap the discussions done in RAN1 on power saving and made the following suggestion for RAN2 focus:

Proposal 1: suggest RAN2 to focus on the high priority issues as identified by RAN1 for power saving, as well necessary parameters XR-awareness to support such enhancements, i.e.:

- DRX enhancements to address the issues of DRX cycle mismatch and jitter

- Enhancements to Rel-17 PDCCH adaptation if RAN1 identifies any RAN2 impact

- Identify necessary parameters from CN for XR-awareness for power saving

Proposal 2: RAN2 will not study further the medium and low priority issues listed in RAN1 unless otherwise requested by RAN1.

# Reference

[1] RP-220285, Revised SID: Study on XR Enhancements for NR, RAN#95.

[2] RP-201145, Revised SI on XR Evaluations for XR

[3] TR38.838, Study on XR (Extended Reality) Evaluations for NR

[4] R1-22xxxxx, RAN1 #109e minutes

[5] R1-2205412, Final Moderator Summary on XR specific power saving techniques, Moderator (Qualcomm Incorporated)