3GPP TSG-RAN WG2 #119e	Tdoc R2-2208030

Electronic meeting, August 15 – 26, 2022

Agenda Item:	6.3.3

Source:	Ericsson

Title:	Mac updates for MUSIM

Document for:	Discussion, Decision

# 1	Introduction

In RAN2#118-e, the following was agreed based on a summary discussion [1]:

15: The UE is allowed to initiate RACH procedure during MUSIM gaps. Capture this in MAC specification according R2-2204895 (first part, sections 5.1.2 and 5.1.2a).

This contribution discusses generally the update of MAC specifications for MUSIM and its impact.

# 2	Discussion

The impact to MAC specification mentioned above was in sections 5.1.2 and 5.1.2a of 38.321, which is also captured below.

5.1.2	Random Access Resource selection

If the selected RA_TYPE is set to 4-stepRA, the MAC entity shall:

[skipped parts]

1>	if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]); and

1>	if ra-AssociationPeriodIndex and si-RequestPeriod are configured:

2>	determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB in the association period given by ra-AssociationPeriodIndex in the si-RequestPeriod permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6] corresponding to the selected SSB).

1>	else if an SSB is selected above:

2>	determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured or indicated by PDCCH (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the selected SSB; the MAC entity may take into account the possible occurrence of measurement gaps and MUSIM gaps when determining the next available PRACH occasion corresponding to the selected SSB).

1>	else if a CSI-RS is selected above:

2>	if there is no contention-free Random Access Resource associated with the selected CSI-RS:

3>	determine the next available PRACH occasion from the PRACH occasions, permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, corresponding to the SSB in candidateBeamRSList which is quasi-colocated with the selected CSI-RS as specified in TS 38.214 [7] (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the SSB which is quasi-colocated with the selected CSI-RS; the MAC entity may take into account the possible occurrence

of measurement gaps and MUSIM gaps when determining the next available PRACH occasion corresponding to the SSB which is quasi-colocated with the selected CSI-RS).

2>	else:

3>	determine the next available PRACH occasion from the PRACH occasions in ra-OccasionList corresponding to the selected CSI-RS (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the PRACH occasions occurring simultaneously but on different subcarriers regardless the FR2 UL gap, corresponding to the selected CSI-RS; the MAC entity may take into account the possible occurrence of measurement gaps and MUSIM gaps when determining the next available PRACH occasion corresponding to the selected CSI-RS).

1>	perform the Random Access Preamble transmission procedure (see clause 5.1.3).

NOTE 1:	When the UE determines if there is an SSB with SS-RSRP above rsrp-ThresholdSSB or a CSI-RS with CSI-RSRP above rsrp-ThresholdCSI-RS, the UE uses the latest unfiltered L1-RSRP measurement.

NOTE 2:	Void.

NOTE 3:	If a RedCap UE in RRC_IDLE or RRC_INACTIVE mode is configured with a BWP indicated by initialDownlinkBWP-RedCap which is not associated with any SSB, SS-RSRP measurement is performed based on the SSB associated with the BWP indicated by initialDownlinkBWP.

Editor's NOTE:	Postpone the discussion on whether and how to capture the agreement: From RAN2 perspective, if a RedCap UE in idle/inactive mode is configured with a separate initial BWP associated with no SSB (CD or NCD) for RACH, it is up to UE implementation to perform new RSRP measurement in a DL BWP associated with CD-SSB before Msg1/A retransmission.

### 5.1.2a	Random Access Resource selection for 2-step RA type

If the selected RA_TYPE is set to 2-stepRA, the MAC entity shall:

[skipped parts]

1>	determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the msgA-SSB-SharedRO-MaskIndex if configured and ra-ssb-OccasionMaskIndex if configured (the MAC entity shall select a PRACH occasion randomly with equal probability among the consecutive PRACH occasions allocated for 2-step RA type according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the selected SSB; the MAC entity may take into account the possible occurrence of measurement gaps and MUSIM gaps when determining the next available PRACH occasion corresponding to the selected SSB);

1>	if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble(s):

2>	select a PUSCH occasion from the PUSCH occasions configured in msgA-CFRA-PUSCH corresponding to the PRACH slot of the selected PRACH occasion, according to msgA-PUSCH-resource-Index corresponding to the selected SSB;

2>	determine the UL grant and the associated HARQ information for the MSGA payload in the selected PUSCH occasion;

2>	deliver the UL grant and the associated HARQ information to the HARQ entity.

1>	else:

2>	select a PUSCH occasion corresponding to the selected preamble and PRACH occasion according to clause 8.1A of TS 38.213 [6];

2>	determine the UL grant for the MSGA payload according to the PUSCH configuration associated with the selected Random Access Preambles group and determine the associated HARQ information;

2>	if the selected preamble and PRACH occasion is mapped to a valid PUSCH occasion as specified in clause 8.1A of TS 38.213 [6]:

3>	deliver the UL grant and the associated HARQ information to the HARQ entity.

1>	perform the MSGA transmission procedure (see clause 5.1.3a).

The change is motivated due to the fact that the UE handling of MUSIM gaps would be similar to the UE handling of measurement gaps. But from that perspective, it should be noted that there are multiple occurrences of “measurement gap” in 38.321, e.g. general handling of MUSIM gaps, scheduling request, DRX, etc. Hence, to introduce only the changes depicted above in 38.321 may be incomplete and inconsistent.

Specifying MUSIM gaps in 38.321 may require multiple changes that are not limited to RACH procedure.

It should thus be discussed whether any change at all should be introduced in 38.321 for MUSIM. If decided that changes are needed, it should be done consistently and across all aspects that may touch upon MUSIM.

RAN2 to discuss whether to introduce MUSIM gaps behaviour in 38.321.

It should also be noted that currently other features that require gaps are introduced and may not be consistently specified in 38.321 as well. Preferably a consistent decision should also be taken for those other cases, e.g. another possibility is to attempt to have a general wording in 38.321 that may serve to different gaps.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Specifying MUSIM gaps in 38.321 may require multiple changes that are not limited to RACH procedure.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to discuss whether to introduce MUSIM gaps behaviour in 38.321.

# References

R2-2206171, 3GPP TSG RAN WG1 Meeting #109-e, Electronic Meeting, May 09-20, 2022; Summary #3 for email discussion on energy saving techniques of NW energy saving SI; Moderator (Intel Corporation)