3GPP TSG-RAN WG2 #119-e	Tdoc R2-2208032

Electronic meeting, August 15th – 26th, 2022

Agenda Item:	6.3.3

Source:	Ericsson

Title:	Discussion on gap length IE optionality

Document for:	Discussion, Decision

# 1	Introduction

This contribution focuses on possible changes related to the optionality of the field musim-GapLength, which is included in MUSIM-GapInfo IE, as defined in [1].

# 2	Discussion

According to [1], the MUSIM-GapInfo IE is used to specify the MUSIM gap parameters. This is used both by the network to configure the gaps (in RRCReconfiguration message) and by the UE to request the preferred gaps (in the UEAssistanceInformation message).

The MUSIM-GapInfo IE is defined as the following:

-- ASN1START

-- TAG-MUSIM-GAPINFO-START

MUSIM-GapInfo-r17 ::=               SEQUENCE {

musim-Starting-SFN-AndSubframe-r17  MUSIM-Starting-SFN-AndSubframe-r17             OPTIONAL, -- Cond aperiodic

musim-GapLength-r17                 ENUMERATED {ms3, ms4, ms6, ms10, ms20}         OPTIONAL, -- Need S

musim-GapRepetitionAndOffset-r17    CHOICE {

ms20-r17                            INTEGER (0..19),

ms40-r17                            INTEGER (0..39),

ms80-r17                            INTEGER (0..79),

ms160-r17                           INTEGER (0..159),

ms320-r17                           INTEGER (0..319),

ms640-r17                           INTEGER (0..639),

ms1280-r17                          INTEGER (0..1279),

ms2560-r17                          INTEGER (0..2559),

ms5120-r17                          INTEGER (0..5119),

...

}                                                                                  OPTIONAL -- Cond periodic

}

MUSIM-Starting-SFN-AndSubframe-r17 ::= SEQUENCE {

starting-SFN-r17                       INTEGER (0..1023),

startingSubframe-r17                   INTEGER (0..9)

}

-- TAG-MUSIM-GAPINFO-STOP

-- ASN1STOP

Since a gap is defined by its periodicity and length (period gap) or by the starting point and length (aperiodic gap), when the network configures the gaps, the MUSIM-GapInfo IE includes:

- Periodic gaps: musim-GapRepetitionAndOffset IE and musim-GapLength IE

- Aperiodic gaps: musim-Starting-SFN-AndSubframe IE and musim-GapLength IE

The musim-GapLength IE is always present when the network configures the gaps.

The musim-GapLength IE is always present when the network configures the gaps (periodic and aperiodic)

The UE uses the UEAssistanceInformation message to request the preferred gaps or to release them by including a list of MUSIM-GapInfo IE (one per each gap):

MUSIM-GapPreferenceList-r17 ::= SEQUENCE (SIZE (1..4)) OF MUSIM-GapInfo-r17

As described in [1], Ch.5.7.4.3, when the UE wants to request new preferred gaps, it includes one MUSIM-GapInfo IE per gap in the list and, regardless if the gap is periodic or aperiodic, the musim-GapLength IE will be always included.

When the UE wants to remove the gaps, the entire list in not included in the UEAssistanceInformation message.

So, the UE sends the MUSIM-GapInfo IE only to request the preferred gaps, and the musim-GapLength IE is always present.

The musim-GapLength IE is always present when the UE requests the preferred gaps (periodic and aperiodic)

Based on the Observations above, the musim-GapLength IE should be mandatory, since it is always sent both by the network and by the UE.
In principle, one could consider to keep the musim-GapLength IE as Optional and update the field description to state the field must always be included. However, it is cleaner to make it mandatory. Hence, at this stage we think it should still be considered to make the field mandatory.

Define the field musim-GapLength as Mandatory IE.

The proposed changes are illustrated in R2-2208029.

# 3	Conclusion

In the previous sections we made the following observations:

- The musim-GapLength IE is always present when the network configures the gaps (periodic and aperiodic)

- The musim-GapLength IE is always present when the UE requests the preferred gaps (periodic and aperiodic)

Based on the discussion in the previous sections we propose the following:

Proposal 1	Define the field musim-GapLength as Mandatory IE.

# References

TS 38.331, “Radio Resource Control (RRC) protocol specification”, Rel.17.1.0