3GPP TSG-RAN WG2 Meeting #119-e	R2-2208033

# Electronic meeting, August 15th – 26th, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0526 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to gap handling for Multi-USIM (38.300) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_MUSIM-Core |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Capture agreements from RAN2-118e. Impact analysis Impacted functionality: MUSIM gaps Inter-operability: If the network implements the CR and the UE does not, no inter-operability issue is expected. The network may assume the UE supports up to 3 periodic gaps, but the UE would only report at most 2, the network may then configure the UE according to one or 2 of those gaps. If the UE implements the CR and the network does not, the network may assume the UE supports up to 2 periodic gaps, while the UE may report up to 3 periodic gaps. If the UE reports 3, it will be unclear which gaps the network would configure the UE with, if any. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated text on number of configurable gap patterns. - Added that the Network should always provide at least one of the requested gap patterns or no gaps, and that the Network cannot provide an alternative gap pattern. - Corrected typos and small rewording to improve readability |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage-2 will not correctly capture the gap behavior for MUSIM. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 20.3 UE notification on Network Switching |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

## 20.3	UE notification on Network Switching

For MUSIM operation, a MUSIM device in RRC_CONNECTED state in Network A may have to switch from Network A to Network B. Network A is NR and Network B can either be E-UTRA or NR. Before switching from Network A, a MUSIM device should notify Network A to either leave RRC_CONNECTED state, or be kept in RRC_CONNECTED state in Network A while temporarily switching to Network B.

When configured to do so, a MUSIM device can signal to the Network A a preference to leave RRC_CONNECTED state by using RRC or NAS signaling. After sending a preference to leave RRC_CONNECTED state by using RRC signaling, if the MUSIM device does not receive an RRCRelease message from the Network A within a certain time period (configured by the Network A), the MUSIM device can enter RRC_IDLE state in Network A.

When configured to do so, a MUSIM device can signal to the Network A a preference to be temporarily switching to network B while remaining in RRC_CONNECTED state in Network A . This is indicated by scheduling gaps preference. This preference can include information for setup or release of gap(s). The Network A can configure at most 4 gap patterns for MUSIM purpose: three periodic gaps and a single aperiodic gap. The network may configure the UE according to one or more of the indicated gap patterns.

END OF CHANGE

# Annex: RAN2 Agreements (LTE_NR_MUSIM-Core; leading WG: RAN2; REL-17; WID: RP-202895)

## RAN2#118-e

Extend signalling to allow UEs to optionally support 3 periodic gaps in Rel-17. This can be included in the RRC CR rapporteur CR in offline [230].

Capture NOTE about INACTIVE UE behaviour if it rejects RAN paging in 38.331. Can discuss exact wording for the NOTE offline. Should refer to CT1 specifications.

- Bulk agreement

[234] 1: From RAN2 perspective, confirm that INACTIVE UE may not be able to send a Service Request message to the network including the Reject Paging indication as a response to the RAN paging due to UE implementation constraints.

[234] 2: From RAN2 perspective, agree to add the following NOTEs in TS 38.331 and TS 36.331 respectively as follows:

- NOTE in TS 38.331: A MUSIM UE may not initiate the RRC connection resumption procedure, e.g. when it decides not to respond to the Paging message due to UE implementation constraints as specified in TS 24.501 [23].

- NOTE in TS 36.331: A MUSIM UE may not initiate the RRC connection resumption procedure, e.g. when it decides not to respond to the Paging message due to UE implementation constraints as specified in TS 24.501 [95].

[234] 4: Send LS to CT1/SA2 to inform them of our decision and to ask them to update their specifications accordingly if needed.

- Bulk agreements

[232] 1: RAN2 does not consider extending the MUSIM gap durations in Rel-17.

[232] 2: RAN2 assumes that handling of RLM/BFD during MUSIM gaps will be addressed by RAN4 in Rel-18.

[232] 3: Signaling a “gap priority” in UAI for MUSIM gaps is not supported in Rel-17.

[232] 4: The propsosal in R2-2204618, where the UE sends UAI with the same MUSIM request if the previous request is not granted, is not agreed.

[232] 5: The proposal in R2-2204618 that UE does not need to stop prohibit timer if the NW disables MUSIM assistance for gap preference is not agreed.

[232] 6: Mandatory MUSIM gap patterns are not introduced in Rel-17. RAN2 keeps the agreement that the UE can request any MUSIM gap pattern defined in TS 38.133.

[232] 7: Add the following sentence in the field description of musim-GapPreferenceList: "musim-GapPreferenceList: Indicates the MUSIM gap(s) that the UE prefers to be configured with. The UE indicates preference for MUSIM gap(s) in accordance with clause 9.1.10 of TS 38.133."

[232] 9: The proposal in R2-2204614 to isolate the periodic and aperiodic gap configurations into separate IEs is not agreed.

[232] 10: Agree to the TP in R2-2204615. This can be included in WI Rapporteur CR for 38.331.

[232] 11: Confirm that the network configured musim-GapRepetitionAndOffset shall be aligned with the UE request.

[232] 17: RAN2 to confirm that MUSIM gap is released only upon receiving signaling from NW for the release.

14: The following statement should be captured in 38.300: "Network should always provide at least one of the requested gap pattern or no gaps. Network providing an alternative gap pattern instead of the one requested by the UE is not supported in this release"

15: The UE is allowed to initiate RACH procedure during MUSIM gaps. Capture this in MAC specification according R2-2204895 (first part, sections 5.1.2 and 5.1.2a).

12: For the aperiodic Gap configuration, if network provides aperiodic gap, it always signals the musim-Starting-SFN-AndSubframe and musim-GapLength.

13: If UE indicates the musim-PrefStarting-SFN-AndSubframe when requesting aperiodic gaps, the network can only configure the aperiodic Gap with the same start point or no aperiodic gap. If the field musim-PrefStarting-SFN-AndSubframe is absent for aperiodic gaps, network can configure any timing. This should be captured in RRC.

Can discuss harmonization of MUSIM gap signaling e.g. as proposed in R2-2205759 in RRC CR discussion.

1	Introduce a conditional mandatory UE capability without capability bit for support of Paging cause in RAN Paging.

Bulk agreements

1: Adopt the TP with the following changes: "Paging cause in RAN paging message: It is mandatory for a UE to support paging cause in RAN paging if UE supports paging cause in CN paging."

2a: Update to update the description of musimGapPreference-r17 as follows: "musimGapPreference-r17: Indicates whether the UE supports providing MUSIM assistance information with MUSIM gap preference and related MUSIM gap configuration, as defined in TS 38.331 [9]."

3: Do not to introduce “without leaving RRC Connected” in the description of musimGapPreference-r17 capability.

2b: The following should be added to the description of musimGapPreference-r17: "UE supporting this feature supports 3 periodic gaps and 1 aperiodic gap."

Capture the above in draft CRs, which will be endorsed (unseen). Discuss Stage-2 correction in post-meeting email.

- RAN2#117-e

As requested by SA2 LS, RAN2 will use same formula with reference to SA2 specification 23.401 for calculation of alternative IMSI. CRs to be updated accordingly.

1:    Introduce gap ID in RRCReconfiguration message for MUSIM to identify each configured periodic gap, and support modification or release of configured gaps via gap ID. And adopt the list with ToAddModList/ToReleaseList in RRCReconfiguration for the scheduling gap configuration. FFS how to handle aperiodic gap configurations.

2:    If the UEAssistanceInformation does not include a field for MUSIM gap preference, it indicates no preference (i.e. no need for the gaps) for the corresponding field for MUSIM gap.

4:    Configuration of MUSIM leave assistance is optional, but if configured, always contains at least the MUSIM leave without response timer as mandatory field. When MUSIM leave assistance is configured, UE can indicate MUSIM assistance information for leaving RRC_CONNECTED (i.e. when UE indicates MUSIM leave, UE will leave when the MUSIM leave without response timer expires or it receives RRCRelease).

6:    The preferred RRC state indicator for switching notification with leaving RRC Connected includes state {RRC_IDLE, RRC_INACTIVE, outOfConnected}

RAN2 will not specify any new behaviour if the wait timer for switching notification to leave RRC connected state is running, and UE detects RLF, triggers re-establishment, receives HO command or triggers CHO. No specification changes are needed.

9:    RAN2 does not specify additional UE behavior on receiving reconfiguration of wait timer while wait timer is running. UE starts/stops/restarts the timer as per legacy procedures for UAI transmission, which means that at least in some cases this is left up to UE implementation.

10:  UE stores musim-GapAssistanceConfig, musim-LeaveAssistanceConfig and musim-GapConfig when entering RRC_INACTIVE state. Upon initiation of RRC connection resume, the UE releases musim-GapAssistanceConfig and musim-LeaveAssistanceConfig from the UE Inactive AS context.

11:  Upon initiation of RRC connection re-establishment, the UE releases musim-GapAssistanceConfig and musim-LeaveAssistanceConfig, if configured.

12:  Upon reception of the RRCReestablishment message, the UE releases the gap configuration indicated by the musim-GapConfig, if configured.

1: Network should always provide at least one of the requested gap pattern or no gaps.  Network providing an alternative gap pattern instead of the one requested by the UE is not supported in this release.

1: The UE is not allowed to report its preferred RRC state to network for MUSIM purpose while musim-LeaveWithoutResponseTimer is running. The following TP can be considered as baseline:

- 2> if configured to provide MUSIM assistance information for leaving RRC_CONNECTED:

- 2> if the UE needs to leave RRC_CONNECTED state and timer T3xx is not running:

- 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide MUSIM assistance information for leaving RRC_CONNECTED;

- 3> start the timer T3xx with the timer value set to the musim-LeaveWithoutResponseTimer;

3: The value range of musim-LeaveWithoutResponseTimer for leaving RRC Connection state is defined as {10ms, 20ms, 40ms, 60ms, 80ms, 100ms, spare2, spare1}.

6: Introduce separate field or IE for aperiodic gap configuration. Details can be discussed during specification implementation phase.

7: Do not introduce an indication in system information to indicate whether busy indication is supported or not.

8: RAN2 does not specify any UE behavior on the interaction between power saving and MUSIM for leaving RRC connection i.e. no specification impact.

5: Aperiodic gap is released implicitly after the gap period is over (i.e. it is configured as a Need N field).

2: If NW releases musim-LeaveAssistanceConfig, UE stops the timer (even if running) (i.e. if UE leaves NW A, it is as per UE implementation-specific operation that is not specified in 3GPP).

4: The prohibit timer range is {0s, 0.1s, 0.2s, 0.3s, 0.4s, 0.5s, 1s, 2s, 3s, 4s, 5s, 6s, 7s, 8s, 9s, 10s}.

1: UE does not need to indicate its supported MUSIM gap patterns in UE AS capability.

2: RAN2 confirms that the paging cause feature is optional (without AS capability signalling). If UE supports NAS capability, also supports corresponding AS parts of the feature. This need not be captured in 3x.306 (can revisit this in the next meeting if issues are found).

## RAN2#116-bis-e

1	No new independent section specific to MUSIM needs to be created in TS 36.300.

3	A new section for MUSIM can be created in TS 38.300 (should be checked with specification rapporteur).

2	The paging cause does not need to be described in TS 36.300.

4	The paging cause does not need to be described in TS 38.300.

5	The remaining FFS and Editor’s note can be removed from TS 38.300

1: Working assumption: RAN2 will define alternative IMSI = (IMSI + Accepted IMSI Offset) in 36.304. If an Accepted IMSI Offset is forwarded by upper layers, UE AS shall use the IMSI Offset value and IMSI to calculate an alternative IMSI value as IMSI + Accepted IMSI Offset. Here IMSI is used for the UE ID in paging offset calculation.

2: Send LS to SA2/CT1 notifying them of RAN2 working assumption and ask if the working assumption is compatible and consistent with SA2/CT1 specifications (with minimal effort). If not, request them to indicate that to RAN2 so the topic can be rediscussed. LS to be drafted in (short) post-meeting email discussion.

For P3 and P5, can discuss in open issue collection whether there is something to address.

4: RAN2 do not define AS-NAS interaction on when and how UE indicate paging collision.

1: From RAN2 perspective, at least the following MGL/MGRP values are applicable for MUSIM periodic gap:

MGL: 1.5ms, 3ms, 3.5ms, 4ms, 5.5ms, 6ms, 10ms, 20ms

MGRP: 20ms, 40ms, 80ms, 160ms, 320ms, 640ms, 1280ms, 2560ms.

Can add additional MGL/MGRP if RAN4 indicates other values are needed

2: From RAN2 perspective, at least the following MGL values are applicable for MUSIM aperiodic gap.

MGL: 1.5ms, 3ms, 3.5ms, 4ms, 5.5ms, 6ms, 10ms, 20ms

Can add additional MGL/MGRP if RAN4 indicates other values are needed

3: keep three gaps agreement (i.e., 2 periodic gaps and 1 aperiodic gap) for now. Ask to RAN4 to clarify if one additional periodic gap can be possible without sacrificing NW A performance (exact LS wording for the question can be discussed offline).

4: In the gap assistance information, UE provides gap repetition period and offset for periodic gaps, and (optionally) provides start SFN and subframe for the aperiodic gap.

10: NW configures start SFN and subframe for the aperiodic gap.

7: UE is allowed to initiate a UAI message with MUSIM preference in the target cell after handover, if the UE has sent UAI during the last 1 second.

Stage-3 details for gap configuration (e.g AddModReleaseList, gap id, gap modification) are postponed for now (pending the general MG discussion). Can consider P8/P9 as starting point from MUSIM perspective.

11: Send LS to RAN4 on gap related agreements (Offline 233, vivo).

NW is allowed to configure prohibit timer for MUSIM UAI, but it has to be allowed to be set to zero (i.e. no prohibit timer). FFS what is the maximum value (should be reasonable)

1: For NW switching with leaving RRC Connected state, RAN2 confirms the following understanding (aligned with SA2 agreements):

- 1. For E-UTRAN/5GS scenario, only NAS-based solution is supported for UE network switching with leaving connected state.

- 2. For NR/5GS scenario, both NAS-based and RRC-based solution are supported for UE network switching with leaving connected state.

- 3. For NAS-based UE network switching with leaving connected state case, UE may provide a Paging Restriction Information to AMF only by NAS signaling.

- 4. For RRC-based UE network switching with leaving connected state case, it is NOT supported to provide the Paging Restriction Information from a UE to RAN by RRC message.

- 5. There is no need to define the interaction between RRC-level connection release procedure and NAS-level connection release procedure.

- 6. When both NAS-level Connection Release and RRC-level connection release are supported by the UE and are configured by the NW, it is up to the UE implementation to determine which one to use.

- 7. When NAS-based solution is used, the UE can only enter IDLE, while if RRC-based solution is used, then the UE can enter IDLE or INACTIVE.

4: Do not introduce a prohibit timer for RRC-level switching procedure for leaving RRC_CONNECTED state.

5: The waiting timer for leaving RRC Connection state should be set to a finite value which can allow the UE to switch in a reasonable time and not delay the urgent procedures on the other NW. Network controls whether UE is allowed to use leaving RRC connection for MUSIM purposes.

2: Switching notification for leaving RRC_CONNECTED state and without leaving RRC_CONNECTED state can be enabled separately.

1: Two capability bits are introduced, one for support gaps and another for “leaving connected”.  There is no need for different capability bits for periodic and aperiodic gaps.

2: Confirm that the MUSIM related capability is per UE (without FRx and xDD differentiation).

## RAN2#116-e

4: RAN2 confirms that E-UTRA connected to 5GC scenario is also in the WID scope for paging collision avoidance. The solution agreed for NR is the baseline solution for this scenario.

1: RAN2 prefers that for EPS, the alternative IMSI or offset should be calculated in AS, i.e., RRC. Send an LS to SA2 and CT1 to indicate RAN2’s preference and request to specify the necessary details. LS will be discussed in offline [230].

Option 1 already agreed earlier, no need to optimize

5: For LTE and NR, RAN2 leaves it up to UE implementation how UE AS indicates to UE NAS that paging collision issue is identified.

7: For LTE and NR, RAN2 leaves other detailed UE behavior up to UE implementation, including how to make predictable UE behavior for RAT/Network selection to avoid paging collision, rules for declaring paging collision issue, and RAT/Network selection for reporting paging collision issue.

=> RAN2 already agreed not to have assistance information

After the session, it was noted that the agreement on no assistance information only applied for 5GS, so the notes were amended by adding the "for 5GS" as per below:

RAN2 already agreed not to have assistance information for 5GS

RAN2 will not specify MN-SN coordination of MUSIM gaps with MR-DC in Rel-17

RAN2 will not create MAC CE activation of gaps in MUSIM, but if the common gap discussion allows this anyway, RAN2 will not prevent that, either.

1: RAN2 will not work in Rel-17 for the case that Dual-RX/Single-TX UE or Single-RX/Single-TX UE stays in RRC_CONNECTED mode in NW A while performing reception and transmission in NW B (in RRC_ CONNECTED or during RRC setup/resume period).

2: MR-DC is not supported in Rel-17.

No need for LS to SA2 on this (no specification efforts needed to prevent or allow dual RRC_CONNECTED with MUSIM in Rel-17).

Wait for RAN4 feedback on gap pattern support (can use FFS in RRC for maximum value)

4: RAN2 understands that the intent of aperiodic gap is as follows (no need to specify):

- -	If until the end of the aperiodic gap the UE still has not completed activity in NW B, e.g. due to the random access for on-demand SI request, the UE should stop the activity in NW B and switch to NW A. If needed, the UE can request another aperiodic gap in NW A.

RAN2 does not intend to specify any new signalling in Rel-17 for early return. If legacy signalling allows it, RAN2 does not intend to preclude it.

5: Do not introduce gap purpose for gap related MUSIM assistance information.

6: FFS how UE indicates release of gap pattern.

7: FFS if UE is allowed to update UAI message after the UE performs cell reselection in NW B or after the UE performs handover in NW A.

8: Autonomous release of MUSIM gap by UE after N repetitions is not supported.

Bulk agreements

1: Introduce paging cause by using the ”nonCriticalExtension” in the Paging record.

2: No need to study solution proposals based on extending legacy Paging record.

4: The solution proposal to introduce paging cause in NR will be used for LTE.

5: No need to send an LS to SA2 asking to consider a NAS solution to introduce paging cause in LTE.

6: For paging reception in RRC_IDLE, UE forwards the paging cause to NAS.  It’s up to NAS whether to accept or reject the paging.

8: The AS-NAS interaction principles for NR are applied to LTE.

No LS to SA3 needed on the LTE solution. Companies can raise this up directly in SA3 if needed.

3: Adopt B.1 (parallel list with 1 optional paging cause value “voice”).

9: Introduction of paging cause impacts 38.331 and 36.331 specs; FFS if it impacts stage 2 specs (38.300 and 36.300)

7: The AS-NAS interaction for paging reception in RRC_INACTIVE is left up to UE implementation.

1	AS capability for paging collision avoidance is not needed (for any cases).

2	There is no need for AS capability for Busy indication.

6	There is no need for AS capability for Paging cause value.

Can discuss UE capabilities for periodic/aperiodic gap request and RRC processing delay requirements for MUSIM in Rel-17 further in the next RAN2 meeting.

The below is used as baseline for MUSIM capabilities (can still discuss exact details in the next meeting). FFS whether we need separate bits for periodic and aperiodic gaps. FFS if we need capability bit for leaving RRC_CONNECTED.

| Features | Index | Feature group | Components | Prerequisite feature groups | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x. Rel-17 MUSIM | X-0 | Gap support for MUSIM UE | Indicates UE support periodic gap for MUSIM | MUSIM support over NAS | UE | No | No |  |  | Optional with capability signalling |
|  |  |  | Indicates UE support aperiodic gap for MUSIM | MUSIM support over NAS | UE | No | No |  |  | Optional with capability signalling |

## RAN2#115-e

- Agreement

- 1	RAN2 retains the agreement on NAS-based busy indication for RRC_INACTIVE, and Reply SA2.

- Agreements

- Scenarios and supported gap types

- 1	RAN2 aims to support at least the below scenarios 1/2/3 in Rel-17 for cases when the UE is allowed to switch to network B without leaving connected state at network A.

- -	Scenarios 1: Periodic switching, including SSB detection/paging reception, serving cell measurement, neighboring cell measurement including intra-frequency,inter-frequency and inter-RAT measurement;

- -	Scenarios 2:  SI receiving at network B;

- -	Scenarios 3: Aperiodic (one-shot) switching with both transmission and reception at network B but will not enter RRC-connected state in NW B (e.g. no RRC connection Resume/Setup) at network B, including On-demand SI request;

- 2	For switching without leaving connected state at network A, support gap types 2a (Normal periodic gap) and 2b (Normal aperiodic gap) in Rel-17.

- 3	Only per UE level scheduling gap is supported in Rel-17 for non-DC. FFS if we support MR-DC.

- The scenarios will only be used for deriving RRC parameters. No need to capture them in e.g. Stage-2.

- Agreements

- Gap configuration and activation

- 5	The network is allowed to configure at most 3 gap patterns (for any MUSIM purpose).

- 6	Only a single aperiodic gap (for MUSIM) is supported in Rel-17. At most two periodic “gaps” (for MUSIM) and a single aperiodic gap (for MUSIM) is supported in Rel-17. FFS if signalling supports more.

- 7  	The SFN and subframe of the PCell of the network A is used in the gap configuration to calculate the gap.

- Agreements

- Periodic/Aperiodic/autonomous Gap configuration and activation

- 8:  The switching gap configuration will explicitly provide the gap starting position (e.g. offset value or start SFN and subframe explicitly), gap length and gap repetition period.

- 10:  Switching Gaps (of any type) are configured or released by RRC signalling (e.g. RRCReconfiguration message) in Rel-17. FFS if gap can be released autonomously by UE after N repetitions.

- Gap configuration assistance information

- 16 	UE is allowed to include assistance information for setup or release of gaps for both 1) periodic gaps and 2) aperiodic gap in one UEAssistanceInformation Msg.

- 18  To report the assistance information, the UE maps the timing info of the Gap on the network B  to the network A and reports the mapped timing info to the network A.

- 20  For the gap assistance information, the Gap start time, Duration of the gap and gap repetition period (for periodic) may be included. FFS is other information is included (e.g. gap purpose).

Do not support autonomous gaps for MUSIM in Rel-17.

1	UE can indicate it wants to leave RRC_CONNECTED in assistance information for MUSIM (FFS for signalling details, e.g. UAI).

3: UEAssistanceInformation message is extended for switching notification in both network switching procedures for leaving RRC_CONNECTED state and without leaving RRC_CONNECTED state.

6: UE is configured to provide assistance info for switching notification via otherConfig of RRCReconfiguration message

8: Introduce a new RRC timer for the “configured time”, used for the UE to leave RRC_CONNECTED without a response.

FFS if it's possible to configure UE to always wait for the network response (e.g. "infinite" waiting time)

7: UE is not allowed to enter RRC_INACTIVE state if no NW response message is received within a certain configured time period after the network switching notification message is sent.

9: As baseline, how to handle the case, that UE performs switching without the response from network for a configured time during switching procedure without leaving RRC_CONNECTED state, is not specified. Can re-discuss if there are serious issues found.

## RAN2#114-e

1: Send an LS to SA2 to inform that RAN2 majority would support, but there is no consensus to support NAS assistant information (similar to UE ID offset for LTE), so RAN2 thinks this issue should be discussed and decided by SA2.

2: RAN2 does not introduce RRC assistant information for paging collision issue for IDLE and INACTIVE. (Can revisit if serious problems are found.)

1: RRC signaling for network switching without leaving RRC_Connected state should allow multiple configurations of periodic “gaps” with different parameters (e.g. periodicities and durations). FFS is multiple can be active at the same time. FFS if multiple aperiodic gaps are supported.

4: UE provides assistance information to the gNB of NW A in Connected state based on the configuration of USIM of NW B for the gNB to determine the necessary switching parameters. Up to network what is the action based on UE assistance information. FFS what assistance information is needed.

1: AS -based solution for network switching includes two steps: 1-) If configured, UE can send an RRC message to leave RRC_CONNECTED for MUSIM purpose 2-) gNB may release the UE to Idle/Inactive.

2: Include the following RAN2#113bis-e agreement in the LS:

- During switching procedure for leaving RRC_CONNECTED state, UE is allowed to enter RRC_IDLE state if it does not receive response message from network within a certain configured time period. FFS for RRC_INACTIVE state

3: The “configured time” for AS-based solution for the UE to leave RRC_CONNECTED without a response is configured by the gNB. Indicate RAN2 is still discussing this for AS-based solution in the LS.

4: Indicate that RAN2 has not discussed the interaction between AS-based solution and any SA2 agreement on NAS messages or NAS-based solution for network switching.

## RAN2#113-bis-e

- Agreements

- 1: For the EPS PO/PF calculation, include the UE_offset to the UE_ID calculation formula.

- 2: No additional modification for the EPS eDRX case.

- Agreements

- 1	Only support NAS-based busy indication (for IDLE and INACTIVE)

- Agreements

- 1	RRC signalling is used for switching procedure without leaving RRC_CONNECTED state in network A for UE temporarily switching to network B as a baseline. FFS on additional need of MAC signalling.

- 2	During switching procedure for leaving RRC_CONNECTED state, UE is allowed to enter RRC_IDLE state if it does not receive response message from network within a certain configured time period. FFS for RRC_INACTIVE state.

1: RAN2 works to support the MUSIM paging cause feature that SA2 is working on and also addresses the paging cause issue raised by SA2 LS.

2: RAN2 attempts to reply LS to SA2 once we progress on solution and agree on CR(s) that support/address the above feature/issue.

5: If RAN2 agrees to add a paging cause value (or any other information that could lead to a specific paging cause) in Uu paging message, RAN2 specifies the relevant UE behavior (i.e. inform or passing to the upper layer) upon its reception in both LTE and NR specifications.

RAN2 does not intend to introduce alternative paging IDs for MUSIM paging (unless requested by SA2).

## RAN2#113-e

There is support for solution 1 (for 5GS) with something else, either solution 3 or 2b.

- Agreement

- Option 2b is the preferred solution to address paging collision for “LTE + LTE”.

- Agreements

- 1	MUSIM UE determines potential paging collision on two networks and triggers actions on potential paging collision avoidance.

- 2	It is left to UE implementation as to how it selects one of the two RATs/networks for paging collision avoidance.

FFS if we can make the UE behaviour predictable for paging collision avoidance

- Agreements

- 1	Switching procedure can be used to notify network A that the UE has a preference to leave RRC_CONNECTED state in network A.

- 2	The switching procedure can be used to notify network A that the UE has a preference to be kept in RRC_CONNECTED state in network A while temporarily switching to network B.

- Agreement

- 1	NAS signalling is baseline for UE reporting paging collision in 5GS side (to be confirmed by SA2).

- 2	It is FFS whether assistant information is needed for paging collision in 5GS side.

## RAN2#112-e

Use: "Extending paging signalling is possible but RAN2 haven’t decided on overall feasibility of paging cause, including how it should be supported."

With this change, the LS is approved in R2-2011241 (unseen)

From RAN2 point of view, Option 1 , 2a, 2b, and 3 are feasible to solve the paging collision issue in 5GS. Each have different effectiveness (as per analysis during the email discussion). When indicating reply to SA2, indicate both feasibility as well as effectiveness.

Indicate to SA2 that RAN2 continues to further evaluate the pros and cons of options 1, 2a, 2b, 3.

Option 4 is still allowed (but RAN2 will not specify UE implementation).

Clarifying "No E-UTRA impact" can be done in RANP.

Option 2c can be evaluated later as it doesn't work alone.

Enhancement for 5GS should be prioritized since it can handle paging collision issue in both NR+NR and NR+LTE scenarios.

Indicate to SA2 that the table 1 is a baseline on the discussion the expected time (in ms) required for UE to send a (NAS) busy indication to Network B.

From RAN2 point of view, it is feasible that the busy indication is sent as an RRC message with security for RRC_INACTIVE. FFS how this works.

RAN2 will continue to discuss RRC-based switching/leaving and returning procedure in 5GS/NR when UE is in RRC_CONNECTED. There may be different mechanisms (short/long, leaving/returning, etc.).

Provide SA2 with information on paging cause costs based on the email discussion + contributions. Indicate that this may change if assumptions change.

From RAN2 perspective, we haven't decided on paging cause feasibility yet.

RAN2 will evaluate short/long time switching in this WI

Agreements

1a: The sub-Case 3-1 is supported in WI, i.e., the switching/leaving and returning procedure in 5GS/NR when UE is in RRC_CONNECTED  includes the case where Dual-RX/Single-TX UE is in RRC_CONNECTED state in NW A while performing only reception in NW B (i.e., in RRC_idle State and RRC inactive state).

1b: For Sub-Case 3-1, whether the Rx capability coordination between UE and NW is needed can be decided after the RRC-based switching/leaving and returning procedure is defined.

2: The Sub-Case 3-2, i.e. Dual-RX/Single-TX UE stays in RRC_CONNECTED mode in NW A while performing reception and transmission in NW B(in RRC_ CONNECTED or during RRC setup/resume period ), is not considered in the WI from RAN2 viewpoint. Scheduling gap is not excluded.

4: 	FFS: The Sub-Case 4-1, i.e. Dual-RX/Dual-TX UE stays in RRC_CONNECTED mode in NW A while performing both reception and transmission in NW B without changing into RRC_CONNECTED state in NW B, is not considered in the WI from RAN2 viewpoint.

5: 	FFS: The Sub-Case 4-2, i.e. Dual-RX/Dual-TX UE stays in RRC_CONNECTED state in NW A while performing both reception and transmission in RRC_ CONNECTED in NW B, is not considered in the WI from RAN2 viewpoint.

=>	FFS if/how to ensure UE doesn't disconnect from RRC_CONNECTED during busy indication

=>	Capability change is not precluded by proposals.