3GPP TSG-RAN WG2 #119	R2-2208069

Electronic meeting, Aug 17th – 29th, 2022

Agenda Item:	5.3.3

Source:	Ericsson

Title:	Correction of TRP beam information field descriptions for UEB DL-AoD

Document for:	Discussion, Decision

# 1	Introduction

Support for DL-AoD UE-based positioning was introduced in Rel 16 via the provisioning of instances of the NR-DL-PRS-BeamInfo IE and refined in Rel 17 via the provisioning of instances of the NR-TRP-BeamAntennaInfo IE. In order to reduce signaling, the intention was to enable associations between configurations so that the same configuration information only needs to be provided once, and instead referred to. These references are represented by an associated DL-PRS ID, indicating the referenced TRP.

In order to be able to get all the benefits of this feature, two TRPs can use the same configurations except that the orientation of the antenna is specific per TRP. Therefore, the lcs-GCS-TranslationParameter shall be possible to provide for a TRP even when it is configured with an associated DL-PRS ID.

The encoding of this feature was correctly introduced in Rel 17 NR-TRP-BeamAntennaInfo IE, but there are still issues with the Rel 16 NR-DL-PRS-BeamInfo IE. This paper discusses how these issues can be resolved.

# 2	Discussion

## 2.1	Signaling reduction via associated DL-PRS ID

The reason for introducing the associated DL-PRS ID was to ensure that information does not need to be repeated, but instead configured once and then referenced to via an association. The NR-DL-PRS-BeamInfo IE describes the beam info configuration of all beams of a specific TRP. Typically, these antenna panels will be the same to a large extent in a network deployment. However, they will not necessarily be oriented in space in the  same way, so the orientation can still be TRP specific. This means that the antenna panel can be defined in in a generic way in a local coordinate system that matches all TRPs or at least several TRPs, while they will still be associated with a TRP specific orientation, described by a local to global coordinate transformation.

The TRP beam information is typically the same when it comes to the beam configurations in a local coordinate system, but the orientation can be TRP specific

There is a need to be able to configure a TRP specific orientation also when using an associated DL-PRS ID reference

In the current field description of the associated DL-PRS ID is

associated-DL-PRS-ID This field specifies the dl-PRS-ID of the associated TRP from which the beam information and parameters for LCS to GCS translation are adopted. If the field is omitted, the beam information is provided via the dl-prs-BeamInfoSet field and the LCS to GCS translation parameter is provided via the lcs-GCS-TranslationParameter. If the field is present, the fields lcs-GCS-TranslationParameter and dl-PRS-BeamInfoSet shall be absent.

Hence, currently the field description does not expect the use of the associated DL-PRS ID together with the TRP-specific orientation provided via the lcs-GCS-TranslationParameter IE. This is unfortunate since this is the typically case, where the beams of the panel is defined once, and then only a TRP specific orientation is needed for the rest of the TRPs, corresponding to a huge saving in signalling costs.

The current field description for the associated DL-PRS ID excludes the use of a TRP specific orientation when using the associated DL-PRS ID

## 2.2	Backwards compatibility

Since the NR-DL-PRS-BeamInfo IE was introduced in Rel 16, the change of field description needs to be analyzed for backwards compatibility

- Location server is updated, device is not. Location server will provide beam info per TRP where some instances of NR-DL-PRS-BeamInfoPerTRP will not include dl-PRS-BeamInfoSet-r16 but will include both associated-DL-PRS-ID-r16 and lcs-GCS-TranslationParameter-r16, while the UE does not expect lcs-GCS-TranslationParameter-r16. Assuming that the device will neglect the provided lcs-GCS-TranslationParameter-r16, the device will instead use the orientation of the associated TRP, which will result in an incorrect position estimate

- Location server is not updated, device is. The location server can only use the associated DL-PRS ID alternative for TRPs with the same beam configuration and antenna panel orientation, which will imply significantly higher signaling costs, but will not impact the positioning performance.

In the case when the location server supports updated field descriptions while the device does not, there can be issues with the positioning performance

Given the issue with backward compatibility, it seems advisable to introduce a new capability indicating support for the combination of associated-DL-PRS-ID-r16 and lcs-GCS-TranslationParameter-r16. At the same time, the current field description was adjusted as an incorrect correction at RP-92 [1], which could motivate to correct this without a capability.  Hence, either we correct the field descriptions without capability or we introduce a capability, for example either as a

- Option 1. Correct the field description without any capability

- Option 2. Correct the field description with a specific new bit enhBeamInfoSup in the field nr-PosCalcAssistanceSupport-r17

- Option 3. Correct the field description with a specific new capability enhBeamInfoSup-r16

This leads to the following proposals

Correct the behaviour description of associated- DL-PRS-ID-r16 and lcs-GCS-TranslationParameter-r16 using one of the Options below

- Option 1. Correct the field description without any capability

- Option 2. Correct the field description with a specific new bit enhBeamInfoSup in the field nr-PosCalcAssistanceSupport-r17

- Option 3. Correct the field description with a specific new capability enhBeamInfoSup-r16

If Option 2 or 3 is agreed, introduce a new capability indicating support for the combination of associated-DL-PRS-ID-r16 and lcs-GCS-TranslationParameter-r16.

Encode the new capability for supporting the combination of associated-DL-PRS-ID-r16 and lcs-GCS-TranslationParameter-r16 possibly with either Option 2: a specific bit enhBeamInfoSup of the nr-PosCalcAssistanceSupport-r17 field, or Option 3: a specific capability enhBeamInfoSup-r16.

Agree to the LPP text proposal in the appendix, adjusted for which capability option that is agreed, if any

# Conclusion

In the previous sections we made the following observations:

Observation 1	The TRP beam information is typically the same when it comes to the beam configurations in a local coordinate system, but the orientation can be TRP specific

Observation 2	There is a need to be able to configure a TRP specific orientation also when using an associated DL-PRS ID reference

Observation 3	The current field description for the associated DL-PRS ID excludes the use of a TRP specific orientation when using the associated DL-PRS ID

Observation 4	In the case when the location server supports updated field descriptions while the device does not, there can be issues with the positioning performance

Based on the discussion in the previous sections we propose the following:

Proposal 1	Correct the behaviour description of associated- DL-PRS-ID-r16 and lcs-GCS-TranslationParameter-r16 using one of the Options below

	Option 1. Correct the field description without any capability

	Option 2. Correct the field description with a specific new bit enhBeamInfoSup in the field nr-PosCalcAssistanceSupport-r17

	Option 3. Correct the field description with a specific new capability enhBeamInfoSup-r16

Proposal 2	If Option 2 or 3 is agreed, introduce a new capability indicating support for the combination of associated-DL-PRS-ID-r16 and lcs-GCS-TranslationParameter-r16.

Proposal 3	Encode the new capability for supporting the combination of associated-DL-PRS-ID-r16 and lcs-GCS-TranslationParameter-r16 possibly with either Option 2: a specific bit enhBeamInfoSup of the nr-PosCalcAssistanceSupport-r17 field, or Option 3: a specific capability enhBeamInfoSup-r16.

Proposal 4	Agree to the LPP text proposal in the appendix, adjusted for which capability option that is agreed, if any

# 4	References

[1] 	37355_CR0294r2_(Rel-16)_R2-2104796

# Appendix – Text Proposal to 37.355

### 6.4.3	Common NR Positioning Information Elements

[…]

#### –	NR-DL-PRS-BeamInfo

The IE NR-DL-PRS-BeamInfo is used by the location server to provide spatial direction information of the DL-PRS Resources.

-- ASN1START

NR-DL-PRS-BeamInfo-r16 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF

NR-DL-PRS-BeamInfoPerFreqLayer-r16

NR-DL-PRS-BeamInfoPerFreqLayer-r16 ::= SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

NR-DL-PRS-BeamInfoPerTRP-r16

NR-DL-PRS-BeamInfoPerTRP-r16 ::= SEQUENCE {

dl-PRS-ID-r16						INTEGER (0..255),

nr-PhysCellID-r16					NR-PhysCellID-r16		OPTIONAL,	-- Need ON

nr-CellGlobalID-r16					NCGI-r15				OPTIONAL,	-- Need ON

nr-ARFCN-r16						ARFCN-ValueNR-r15		OPTIONAL,	-- Need ON

associated-DL-PRS-ID-r16			INTEGER (0..255)		OPTIONAL,	-- Need OP

lcs-GCS-TranslationParameter-r16	LCS-GCS-TranslationParameter-r16

OPTIONAL,	-- Need OP

dl-PRS-BeamInfoSet-r16				DL-PRS-BeamInfoSet-r16	OPTIONAL,	-- Need OP

...

}

DL-PRS-BeamInfoSet-r16 ::= SEQUENCE (SIZE(1..nrMaxSetsPerTrpPerFreqLayer-r16)) OF

DL-PRS-BeamInfoResourceSet-r16

DL-PRS-BeamInfoResourceSet-r16 ::= SEQUENCE (SIZE(1..nrMaxResourcesPerSet-r16)) OF

DL-PRS-BeamInfoElement-r16

DL-PRS-BeamInfoElement-r16 ::= SEQUENCE {

dl-PRS-Azimuth-r16				INTEGER (0..359),

dl-PRS-Azimuth-fine-r16			INTEGER (0..9)					OPTIONAL,	-- Need ON

dl-PRS-Elevation-r16			INTEGER (0..180)				OPTIONAL,	-- Need ON

dl-PRS-Elevation-fine-r16		INTEGER (0..9)					OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

NR-DL-PRS-Beam-Info field descriptions

dl-PRS-ID This field is used along with a DL-PRS Resource Set ID and a DL-PRS Resources ID to uniquely identify a DL-PRS Resource. This ID can be associated with multiple DL-PRS Resource Sets associated with a single TRP. Each TRP should only be associated with one such ID.

nr-PhysCellID This field specifies the physical cell identity of the associated TRP, as defined in TS 38.331 [35].

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, of the associated TRP, as defined in TS 38.331 [35]. The server should include this field if it considers that it is needed to resolve ambiguity in the TRP indicated by nr-PhysCellID.

nr-ARFCN This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID.

associated-DL-PRS-ID This field specifies the dl-PRS-ID of the associated TRP from which certain field data is obtained, see the field descriptions of dl-PRS-BeamInfoSet and lcs-GCS-TranslationParameter fields.

lcs-GCS-TranslationParameter This field provides the angles α (bearing angle), β (downtilt angle) and γ (slant angle) for the translation of a Local Coordinate System (LCS) to a Global Coordinate System (GCS) as defined in TR 38.901 [44]. If this field and the field associated-DL-PRS-ID are absent, the dl-PRS-Azimuth and dl-PRS-Elevation are provided in a GCS. If this field is absent and the associated-DL-PRS-ID field is present, then the lcs-GCS-TranslationParameter for this TRP is obtained from the lcs-GCS-TranslationParameter of the associated TRP.

dl-PRS-BeamInfoSet This field provides the DL-PRS beam information for each DL-PRS Resource of the DL-PRS Resource Set associated with this TRP. If this field is absent and the field associated-DL-PRS-ID is present, the dl-PRS-BeamInfoSet for this TRP are obtained from the dl-PRS-BeamInfoSet of the associated TRP.

dl-PRS-Azimuth This field specifies the azimuth angle of the boresight direction in which the DL-PRS Resources associated with this DL-PRS Resource ID in the DL-PRS Resource Set are transmitted. For a Global Coordinate System (GCS), the azimuth angle is measured counter-clockwise from geographical North. For a Local Coordinate System (LCS), the azimuth angle is measured measured counter-clockwise from the x-axis of the LCS. Scale factor 1 degree; range 0 to 359 degrees.

dl-PRS-Azimuth-fine This field provides finer granularity for the dl-PRS-Azimuth. The total azimuth angle of the boresight direction is given by dl-PRS-Azimuth + dl-PRS-Azimuth-fine. Scale factor 0.1 degrees; range 0 to 0.9 degrees.

dl-PRS-Elevation This field specifies the elevation angle of the boresight direction in which the DL-PRS Resources associated with this DL-PRS Resource ID in the DL-PRS Resource Set are transmitted. For a Global Coordinate System (GCS), the elevation angle is measured relative to zenith and positive to the horizontal direction (elevation 0 deg. points to zenith, 90 deg to the horizon). For a Local Coordinate System (LCS), the elevation angle is measured relative to the z-axis of the LCS (elevation 0 deg. points to the z-axis, 90 deg to the x-y plane). Scale factor 1 degree; range 0 to 180 degrees.

dl-PRS-Elevation-fine This field provides finer granularity for the dl-PRS-Elevation. The total elevation angle of the boresight direction is given by dl-PRS-Elevation + dl-PRS-Elevation-fine. Scale factor 0.1 degrees; range 0 to 0.9 degrees.

[…]

#### 6.5.11.6	NR DL-AoD Capability Information

#### –	NR-DL-AoD-ProvideCapabilities

The IE NR-DL-AoD-ProvideCapabilities is used by the target device to indicate its capability to support NR DL-AoD and to provide its NR DL-AoD positioning capabilities to the location server.

-- ASN1START

NR-DL-AoD-ProvideCapabilities-r16 ::= SEQUENCE {

nr-DL-AoD-Mode-r16						PositioningModes,

nr-DL-AoD-PRS-Capability-r16			NR-DL-PRS-ResourcesCapability-r16,

nr-DL-AoD-MeasurementCapability-r16		NR-DL-AoD-MeasurementCapability-r16,

nr-DL-PRS-QCL-ProcessingCapability-r16	NR-DL-PRS-QCL-ProcessingCapability-r16,

nr-DL-PRS-ProcessingCapability-r16		NR-DL-PRS-ProcessingCapability-r16,

periodicalReporting-r16					PositioningModes							OPTIONAL,

...,

[[

ten-ms-unit-ResponseTime-r17			PositioningModes							OPTIONAL,

nr-PosCalcAssistanceSupport-r17			BIT STRING {	trpLocSup 		(0),

beamInfoSup		(1),

rtdInfoSup		(2),

beamAntInfoSup	(3),

enhBeamInfoSup	(4)

}	(SIZE (1..8))				OPTIONAL,

nr-los-nlos-AssistanceDataSupport-r17	SEQUENCE {

type-r17		LOS-NLOS-IndicatorType2-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity2-r17,

...

}											OPTIONAL,

nr-DL-PRS-ExpectedAoD-or-AoA-Sup-r17	BIT STRING {	eAoD 		(0),

eAoA		(1)

}	(SIZE (1..8))				OPTIONAL,

nr-DL-PRS-BeamInfoSup-r17				ENUMERATED { sameSet, differentSet, sameOrDifferentSet }

OPTIONAL,

dl-PRS-ResourcePrioritySubset-Sup-r17	ENUMERATED { supported }					OPTIONAL,

nr-DL-AoD-On-Demand-DL-PRS-Support-r17	NR-On-Demand-DL-PRS-Support-r17				OPTIONAL,

nr-los-nlos-IndicatorSupport-r17		SEQUENCE {

type-r17		LOS-NLOS-IndicatorType2-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity2-r17,

...

}											OPTIONAL,

scheduledLocationRequestSupported-r17	ScheduledLocationTimeSupportPerMode-r17

OPTIONAL,

nr-dl-prs-AssistanceDataValidity-r17	SEQUENCE {

area-validity-r17	INTEGER (1..maxNrOfAreas-r17)			OPTIONAL,

...

}															OPTIONAL,

multiMeasInSameMeasReport-r17			ENUMERATED { supported }					OPTIONAL,

mg-ActivationRequest-r17				ENUMERATED { supported }					OPTIONAL

]],

[[

enhBeamInfoSup-r16					ENUMERATED { supported }					OPTIONAL

]]

}

-- ASN1STOP

NR-DL-AoD-ProvideCapabilities field descriptions

nr-DL-AoD-Mode This field specifies the NR DL-AoD mode(s) supported by the target device.

periodicalReporting This field, if present, specifies the positioning modes for which the target device supports periodicalReporting. This is represented by a bit string, with a one-value at the bit position means periodicalReporting for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

ten-ms-unit-ResponseTime This field, if present, specifies the positioning modes for which the target device supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation. This is represented by a bit string, with a one-value at the bit position means 'ten-milli-seconds' response time unit for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support 'ten-milli-seconds' response time unit in CommonIEsRequestLocationInformation.

nr-PosCalcAssistanceSupport This field indicates the Position Calculation Assistance Data supported by the target device for UE-based DL-AoD. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is supported; a zero-value means not supported. - bit 0 indicates whether the field nr-TRP-LocationInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 1 indicates whether the field nr-DL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 2 indicates whether the field nr-RTD-Info in IE NR-PositionCalculationAssistance is supported or not; - bit 3 indicates whether the field nr-TRP-BeamAntennaInfo in IE NR-PositionCalculationAssistance is supported or not. - bit 4 indicates whether the field nr-DL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is supported with the combination of associated-DL-PRS-ID-r16 and lcs-GCS-TranslationParameter-r16 or not;

nr-los-nlos-AssistanceDataSupport This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedLOS-NLOS-Assistance in IE NR-PositionCalculationAssistance: - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in LOS-NLOS-Indicator in IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance. - granularity indicates whether the target device supports nr-los-nlos-indicator in IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance 'per-trp', 'per-resource', or both.

nr-DL-PRS-ExpectedAoD-or-AoA-Sup This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedAoD-or-AoA in NR-DL-PRS-AssistanceData.

nr-DL-PRS-BeamInfoSup This field, if present, indicates that the target device supports the NR-DL-PRS-BeamInfo in IE NR-DL-AoD-ProvideAssistanceData.

dl-PRS-ResourcePrioritySubset-Sup This field, if present, indicates that the target device supports the DL-PRS-ResourcePrioritySubset in IE NR-DL-PRS-Info. Enumerated value indicates the supported resource set relationship for the target DL-PRS Resource and the associated subset.

nr-DL-AoD-On-Demand-DL-PRS-Support This field, if present, indicates that the target device supports on-demand DL-PRS requests.

nr-los-nlos-IndicatorSupport This field, if present, indicates that the target device supports nr-los-nlos-Indicator reporting in IE NR-DL-AoD-SignalMeasurementInformation. - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in IE LOS-NLOS-Indicator. - granularity indicates whether the target device supports LOS-NLOS-Indicator reporting per TRP, per DL-PRS Resource, or both.

scheduledLocationRequestSupported This field, if present, specifies the positioning modes for which the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the target device does not support scheduled location requests.

nr-dl-prs-AssistanceDataValidity This field, if present, indicates that the target device supports validity conditions for pre-configured assistance data and comprises the following subfields: - area-validity indicates that the target device supports pre-configured assistance data with area validity. The integer number indicates the maximum number of areas the target device supports.

multiMeasInSameMeasReport This field, if present, indicates that the target device supports multiple measurement instances in a single measurement report.

mg-ActivationRequest This field, if present, indicates that the target device supports low latency measurement gap activation request for DL-PRS measurements.

enhBeamInfoSup This field, if present, indicates that the target device supports the combination of the fields associated-DL-PRS-ID-r16 and lcs-GCS-TranslationParameter-r16 both being present in the nr-DL-PRS-BeamInfo of he IE NR-PositionCalculationAssistance.