3GPP TSG-RAN WG2 #119e	R2-2208072

Electronic meeting, 2022-08-17 - 2022-08-29

Agenda Item:	6.11.2.2

Source:	Ericsson

# Title:	On transferring SDT configuration and SRS positioning Inactive configuration from DU to CU

Document for:	Discussion, Decision

# Introduction

RAN3 has sent an LS to RAN2 provided below as reference:

In case of CG-SDT, gNB-DU provides the CG-SDT related resource configuration to gNB-CU within the DU to CU RRC Information IE in F1AP. According to TS 38.331 v17.0.0, there are two RRC IEs for the CG-SDT configuration: - SDT-MAC-PHY-CG-Config-r17 (actual IE that defines CG-SDT configuration generated by DU) - SDT-CG-Config-r17 (OCTET STRING container for SDT-MAC-PHY-CG-Config-r17) RAN3 would like to know which RRC IE should be carried within the DU to CU RRC Information IE in F1AP when the gNB-CU generates the RRCRelease message including suspend configuration. In the same line of question, the SRS Positioning INACTIVE configuration which is generated by gNB-DU is associated with two RRC IEs according to the baseline (R2-2206384): - SRS-PosRRC-InactiveConfig-r17 (actual IE) - SRS-PosRRC-Inactive-r17 (OCTET STRING container for SRS-PosRRC-InactiveConfig-r17) RAN3 also would like to know which RRC IE should be carried in F1AP when the gNB-CU generates the RRCRelease message including suspend configuration.

# Discussion

As per the RAN2 agreements in SDT and positioning session; DU generates the necessary lower layer MAC, PHY parameters and sends to CU as an octet string via F1AP and CU provides the info to UE via RRC.

Considering for positioning; DU would generate SRS-PosRRC-InactiveConfig-r17 whereas CU would generate SRS-PosRRC-Inactive-r17.

The ASN.1 would be encoded as an OS by F1AP when providing the content to CU.

Pointing to CU would imply an embedded OS inside an OS; i.e

SRS-PosRRC-Inactive-r17 ::= OCTET STRING (CONTAINING OS)

The question posed by RAN3 is which IE shall F1AP spec refer to. This seems to be a unique issue as this sort of ASN.1 reference issue whether referring to an OS or nested OS has not happened prior. Hence, it is good to have a general understanding and agreement which can be made norm for any such future ASN.1 work.

As such it should be OK to refer to any of the IE; either pointing to DU (SRS-PosRRC-InactiveConfig-r17) or pointing to CU (SRS-PosRRC-Inactive-r17) as shown in below Figure.

The above may have an advantage if F1AP points to this; this would imply that RRC simply re-uses/copies SRS-PosRRC-Inactive-r17 from F1 and puts in RRC and thus reduces any overhead in CU.

Observation 1:  From RAN2 perspective, both IE can work. If F1AP refers IE on CU side that would imply less    overhead for CU.

Send LS to RAN3 capturing the Observation that “From RAN2 perspective, both IE can work. If F1AP refers IE SRS-PosRRC-Inactive-r17 that would imply less overhead for CU”.

# Draft LS to RAN3

3GPP TSG-RAN WG2 #119e	Tdoc R2-22xxxxx

Electronic meeting, 2022-08-17 - 2022-08-29

# Title:	[draft LS out] on transferring SDT configuration and SRS positioning Inactive configuration from DU to CU

# Response to:	-

# Release:	Release 17

# Work Item:	NR_pos_enh, NR_SmallData_INACTIVE-Core

Source:	RAN2

To:	RAN3

Cc:	-

Contact Person:

Name:	Ritesh Shreevastav

Tel. Number:	-

E-mail Address:	ritesh.shreevastav@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

RAN2 discussed the F1AP IE reference issue with respect to the RRC IE. RAN2 has the understanding as captured in the below observation.

- From RAN2 perspective, both IE can work. If F1AP refers IE on CU side that would imply less overhead for CU.

2. Actions:

To RAN3 group.

ACTION: 	RAN2 respectfully asks RAN3 to take the above RAN2 understanding into account for any RAN3 specification impacts.

3. Date of Next RAN WG2 Meetings:

RAN2 Meeting #119bis-e	     October 10 – 18, 2022		Electronic Meeting

RAN2 Meeting #120               November 14-18, 2022 		Canada

# Conclusion

In the previous sections we made the following observations:

Observation 1	From RAN2 perspective, both IE can work. If F1AP refers IE on CU side that would imply less overhead for CU.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Send LS to RAN3 capturing the Observation that “From RAN2 perspective, both IE can work. If F1AP refers IE on CU side that would imply less overhead for CU”.

# References

R3-223955, “LS on transferring SDT configuration and SRS positioning Inactive configuration from DU to CU”, Google, May 2022

R2-2206384, “[I512] [I010] SRS Positioning configuration provided for SDT”, Intel, Ericsson, May 2022