3GPP TSG-RAN WG2 Meeting #119-e	R2-2208083

# E-meeting, 17th August – 29th August 2022

Agenda item:	8.9.2

Source:	vivo

Title:	Discussion on L2 and L3 U2U relay

Document for:		Discussion & Decision

1. Introduction

According to the WID of NR sidelink relay enhancements [1], objective 1 is shown below:

- Specify mechanisms to support single-hop Layer-2 and Layer-3 UE-to-UE relay (i.e., source UE -> relay UE -> destination UE) for unicast [RAN2, RAN3, RAN4]. - Common part for Layer-2 and Layer-3 relay to be prioritized until RAN#98 - Relay discovery and (re)selection [RAN2, RAN4] - Signalling support for Relay and remote UE authorization if SA2 concludes it is needed [RAN3] - Layer-2 relay specific part - UE-to-UE relay adaptation layer design [RAN2] - Control plane procedures [RAN2] - QoS handling if needed, subject to SA2 progress [RAN2] Note 1A: This work should take into account the forward compatibility for supporting more than one hop in a later release. Note 1B: A remote UE is connected to only a single relay UE at a given time for a given destination UE.

In this contribution, we will discuss the (sub-)bullets in objective 1 which have RAN2 impact and give our suggestions on each.

2. Discussion

2.1 U2U Relay discovery

For Rel-18 U2U relay discovery, there are ongoing discussions for the procedure design in SA2 [2], e.g. Procedure for UE-to-UE Relay discovery with Model A, Procedure for UE-to-UE Relay discovery with Model B, UE-to-UE discovery procedure which is integrated into the unicast link establishment procedure, etc. As the RAN study has dependency on the procedure design in SA2, we should first discuss which aspects could be discussed in RAN2 in parallel with SA2 discussion.

Observation 1: Discovery procedures for U2U relay are being studied by SA2.

In our understanding, the aspects that are more RAN-independent are as follows:

1. Protocol stack.

The protocol stack for discovery message of U2U relay has been studied by RAN2 in Rel-17 SI phase, which is the same as the protocol of U2N relay that is specified in subsequent WI. So it can be discussed whether the protocol for discovery of U2N relay can be reused in U2U relay architecture.

2. Resource configuration.

Considering the similarity of U2U relay discovery and U2N relay discovery, the resources used for U2N relay discovery and U2U relay discovery should be similar or the same, e.g. the same dedicated discovery resource pool. On the other hand, as there are two remote UEs in U2U relay architecture, which node should determine the resource can also be discussed, as well as the resource allocation mode to be supported.

3. Other configuration used for U2U relay discovery control.

It can be discussed about e.g. some AS parameters, such as threshold configuration (used to control U2U relay discovery transmission), gNB capability indication (similar as L2/L3 U2N relay discovery indication in SIB12), SL-SRB configurations, etc. These configurations are supposed to be independent to SA2 discussion.

Therefore, we propose:

Proposal 1: Independently from SA2 discussion about discovery procedure, RAN2 can discuss the following aspects for U2U relay discovery in parallel:

- Discovery protocol stack

- Resource configuration/allocation

  - Resource pool configuration (e.g. dedicated pool vs. shared pool)

  - Mode 1/ Mode 2 for remote UEs and U2U relay UE

- Other configuration

  - Threshold configuration and associated condition to transmit U2U relay discovery message

  - gNB capability indication (e.g. similar as L2/L3 U2N relay discovery indication in SIB12)

  - SL-SRB configuration

For the bullets listed above, we think the basic principle is to take the U2N relay design as the baseline, with considering that gNB does not play a key role in U2U relay architecture.

For discovery protocol stack, the one defined in TS 38.300 for U2N relay can be taken as the baseline for U2U relay in RAN2, as follows:

Figure 16.12.3.1: Protocol Stack of Discovery Message for UE-to-Network Relay

Proposal 2: The Protocol Stack of Discovery Message for UE-to-Network Relay is taken as baseline for UE-to-UE relay discovery message.

For resource configuration, as the U2U relay discovery message is sent on PC5 without gNB involvement, we can simply take it as a normal PC5 message and reuse Rel-16 sidelink mechanism for its transmission/reception, which means the TX UE’s gNB (pre)configure the transmission resource and the UE can work in both mode 1 and mode 2.

Proposal 3: R16 resource allocation mechanism is reused in U2U relay discovery transmission, i.e. for Source remote UE/ U2U relay UE/ target remote UE, both mode 1 and mode 2 can be supported when it serves as TX UE.

As for the dedicated resource pool introduced in Rel-17 for U2N relay discovery, we think the motivation for that dedicated pool remains the same for U2N relay discovery which considers power-efficiency, and thus the dedicated pool can also be used for U2U relay discovery. Meanwhile, we don’t see any need to further divide the dedicated pool into dedicated U2N relay discovery pool and dedicated U2U relay discovery pool, because the resource fragmentation problem should also be taken into account. Therefore,

Proposal 4: The dedicated discovery resource pool introduced in Rel-17 for U2N relay discovery is used for U2U relay discovery as well.

For threshold configuration as mentioned above, it is straightforward that Uu RSRP threshold is not needed anymore to control U2U discovery transmission. However, whether other threshold is needed in U2U relay is FFS.

Proposal 5: Uu RSRP threshold is not needed to control the U2U relay discovery message transmission.

For gNB capability, the key point is whether the U2U relay procedure should be controlled by gNB. After all, the U2U relay transmission is among the source remote UE, U2U relay UE and target remote UE, while the gNB only serve to provide resources for some hops. In our understanding, On one hand, gNB can just take the U2U relay discovery message as other sidelink messages and control it by providing/not providing related resources. On the other hand, we also think it is simple to introduce similar indication in SIB for gNB support of U2U relay. Therefore,

Proposal 6: RAN2 to discuss whether the indication is needed for whether the gNB is capable of U2U relay discovery (similar as L2/L3 U2N relay discovery indication in SIB12).

For SL-SRB, we think the SL-SRB4 could be simply reused and extended to cover the U2U relay discovery message. After that, all discovery messages, including non-relay discovery and U2N/U2U relay discovery, could share a same SL-SRB and the configuration can be least complicated.

Proposal 7: SL-SRB4 is reused for U2U relay discovery message transmission.

2.2 U2U Relay (re)selection

For Relay selection, in U2N relay, the U2N Remote UE performs U2N Relay selection as follows[3]:

The U2N Remote UE performs radio measurements at PC5 interface and uses them for U2N Relay selection and reselection along with higher layer criteria, as specified in TS 23.304 [48]. When there is no unicast PC5 connection between the U2N Relay UE and the U2N Remote UE, the U2N Remote UE uses SD-RSRP measurements to evaluate whether PC5 link quality towards a U2N Relay UE satisfies relay selection criterion.

In U2U relay, we think the same principle can be taken as baseline, i.e. PC5 RSRP (SL-RSRP or SD-RSRP) is used for U2U relay (re)selection. The only difference is that as there are now more than one hops for U2U relay communication, PC5 RSRP on all hops should be considered. How to consider the PC5 RSRP for all hops at the same time could be FFS.

Proposal 8: PC5 signal strength for all hops should be considered to select a U2U relay UE, i.e. only when the PC5 signal strength for all hops is higher than a configured signal strength threshold, a U2U Relay UE can be selected as the candidate U2U relay UE.

After selecting a U2U relay UE, we think the general principle for U2N relay can be reused here in U2U relay that when PC5 signal strength of current Relay UE is below a (pre)configured signal strength threshold, the relay reselection is triggered. Therefore,

Proposal 9: The general principle for U2N relay reselection is taken as the baseline for U2U relay that when PC5 signal strength of the current Relay UE is below a (pre)configured signal strength threshold, the relay reselection is triggered.

There are some other topics on relay (re)selection which needs to be considered, e.g. as now there are two remote UEs, which one should trigger this relay (re)selection, and whether the PC5 signal strength on different hops is considered by one remote UE alone or by different remote UEs respectively, etc. However, these kinds of discussions have more dependency on the relay (re)selection procedure designed in SA2, so we think they can be discussed later in RAN2 once SA2 conclude the general procedures for U2U relay (re)selection.

Proposal 10: Other U2U relay (re)selection aspects (e.g. which UE to trigger, how the PC5 RSRP on all hops being considered) should wait for SA2 progress.

2.3 U2U relay Layer-2 relay specific part

2.3.1 Adaptation layer design

In the running CR [4] for RAN#96e, in case of L2 U2N relay the SRAP header for data PDU has been defined for data relaying between the remote UE and the gNB. In the header, there is Bearer ID and UE ID. For SRAP data PDU relayed in DL, the relay UE relies on the UE ID to determine to which remote UE the SRAP data PDU is to be forwarded. For SRAP data PDU relayed in UL, the gNB relies on the UE ID to determine from which remote UE the SRAP data PDU is received. The Bearer ID in the header is used by the receiver to determine the destination PDCP entity to which the received SRAP SDU is to be delivered.

Figure 6.2.2-1: SRAP Data PDU format with SRAP header

Observation 3: For L2 U2N relay, the SRAP header comprises Bearer ID and the UE ID, wherein

  - The UE ID is used to determine the remote UE by relay UE or gNB,

  - The Bearer ID is used to determine the correspondence with PDCP entity by the receiver.

In order to save space of the SRAP header, the UE ID is defined to 8-bit length. It is allocated by the gNB when the remote UE access the NW via the serving relay UE.

Observation 4: For L2 U2N relay, the UE ID for the remote UE is defined to be 8-bit length and allocated by the serving gNB only.

For L2 U2U relay, it is evident that SRAP layer is still required for data relaying by the relay UE between two remote UEs. There is no doubt that the Bearer ID should be included in the SRAP header so that the receiver remote UE can identify the correct receiver PDCP entity for a received SRAP data SDU.

Proposal 11: Bearer ID is included in the SRAP header for L2 U2U relay.

Regarding whether/how to include the UE ID in the SRAP header, there are various options listed in the following:

Option 1: No UE ID in SRAP header

For this option, it relies on the relay UE to deliver the data to the correct target remote UE based on 1) the correspondence between the source remote UE – relay UE link and the relay UE – target remote UE link, and 2) the ingress PC5 RLC channel to egress PC5 RLC channel mapping. The benefit for this option is that the overhead of UE ID can be saved. This option can work well for single remote UE to single remote UE relaying as illustrated in Figure 1.

Figure 1: One-to-one communication via relay UE

However, for N (remote UE) :1 (remote UE) communications via the same relay UE as illustrated in Figure 2, there are the following restrictions for Option 1:

- Restriction 1: The SRAP PDUs from different source remote UEs to the same target remote UE cannot be multiplexed in the same egress PC5 RLC channel in the egress link, as the target remote UE cannot identify the SRAP PDU is from which source remote UE based on the SRAP header.

- Restriction 2: The source remote UE should not transmit SRAP PDUs to different target remote UEs using the same PC5 RLC channel as the relay UE cannot identify to correct target remote UEs respectively for these SRAP PDUs.

With these restrictions, the N:1 communication via relay UE cannot be achieved.

Figure 2: N-to-1 communication via the same relay UE

Option 2: Only UE ID of source remote UE in SRAP header

For this option, Restriction 1 for N : 1 communication via the same relay UE can be conquered as the target remote UE can identify the SRAP PDUs from different source remote UEs based on the source remote UE ID in the SRAP header. However, Restriction 2 still exists.

Option 3: Only UE ID of target remote UE in SRAP header

For this option, Restriction 2 for N : 1 communication via the same relay UE can be conquered as the relay UE can identify the SRAP PDUs to different target remote UEs based on the target remote UE ID in the SRAP header. However, Restriction 1 still exists.

Option 4: Both UE ID of source remote UE and that of target remote UE in SRAP header

For this option, both Restriction 1 and Restriction 2 have been conquered at the cost of two UE IDs carrying in the SRAP header.

Observation 5: There are 4 options to include remote UE ID in the SRAP header:

- Option 1: no UE ID in SRAP header

- Option 2: only UE ID of source remote UE in SRAP header

- Option 3: only UE ID of target remote UE in SRAP header

- Option 4: both UE IDs of source remote UE and target remote UE in SRAP header

Observation 6: There are PC5 RLC channel multiplexing restrictions for Option 1, 2 and 3 for U2U relay for N (remote UE) : 1 (remote UE) communication via the same relay UE.

Based on the above discussions, we propose:

Proposal 12: RAN2 to study the feasibility to include both UE IDs respectively for the source remote UE and the target remote UE ID in the SRAP header.

Then there is one more issue on what UE ID is to be included in the SRAP header. In R17 U2N relay, a local UE ID of 8 bits, which is allocated by the serving gNB, is used. For L2 U2U relay, other candidate UE IDs such as L2 UE ID could also be considered for sake of a larger UE ID space.

Proposal 13: RAN2 to study whether to include local UE ID or L2 UE ID in the SRAP header.

If local UE ID is reused, it is preferred to consider the option to allow the relay UE to allocate the local UE ID for the remote UE as the remote UE may not always have Uu RRC connections. The relay UE could either allocate the UE ID via PC5 RRC signalling or PC5-S signalling.

Proposal 14: If local UE ID is used in the SRAP header, the relay UE should be responsible to allocate the local UE ID for the remote UE, via either PC5 RRC signalling or PC5-S signalling.

For e2e bearer mapping, we think there is no clear arguments object to reuse PC5 RLC channel defined in R17 to relay the traffic between two remote UEs. The relay UE can determine the ingress PC5 RLC channel to egress PC5 RLC channel mapping based on the QoS requirement of e2e traffic.

Proposal 15: The relay UE should determine the ingress PC5 RLC channel to egress PC5 RLC channel mapping based on the QoS requirement of the relayed traffic.

2.3.2 Control plane procedures

A. SL radio bearer and RLC channel configuration

For radio bearer and RLC channel configuration, in Rel-17 U2N relay, it’s serving gNB to manage end-to-end RB configuration, hop-by-hop RLC bearer configuration and their mapping relationship via Uu RRC since both remote UE and relay UE are in RRC-Connected mode.

However, as the U2U relay communication is among source remote UE, relay UE and target remote UE, it is also more or less similar to the Rel-16 sidelink communication, when source UE’s serving gNB is responsible for SL radio bearer configuration for each direction.

Observation 7: According to Rel-16 NR sidelink, Source UE or Source UE’s serving gNB is responsible for SL radio bearer configuration.

Observation 8: According to Rel-17 U2N relay, Remote UE’s serving gNB is responsible for SL radio bearer and RLC channel configuration.

Therefore, in Rel-18, two options can be considered as follows:

- Option 1: Centralized control

  - Option 1a: Source remote UE (or its serving gNB if RRC CONNECTED) decides E2E configurations (i.e., PC5-SDAP,PC5-PDCP) and HbH configurations (i.e.,PC5-SRAP, PC5-MAC, PC5-PHY) of both hops

  - Option 1b: U2U Relay UE (or its serving gNB if RRC CONNECTED) decides E2E configurations (ie.PC5-SDAP,PC5-PDCP) and HbH configurations (i.e.,PC5-SRAP, PC5-MAC, PC5-PHY) of both hops

- Option 2: Distributed control

  - Source remote UE (or its serving gNB if RRC CONNECTED) decides E2E configurations and HbH configurations with U2U Relay UE

  - U2U Relay UE (or its serving gNB if RRC CONNECTED) decides HbH configurations with Target remote UE

The sketch figure for option-1 and option-2 is shown below:

Figure 3. Option 1-Centralized control signalling procedure (e.g., All by Source remote UE decision)

Figure 4. Option 2-Distributed control signalling procedure (e.g., TX UE decision on each hop)

Therefore, we think RAN2 can discuss which option should be taken as the baseline for U2U relay configuration procedure.

Proposal 16: RAN2 to discuss the following options for configuring SL radio bearer and RLC channel for L2 U2U relay:

- Option 1: Centralized control

  - Option 1a: Source UE (or its serving gNB if RRC CONNECTED) decides E2E configurations (ie.PC5-SDAP,PC5-PDCP) and HbH configurations (i.e.,PC5-SRAP, PC5-MAC, PC5-PHY)

  - Option 1b: Relay UE (or its serving gNB if RRC CONNECTED) decides E2E configurations (ie.PC5-SDAP,PC5-PDCP) and HbH configurations (i.e.,PC5-SRAP, PC5-MAC, PC5-PHY)

- Option 2: Distributed control

  - i.e., Source UE (or its serving gNB if RRC CONNECTED) decides E2E configurations and HbH configurations for hop-0, and Relay UE (or its serving gNB if RRC CONNECTED) decides HbH configurations for hop-1

B. How to detect E2E PC5 link failure

In Rel-17 U2N relay, the E2E link for the U2N Remote UE is consisted of PC5 hop (between the U2N Remote UE and the U2N Relay UE) and Uu hop (between the U2N Relay UE and serving gNB). Moreover, the U2N Remote UE declares RLF with following new conditions, which may trigger RRC connection re-establishment:

- Upon detecting PC5 RLF by itself (i.e., due to failure at PC5 hop);

- Upon receiving indication from its serving U2N Relay UE after the U2N Relay UE declares RLF (i.e., due to failure at Uu hop).

In general, failure at either hop leads to E2E link failure. When it comes to Rel-18 U2U relay, we think the similar mechanism for E2E link failure detection can be reused. In other words, the Source Remote UE declares E2E PC5 link failure with following new conditions, see below Figure 5:

- Upon detecting PC5 RLF by itself (i.e., between the Source Remote UE and the U2U Relay UE);

- Upon receiving indication from its serving U2U Relay UE after the U2U Relay UE declares PC5 RLF (i.e., between the U2U Relay UE and the Target Remote UE).

After the Source Remote UE declares E2E PC5 link failure, the U2U Relay UE becomes no longer suitable. Therefore, it may be better for the Source Remote UE to perform relay reselection and find another U2U Relay UE to continue the U2U relay service transmission.

Figure 5. E2E radio link failure detection in Rel-18 U2U Relay

Therefore, we propose:

Proposal 17: Define E2E link failure conditions due to failure at either PC5 hop, i.e., PC5 RLF between Source Remote UE and U2U Relay UE, or PC5 RLF between U2U Relay UE and Target Remote UE.

Proposal 18: The Source Remote UE may perform relay reselection after receiving the PC5 notification by U2U Relay UE of PC5 RLF between U2U Relay UE and Target Remote UE.

2.3.2 QoS handling

In R17 U2N relay user data transmission scenario, both relay UE and remote UE are in RRC-Connected mode and controlled by the same serving gNB. Hence, parameters splitting/configuration and E2E QoS guarantee are totally left to gNB implementation. And a new configured field had been introduced for PC5 PDB.

Observation 9: In U2N, QoS split and handling of two hops are left to the serving gNB, e.g. parameters decision and new PDB configuration in PC5.

However, in single-hop U2U relay scenario, three UEs (source and target remote UEs and one U2U relay UE) may communicate with each other without involving any gNB. Furthermore, these three UEs may locate in different cells and get configuration from different cells/ methods (e.g. dedicated/SIB/pre-configuration signaling). In a word, it is difficult to find a centralized NW node to control QoS parameter splitting and configuration.

Observation 10: In U2U, it is not always applicable to find a centralized NW node to control QoS parameter splitting and configuration.

In SA2 R18 CR [2], potential QoS handling solutions for L2 U2U relay and L3 U2U relay had been proposed and analyzed, e.g. solution 4 and so on. Based on these solutions, for L3 U2U relay, no extra RAN2 impact are foreseen since each hop is a complete legacy PC5 link and PC5 QoS parameters splitting and distribution is up to higher layer(s), i.e. in the scope of SA2. While for L2 U2U relay, RAN2 may be involved. E2E PC5 QoS parameters are delivered by higher layer(s) to AS layer and it is up to AS layer to ensure the PC5 QoS over the two PC5 links.

Observation 11: For L3 U2U relay, no extra RAN2 impact are foreseen. While for L2 U2U relay, RAN2 may be involved.

From the perspective of AS layers, parameters control and configuration is the main functionality of PC5 RRC layer. E2E PC5 RRC procedure can be used to control the E2E PC5 QoS parameters (e.g. legacy PC5 RRC procedure can be baseline) and Hop-by-Hop PC5 RRC procedure can be used for PC5 QoS guarantee in each link.

According to legacy PC5 configuration rule, it is up to TX side to decide parameters and deliver configurations. Hence, in each hop, TX side can decide QoS parameters splitting based on the initial E2E QoS info, the rest of QoS budgets, radio link quality, the radio resource congestion of this hop and so on. For a intermediate UE, the QoS info (e.g. E2E QoS and/or the rest of QoS budgets) and hop info (may be omitted in a single-hop scenario) can be delivered from its preceding UE.

In a single-hop scenario of L2 U2U relay, QoS splitting and handling may be performed in a centralized way, e.g. by the relay UE, since the relay UE can know the status of two links at the same time. However, this centralized way may not be forward compatible for a multi-hop scenario. On the other hand, a simpler splitting algorithm can be considered, e.g. the average distribution, to reduce the complexity and signaling overhead. Details can be FFS now. We can conclude some basic rules for U2U relay QoS splitting and handling as followings:

Proposal 19: Using Hop-by-Hop PC5 RRC procedure in L2 U2U relay scenario to perform the E2E QoS splitting over the two hops.

Proposal 20: The QoS info and/or hop info for U2U relay QoS handling may be delivered from the preceding UE to the next one.

3. Conclusion

In this contribution, we discussed objective-1 according to the WID of NR sidelink relay enhancements, including common part for L2/L3 (discovery and relay (re-)selection), as well as L2 specific topics such as Adaptation layer design, CP procedures and QoS handling.

We have the following observations and proposals:

For discovery

Observation 1: Discovery procedures for U2U relay are being studied by SA2.

Proposal 1: Independently from SA2 discussion about discovery procedure, RAN2 can discuss the following aspects for U2U relay discovery in parallel:

- Discovery protocol stack

- Resource configuration/allocation

  - Resource pool configuration (e.g. dedicated pool vs. shared pool)

  - Mode 1/ Mode 2 for remote UEs and U2U relay UE

- Other configuration

  - Threshold configuration and associated condition to transmit U2U relay discovery message

  - gNB capability indication (e.g. similar as L2/L3 U2N relay discovery indication in SIB12)

  - SL-SRB configuration

Proposal 2: The Protocol Stack of Discovery Message for UE-to-Network Relay is taken as baseline for UE-to-UE relay discovery message.

Proposal 3: R16 resource allocation mechanism is reused in U2U relay discovery transmission, i.e. for Source remote UE/ U2U relay UE/ target remote UE, both mode 1 and mode 2 can be supported when it serves as TX UE.

Proposal 4: The dedicated discovery resource pool introduced in Rel-17 for U2N relay discovery is used for U2U relay discovery as well.

Proposal 5: Uu RSRP threshold is not needed to control the U2U relay discovery message transmission.

Proposal 6: RAN2 to discuss whether the indication is needed for whether the gNB is capable of U2U relay discovery (similar as L2/L3 U2N relay discovery indication in SIB12).

Proposal 7: SL-SRB4 is reused for U2U relay discovery message transmission.

For relay (re)selection

Proposal 8: PC5 signal strength for all hops should be considered to select a U2U relay UE, i.e. only when the PC5 signal strength for all hops is higher than a configured signal strength threshold, a U2U Relay UE can be selected as the candidate U2U relay UE.

Proposal 9: The general principle for U2N relay reselection is taken as the baseline for U2U relay that when PC5 signal strength of the current Relay UE is below a (pre)configured signal strength threshold, the relay reselection is triggered.

Proposal 10: Other U2U relay (re)selection aspects (e.g. which UE to trigger, how the PC5 RSRP on all hops being considered) should wait for SA2 progress.

For Layer-2 relay specific part

Observation 3: For L2 U2N relay, the SRAP header comprises Bearer ID and the UE ID, wherein

- The UE ID is used to determine the remote UE by relay UE or gNB,

- The Bearer ID is used to determine the correspondence with PDCP entity by the receiver.

Observation 4: For L2 U2N relay, the UE ID for the remote UE is defined to be 8-bit length and allocated by the serving gNB only.

Observation 5: There are 4 options to include remote UE ID in the SRAP header:

- Option 1: no UE ID in SRAP header

- Option 2: only UE ID of source remote UE in SRAP header

- Option 3: only UE ID of target remote UE in SRAP header

- Option 4: both UE IDs of source remote UE and target remote UE in SRAP header

Observation 6: There are PC5 RLC channel multiplexing restrictions for Option 1, 2 and 3 for U2U relay for N (remote UE) : 1 (remote UE) communication via the same relay UE.

Observation 7: According to Rel-16 NR sidelink, Source UE or Source UE’s serving gNB is responsible for SL radio bearer configuration.

Observation 8: According to Rel-17 U2N relay, Remote UE’s serving gNB is responsible for SL radio bearer and RLC channel configuration.

Observation 9: In U2N, QoS split and handling of two hops are left to the serving gNB, e.g. parameters decision and new PDB configuration in PC5.

Observation 10: In U2U, it is not always applicable to find a centralized NW node to control QoS parameter splitting and configuration.

Observation 11: For L3 U2U relay, no extra RAN2 impact are foreseen. While for L2 U2U relay, RAN2 may be involved.

Proposal 11: Bearer ID is included in the SRAP header for L2 U2U relay.

Proposal 12: RAN2 to study the feasibility to include both UE IDs respectively for the source remote UE and the target remote UE ID in the SRAP header.

Proposal 13: RAN2 to study whether to include local UE ID or L2 UE ID in the SRAP header.

Proposal 14: If local UE ID is used in the SRAP header, the relay UE should be responsible to allocate the local UE ID for the remote UE, via either PC5 RRC signalling or PC5-S signalling.

Proposal 15: The relay UE should determine the ingress PC5 RLC channel to egress PC5 RLC channel mapping based on the QoS requirement of the relayed traffic.

Proposal 16: RAN2 to discuss the following options for configuring SL radio bearer and RLC channel for L2 U2U relay:

- Option 1: Centrilized control

  - Option 1a: Source UE (or its serving gNB if RRC CONNECTED) decides E2E configuraitons (ie.PC5-SDAP,PC5-PDCP) and HbH configurations (i.e.,PC5-SRAP, PC5-MAC, PC5-PHY)

  - Option 1b: Relay UE (or its serving gNB if RRC CONNECTED) decides E2E configuraitons (ie.PC5-SDAP,PC5-PDCP) and HbH configurations (i.e.,PC5-SRAP, PC5-MAC, PC5-PHY)

- Option 2: Distributed control

  - i.e., Source UE (or its serving gNB if RRC CONNECTED) decides E2E configuraitons and HbH configuraions for hop-0, and Relay UE (or its serving gNB if RRC CONNECTED) decides HbH configuraions for hop-1

Proposal 17: Define E2E link failure conditions due to failure at either PC5 hop, i.e., PC5 RLF between Source Remote UE and U2U Relay UE, or PC5 RLF between U2U Relay UE and Target Remote UE.

Proposal 18: The Source Remote UE may perform relay reselection after receiving the PC5 notification by U2U Relay UE of PC5 RLF between U2U Relay UE and Target Remote UE.

Proposal 19: Using Hop-by-Hop PC5 RRC procedure in L2 U2U relay scenario to perform the E2E QoS splitting over the two hops.

Proposal 20: The QoS info and/or hop info for U2U relay QoS handling may be delivered from the preceding UE to the next one.

4. References

RP-221262, New WID on NR sidelink relay enhancements, LG Electronics;

3GPP TR 23.700-33 V0.3.0 (2022-05)

3GPP TS 38.300 V17.1.0 (2022-06)

R2-2206238, Correction on SRAP for L2 UE-to-Network Relay, OPPO