3GPP TSG-RAN2 Meeting #119-e	R2-2208084

eMeeting, 15-26 August 2022

Agenda Item:	6.1.3 Other CP corrections

Source: 	Ericsson

Title:  	Broadcast sessions with the same MRB configuration

Document for:	Discussion and Decision

# Introduction

There are two use cases where broadcast sessions have the same MRB configuration:

- Broadcast sessions are mapped onto the same g-RNTI (Rel-17)

- RAN sharing with broadcast sessions with the same content from different operators (Rel-18)

The PTM configuration in MCCH for these use cases leads to duplication and increased overhead because the same information is repeated. This has a negative impact on UE power consumption and NW resource usage.

# Background

## Use case 1: TMGI to g-RNTI mapping

In RAN2#114-e/RAN2#116-e the following agreements were reached for TMGI to g-RNTI mapping:

- One-to-one mapping between G-RNTI and MBS session is supported in NR MBS. Other mappings FFS

- one-to-many mapping between G-RNTI and MBS sessions is supported and it is assumed that this does not introduce additional specification work.

Multiple sessions can be mapped onto the same g-RNTI. The DRX is configured per g-RNTI, and the UE discards the multicast data related to the sessions/TMGIs it is not interested in based on LCID.

## Use case 2: RAN sharing

RAN sharing, i.e. 5G Multi-Operator Core Network (5G MOCN), is already supported in Rel-17. The PLMN info of different operators supporting MBS can be configured in the PLMN-IdentityInfoList in SIB1, enabling UEs supporting MBS to camp and access the cell. When the gNB supports MBS and RAN sharing is configured, then UEs from different operators can receive the MBS content from their operators on that cell:

The session from each operator has allocated a different TMGI, irrespective whether the content of the session is the same or different. This implies that each session occupies separate resources over the air-interface. So RAN sharing, without resource optimization, is supported in Rel-17 for both MBS broadcast and multicast services.

The Rel-18 RAN WID objective specifies RAN3 to study resource efficiency in RAN sharing scenarios (when the same content is transmitted by different operators) [1]:

- Study and if necessary, specify enhancements to improve the resource efficiency for MBS reception in RAN sharing scenarios [RAN3]

Note: collaboration with SA2 is expected in due course for the above objectives.

The aim is to avoid the use of duplicate PTM resources for the same content in the cell.

SA2 has started to study several MBS enhancements in Rel-18 [2]. One objective is to study efficient resource utilization when the same MBS broadcast content is transmitted in 5G MOCN network sharing scenario:

WT#1.2	Study feasible and efficient resource utilization for the same broadcast content to be provided to 5G MOCN network sharing scenarios (i.e., multiple CNs are connected to the same NG-RAN);

NOTE 2: The NG-RAN is assumed not to be aware of the same content via the application layer information detection.

NOTE 3: Collaboration with RAN WGs is needed.

SA2 has documented different solutions in their technical report [3] and SA2 will evaluate these solutions in coming meetings. In one solution (solution #7) an associated session ID is introduced to indicate when the content of the session is the same from different operators:

When another session is activated with the same associated session ID, the RAN does not need to configure new MRB(s) to convey the content. But the RAN only needs to associate the new TMGI with the configured MRB(s).

With this approach the same content from different operators is only transmitted once, and resource utilization over the air-interface is optimized.

# Discussion

MBS broadcast configuration

An MBS session is identified by a TMGI and for an MBS session one or more MRBs can be configured. For each session it is indicated which g-RNTI the UE should monitor to receive it. For MBS broadcast a TMGI is configured per MBS-SessionInfo on MCCH:

The MRB configuration is duplicated for the sessions that have the same TMGI, e.g. for use case 1 and 2 above. This increases the MBSBroadcastConfiguration message on MCCH.

To configure multiple broadcast sessions with the same MRB configuration a separate MBS-SessionInfo needs to be configured for each session:

MBSBroadcastConfiguration-r17-IEs ::= SEQUENCE {

mbs-SessionInfoList-r17               MBS-SessionInfoList-r17 OPTIONAL,   -- Need R

…

MBS-SessionInfoList-r17 ::=      SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfo-r17

MBS-SessionInfo-r17 ::=          SEQUENCE {

mbs-SessionId-r17                TMGI-r17,                                                          28 bits

g-RNTI-r17                       RNTI-Value,                                                        16 bits

mrb-ListBroadcast-r17            MRB-ListBroadcast-r17,                                    n x (10-24) bits

mtch-SchedulingInfo-r17          DRX-ConfigPTM-Index-r17                      OPTIONAL, -- Need S  1-6 bits

mtch-NeighbourCell-r17           BIT STRING (SIZE(maxNeighCellMBS-r17))       OPTIONAL, -- Need S  1-3 bits

pdsch-ConfigIndex-r17            PDSCH-ConfigIndex-r17                        OPTIONAL, -- Need S  1-4 bits

mtch-SSB-MappingWindowIndex-r17  MTCH-SSB-MappingWindowIndex-r17              OPTIONAL  -- Need R  1-4 bits

}

DRX-ConfigPTM-Index-r17 ::=          INTEGER (0..maxNrofDRX-ConfigPTM-1-r17)

PDSCH-ConfigIndex-r17  ::=           INTEGER (0..maxNrofPDSCH-ConfigPTM-1-r17)

MTCH-SSB-MappingWindowIndex-r17  ::= INTEGER (0..maxNrofMTCH-SSB-MappingWindow-1-r17)

MRB-ListBroadcast-r17 ::=            SEQUENCE (SIZE (1..maxNrofMRB-Broadcast-r17)) OF MRB-InfoBroadcast-r17

MRB-InfoBroadcast-r17 ::=            SEQUENCE {

pdcp-Config-r17                      MRB-PDCP-ConfigBroadcast-r17,                                3-12 bits

rlc-Config-r17                       MRB-RLC-ConfigBroadcast-r17,                                 7-12 bits

...

}

MRB-PDCP-ConfigBroadcast-r17 ::=     SEQUENCE {

pdcp-SN-SizeDL-r17                   ENUMERATED {len12bits}                   OPTIONAL, -- Need S  1 bit

headerCompression-r17                CHOICE {                                                      1/7 bits

notUsed                              NULL,

rohc                                 SEQUENCE {

maxCID-r17                           INTEGER (1..16)               DEFAULT 15,

profiles-r17                         SEQUENCE {

profile0x0000-r17                    BOOLEAN,

profile0x0001-r17                    BOOLEAN,

profile0x0002-r17                    BOOLEAN

}

}

},

t-Reordering-r17                     ENUMERATED {ms1, ms10, ms40, ms160, ms500, ms1000, ms1250, ms2750}  OPTIONAL -- Need S                                                                                     1/4 bits

}

MRB-RLC-ConfigBroadcast-r17 ::=      SEQUENCE {

logicalChannelIdentity-r17           LogicalChannelIdentity,                                       5 bits

sn-FieldLength-r17                   ENUMERATED {size6}                       OPTIONAL, -- Need S  1/2 bits

t-Reassembly-r17                     T-Reassembly                             OPTIONAL  -- Need S  1/5 bits

}

Duplication

This means that the following information is duplicated:

- TMGI 				28 bits

- g-RNTI 				16 bits

- MRB list 				n x (10-24) bits

- DRX config index

- PDSCH config index

- MTCH-SSB mapping index

- MTCH neighbour cell info (if configured)

The increased MCCH size due to duplication of the same information is mainly caused by repeating the same TMGI, g-RNTI and MRB configurations for each session.

By using an index the same DRX config, PDSCH config and MTCH-SSB mapping can be shared between sessions. Furthermore when default radio configurations are used (section 9.1.1.7) then the MRB configuration size is further reduced (but duplication is not avoided).

Example

In case different TV/video sessions are mapped onto to the same g-RNTI then it is likely that multiple sessions are broadcasted at the same time. In case there is RAN sharing there could be a couple of operators with the same sessions.

If we take 3 sessions with 2 MRBs as an example then the increased size MCCH size due to duplication is in the order of 2x28+2x16+2x(10-24)=102-136 bits.

For these use cases, the MCCH size due to duplication can be reduced significantly by using an index for the TMGI, g-RNTI and MRB list as well:

Observation 1: The ASN.1 structure is not optimized for multiple MBS broadcast sessions with the same MRB configuration.

Proposal 1: RAN2 to discuss if the MBS broadcast configuration should be optimized for multiple sessions with the same MRB configuration.

Proposal 2: In case an optimization is agreed, RAN2 to discuss for which release the ASN.1 structure should be changed.

A possible ASN.1 structure is provided in the TP provided in the Annex.

# Summary

RAN2 is kindly asked to discuss if the ASN.1 structure should be optimized for broadcast sessions with the same MRB configuration:

Observation 1: The ASN.1 structure is not optimized for multiple MBS broadcast sessions with the same MRB configuration.

Proposal 1: RAN2 to discuss if the MBS broadcast configuration should be optimized for multiple sessions with the same MRB configuration.

Proposal 2: In case an optimization is agreed, RAN2 to discuss for which release the ASN.1 structure should be changed.

# References

- RP-221458, Revised WID: Enhancements of NR Multicast and Broadcast Services, CATT, WID, RAN#94e

- SP-211645, New SID: Architectural enhancements for 5G multicast-broadcast services Phase 2, Huawei, BCN, SA#94-e

- TR 23.700-47, Study on architectural enhancements for 5G multicast-broadcast services; Phase 2, TR, SA2, v0.3.0

# Annex: TP 38.331

### 6.3.6	MBS information elements

<TEXT OMITTED>

MBSBroadcastConfiguration message

-- ASN1START

-- TAG-MBSBROADCASTCONFIGURATION-START

MBSBroadcastConfiguration-r17 ::= SEQUENCE {

criticalExtensions                CHOICE {

mbsBroadcastConfiguration-r17     MBSBroadcastConfiguration-r17-IEs,

criticalExtensionsFuture          SEQUENCE {}

}

}

MBSBroadcastConfiguration-r17-IEs ::= SEQUENCE {

mbs-SessionInfoList-r17               MBS-SessionInfoList-r17                                              OPTIONAL,   -- Need R

mbs-NeighbourCellList-r17             MBS-NeighbourCellList-r17                                            OPTIONAL,   -- Need S

tmgi-List-r17                         SEQUENCE (SIZE (1..maxNrofDuplicateMBS-Session-r17)) OF TMGI-r17,

rnti-ValueList-r17                    SEQUENCE (SIZE (1..maxNrofDuplicateMBS-Session-r17)) OF RNTI-Value,

mrb-ListBroadcastList-r17             SEQUENCE (SIZE (1..maxNrofDuplicateMBS-Session-r17)) OF MRB-ListBroadcast-r17,

drx-ConfigPTM-List-r17                SEQUENCE (SIZE (1..maxNrofDRX-ConfigPTM-r17)) OF DRX-ConfigPTM-r17   OPTIONAL,   -- Need R

pdsch-ConfigMTCH-r17                  PDSCH-ConfigBroadcast-r17                                            OPTIONAL,   -- Need S

mtch-SSB-MappingWindowList-r17        MTCH-SSB-MappingWindowList-r17                                       OPTIONAL,   -- Need R

lateNonCriticalExtension              OCTET STRING                                                         OPTIONAL,

nonCriticalExtension                  SEQUENCE {}                                                          OPTIONAL

}

-- TAG-MBSBROADCASTCONFIGURATION-STOP

-- ASN1STOP

MBSBroadcastConfiguration field descriptions

pdsch-ConfigMTCH Provides parameters for acquiring the PDSCH for MTCH. When this field is absent, the UE shall use parameters in pdsch-ConfigMCCH to acquire the PDSCH for MTCH.

mbs-SessionInfoList Provides the configuration of each MBS session provided by MBS broadcast in the current cell.

mbs-NeighbourCellList List of neighbour cells providing MBS broadcast services via broadcast MRB. This field is used by the UE together with mtch-NeighbourCell field signalled for each MBS session in the corresponding MBS-SessionInfo. When an empty mbs-NeighbourCellList list is signalled, the UE shall assume that MBS broadcast services signalled in mbs-SessionInfoList in the MBSBroadcastConfiguration message are not provided in any neighbour cell. When the field mbs-NeighbourCellList is absent, the current serving cell does not provide information about MBS broadcast services in the neighbouring cells, i.e. the UE cannot determine the presence or absence of an MBS service in neighbouring cells based on the absence of this field.

<TEXT OMITTED>

#### –	MBS-SessionInfoList

The IE MBS-SessionInfoList provides the list of ongoing MBS broadcast sessions transmitted via broadcast MRB and, for each MBS broadcast session, the associated G-RNTI and scheduling information.

MBS-SessionInfoList information element

-- ASN1START

-- TAG-MBS-SESSIONINFOLIST-START

MBS-SessionInfoList-r17 ::=      SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfo-r17

MBS-SessionInfo-r17 ::=          SEQUENCE {

mbs-SessionId-r17                TMGI-Index-r17,

g-RNTI-r17                       RNTI-ValueIndex-r17,

mrb-ListBroadcast-r17            MRB-ListBroadcast-Index-r17,

mtch-SchedulingInfo-r17          DRX-ConfigPTM-Index-r17                      OPTIONAL, -- Need S

mtch-NeighbourCell-r17           BIT STRING (SIZE(maxNeighCellMBS-r17))       OPTIONAL, -- Need S

pdsch-ConfigIndex-r17            PDSCH-ConfigIndex -r17                        OPTIONAL, -- Need S

mtch-SSB-MappingWindowIndex-r17  MTCH-SSB-MappingWindowIndex-r17              OPTIONAL  -- Need R

}

TMGI-Index-r17 ::=                  INTEGER (0..maxNrofDuplicateMBS-Session-1-r17)

RNTI-Index-Value-r17::=             INTEGER (0..maxNrofDuplicateMBS-Session-1-r17)

MRB-ListBroadcast-Index-r17::=      INTEGER (0..maxNrofDuplicateMBS-Session-1-r17)

DRX-ConfigPTM-Index-r17 ::=          INTEGER (0..maxNrofDRX-ConfigPTM-1-r17)

PDSCH-ConfigIndex-r17  ::=           INTEGER (0..maxNrofPDSCH-ConfigPTM-1-r17)

MTCH-SSB-MappingWindowIndex-r17  ::= INTEGER (0..maxNrofMTCH-SSB-MappingWindow-1-r17)

-- TAG-MBS-SESSIONINFOLIST-STOP

-- ASN1STOP

MBS-SessionInfoList field descriptions

g-RNTI G-RNTI used to scramble the scheduling and transmission of MTCH.

mbsSessionId Indicates an identifier of the MBS session provided by the MTCH.

mtch-neighbourCell Indicates neighbour cells which provide this service on MTCH. The first bit is set to 1 if the service is provided on MTCH in the first cell in mbs-NeighbourCellList, otherwise it is set to 0. The second bit is set to 1 if the service is provided on MTCH in the second cell in mbs-NeighbourCellList, and so on. If the service is not available in any neighbouring cell and mbs-NeighbourCellList is signalled, the network sets all bits in this field to 0. If this field is absent, the related service may or may not be available in any neighbouring cell, i.e. the UE cannot determine the presence or absence of an MBS service in neighbouring cells based on the absence of this field.

mtch-schedulingInfo Indicates the index of DRX configuration entry in drx-ConfigPTM-List that is used for scheduling the MTCH. The value 0 corresponds to the first entry in drx-ConfigPTM-List, the value 1 corresponds to the second entry in drx-ConfigPTM-List and so on. In case mtch-schedulingInfo is absent for a G-RNTI (i.e. no PTM DRX), the UE shall monitor for PDCCH scrambled with G-RNTI in any slot according to the search space configured for MTCH [see TS 38.213 [13], clause 10.1].

mtch-SSB-MappingWindowIndex Indicates the index of MTCH-SSB-MappingWindowCycleOffset configuration entry in MTCH-SSB-MappingWindowList. The value 0 corresponds to the first entry in MTCH-SSB-MappingWindowList, the value 1 corresponds to the second entry in MTCH-SSB-MappingWindowList and so on. This field is set to the same value for all MBS sessions mapped to the same G-RNTI.

pdschConfigIndex Indicates the index of PDSCH configuration entry in pdschConfigList for MTCH. Value 0 corresponds to the first entry in pdschConfigList, the value 1 corresponds to the second entry in pdschConfigList and so on. When the field is absent the UE applies the first entry in pdschConfigList for MTCH.

<TEXT OMITTED>

#### –	MRB-ListBroadcast

The IE MRB-ListBroadcast provides a list of broadcast MRBs for a number of sessions

MRB-ListBroadcast information element

-- ASN1START

-- TAG-MRB-ListBroadcast-START

MRB-ListBroadcast-r17 ::=            SEQUENCE (SIZE (1..maxNrofMRB-Broadcast-r17)) OF MRB-InfoBroadcast-r17

MRB-InfoBroadcast-r17 ::=            SEQUENCE {

pdcp-Config-r17                      MRB-PDCP-ConfigBroadcast-r17,

rlc-Config-r17                       MRB-RLC-ConfigBroadcast-r17,

...

}

MRB-PDCP-ConfigBroadcast-r17 ::=     SEQUENCE {

pdcp-SN-SizeDL-r17                   ENUMERATED {len12bits}                   OPTIONAL, -- Need S

headerCompression-r17                CHOICE {

notUsed                              NULL,

rohc                                 SEQUENCE {

maxCID-r17                           INTEGER (1..16)               DEFAULT 15,

profiles-r17                         SEQUENCE {

profile0x0000-r17                    BOOLEAN,

profile0x0001-r17                    BOOLEAN,

profile0x0002-r17                    BOOLEAN

}

}

},

t-Reordering-r17                     ENUMERATED {ms1, ms10, ms40, ms160, ms500, ms1000, ms1250, ms2750}    OPTIONAL -- Need S

}

MRB-RLC-ConfigBroadcast-r17 ::=      SEQUENCE {

logicalChannelIdentity-r17           LogicalChannelIdentity,

sn-FieldLength-r17                   ENUMERATED {size6}                       OPTIONAL, -- Need S

t-Reassembly-r17                     T-Reassembly                             OPTIONAL  -- Need S

}

-- TAG-MRB-ListBroadcast-STOP

-- ASN1STOP

MRB-ListBroadcast field descriptions

headerCompression If rohc is configured, the UE shall apply the configured ROHC profile(s) in downlink. When the field is absent the UE applies the value as specified in 9.1.1.7.

mrb-listBroadcast A list of broadcast MRBs to which the associated broadcast MBS session is mapped to.

pdcp-SN-SizeDL Indicates that PDCP sequence number size of 12 bits is used, as specified in TS 38.323 [5]. When the field is absent the UE applies the value as specified in 9.1.1.7.

sn-FieldLength Indicates that the RLC SN field size of 6 bits is used, see TS 38.322 [4]. When the field is absent the UE applies the value as specified in 9.1.1.7.

t-Reassembly Timer for reassembly in TS 38.322 [4], in milliseconds. Value ms0 means 0 ms, value ms5 means 5 ms and so on. When the field is absent the UE applies the value in specified in 9.1.1.7.

t-Reordering Value in ms of t-Reordering specified in TS 38.323 [5]. Value ms1 corresponds to 1 ms, value ms10 corresponds to 10 ms, and so on. When the field is absent the UE applies the value as specified in 9.1.1.7.

<TEXT OMITTED>

### –	Multiplicity and type constraint definitions

<TEXT OMITTED>

maxFreqMBS-r17                          INTEGER ::= 16      -- Maximum number of MBS frequencies reported in MBSInterestIndication

maxNrofDRX-ConfigPTM-r17                INTEGER ::= 64      -- Max number of DRX configuration for PTM provided in MBS broadcast in a

-- cell

maxNrofDRX-ConfigPTM-1-r17              INTEGER ::= 63      -- Max number of DRX configuration for PTM provided in MBS broadcast in a

-- cell minus 1

maxNrofMBS-ServiceListPerUE-r17         INTEGER ::= 16      -- Maximum number of services which the UE can include in the  MBS interest

-- indication

maxNrofMBS-Session-r17                  INTEGER ::= 1024    -- Maximum number of MBS sessions provided in MBS broadcast in a cell

maxNrofDuplicateMBS-Session-r17         INTEGER ::= 16      -- Maximum number of MBS sessions provided in MBS broadcast in a cell that

-- have the same MRB configuration

maxNrofDuplicateMBS-Session-1-r17       INTEGER ::= 15      -- Maximum number of MBS sessions provided in MBS broadcast in a cell that

-- have the same MRB configuration minus 1

maxNrofMTCH-SSB-MappingWindow-r17       INTEGER ::= 16      -- Maximum number of MTCH to SSB beam mapping pattern

maxNrofMTCH-SSB-MappingWindow-1-r17     INTEGER ::= 15      -- Maximum number of MTCH to SSB beam mapping pattern minus 1

maxNrofMRB-Broadcast-r17                INTEGER ::= 4       -- Maximum number of broadcast MRBs configured for one MBS broadcast service

<TEXT OMITTED>