3GPP TSG-RAN2 Meeting #119-e	R2-2208086

eMeeting, 15-26 August 2022

Agenda Item:	6.1.1 Organizational and Stage-2

Source: 	Ericsson

Title:  	Clarification of group paging

Document for:	Discussion and Decision

# Introduction

The following topics, described in 38.300, need further clarifications:

- (Do not) Release to RRC_IDLE when the multicast session is active

- Group paging during session activation and new multicast data when the session is active

- Monitoring for group paging

# Discussion

## (Do not) Release to RRC_IDLE when the multicast session is active

For data inactivity during a MBS multicast session it says in section 16.10.5.2 in 38.300:

When there is (temporarily) no data to be sent to the UEs for a multicast session, the gNB may move the UE to RRC IDLE/INACTIVE state.

The following observations and concerns are raised:

- It is possible that “temporarily no data” refers to the case when the multicast session is in Active state (TS 23.247 [1]) and there is no data for some time, while “no data” refers to the case when the multicast session is in Inactive state and hence no multicast data is transmitted. However this is not clear from the description.

- In case the multicast UE is released to RRC_IDLE when the multicast session is in Active state, then the UE will not receive multicast data when the multicast data transmissions start again, because the CN only triggers CM_IDLE group Paging when the multicast session is activated (see clause 7.2.5.2 in TS 23.247 [1]). For an active MBS session the CN does not trigger group paging to bring the UE from CM_IDLE to CM_CONNECTED:

- Support group paging at multicast session activation over radio toward UEs in CM-IDLE state and CM-CONNECTED with RRC INACTIVE state.

- Active state: Multicast MBS session is established and MBS data can be transmitted to the UEs that have joined th Multicast MBS session. Radio resources for the Multicast MBS session are established. To receive multicast MBS session data, UEs that joined the Multicast MBS session shall be in CM-CONNECTED state for receiving data of the Multicast MBS session. UEs are allowed to join the Multicast MBS session (subject to authorization check). 5GC resources and radio resources for the Multicast MBS session are reserved for UEs that joined the Multicast MBS session.

- Inactive state: Multicast MBS session is established but no MBS data is transmitted to the UEs that have joined the Multicast MBS session. Radio resources for the Multicast MBS session are released, and the UEs that joined the Multicast MBS session may be in CM-CONNECTED or CM-IDLE state. UEs are allowed to join the Multicast MBS session (subject to authorization check).

- The gNB should not release the UE when there is (temporarily) no multicast data, while a DRB is configured and there are still unicast data. This condition is also not explicitly described.

When the last UE has left the multicast session and the session is released in RAN, then the UE can be released to either RRC_INACTIVE or RRC_IDLE.

Observation 1: gNB should not release a multicast UE to RRC_IDLE while the session is Active.

Proposal 1: Clarify that the gNB may release a multicast UE to RRC_IDLE or RRC_INACTIVE when the CN deactivates the session and there is no unicast data for some time.

Proposal 2: Clarify that the gNB may release a multicast UE to RRC_INACTIVE when the session is activated and there is no multicast nor unicast data for some time.

A TP is provided in the Annex.

## Group paging during session activation and new multicast data when session is active

For the group notification mechanism it says in section 16.10.5.2 in 38.300:

gNBs supporting MBS use a group notification mechanism to notify the UEs in RRC IDLE/INACTIVE state when a multicast session has been activated by the CN or the gNB has multicast session data to deliver.

This sentence is a bit misleading because RRC_IDLE and RRC_INACTIVE are used in the same sentence for when the session is activated and there is new multicast data for an active session, i.e. these two cases should be separated.

Some further details are provided below, even though those details should not be captured in 38.300. However it is good to discuss the details, to identify the essential parts for 38.300:

Session activation:

- When a session is deactivated the RAN does not expect new multicast data before the session is activated again by the CN. This applies both for the case the multicast UE is in RRC_INACTIVE and RRC_IDLE.

- The CN triggers group paging for multicast UEs in CM-IDLE before the session is activated (again). The CN sends a MULTICAST GROUP PAGING message to the RAN nodes where multicast UEs are in CM_IDLE. The RAN sends a Paging message including the MBS session ID for UEs in RRC_IDLE.

- The CN sends a MULTICAST SESSION ACTIVATION REQUEST message to the RAN nodes where UEs that have joined the session are in CM_CONNECTED. This enables the RAN nodes to trigger RAN group paging for the multicast UEs that are in RRC_INACTIVE.

  - The anchor gNB can trigger group paging to neighbours in the RNA via Xn interface using RAN MULTICAST GROUP PAGING message (see 38.413 section 8.2.11).

New multicast data during an active session:

- In case a multicast UE is released to RRC_INACTIVE during an active session due to data inactivity, then the last serving (anchor) gNB triggers RAN group paging, i.e. Paging message including MBS session ID, when there is new multicast data.

- In case a multicast UE is released to RRC_INACTIVE during an active session due to data inactivity, the gNB triggers normal RAN Paging, i.e. Paging message including UE identity, when there is new unicast data (see clause 9.2.2.1 in TS 38.300):

If the last serving gNB receives DL data from the UPF or DL UE-associated signalling from the AMF (except the UE Context Release Command message) while the UE is in RRC_INACTIVE, it pages in the cells corresponding to the RNA and may send XnAP RAN Paging to neighbour gNB(s) if the RNA includes cells of neighbour gNB(s).

General comments:

- A group Paging message can be triggered by CN CM-Idle group paging or RAN group paging. UEs in RRC_INACTIVE and RRC_IDLE that monitor the same PO receive this group Paging message. UEs in RRC_IDLE for the MBS session ID to NAS, which triggers connection establishment, while the UEs in RRC_INACTIVE trigger resume procedure.

The text in 38.300 should not include all these details, but it should be clarified that group paging is only used during session activation:

Proposal 3: Clarify the use of group notification for session activation and for new multicast data during an active session separately.

A TP is provided in the Annex.

## There are three different ways to leave

For monitoring of group paging it says in section 16.10.5.2 in 38.300:

The UE stops monitoring for group notifications related to a specific multicast session once the UE leaves this multicast session.

- The UE continues to monitor normal paging and receive the Paging message on PDSCH after it has left the multicast session. When the UE stops monitoring for group notifications it means that the UE does not check PagingGroupList in the Paging message.

- There are three different ways to leave, i.e. three different use cases when the UE stops monitoring for group notifications (see section 7.2.2 in [1]):

  - Multicast session leave requested by the UE

  - Multicast session leave requested by the network

  - Multicast session release by the network

Only case 1 have been explicitly captured.

Proposal 4: Clarify that monitoring for group notifications means checking for MBS session ID in the Paging message.

Proposal 5: Clarify that the UE stops monitoring for group paging when the UE leaves the multicast session or the network releases the multicast session.

# Summary

RAN2 is kindly asked to discuss clarifications for group notification (aka group paging):

Observation 1: gNB should not release a multicast UE to RRC_IDLE while the session is Active.

Proposal 1: Clarify that the gNB may release a multicast UE to RRC_IDLE or RRC_INACTIVE when the CN deactivates the session and there is no unicast data for some time.

Proposal 2: Clarify that the gNB may release a multicast UE to RRC_INACTIVE when the session is activated and there is no multicast nor unicast data for some time.

Proposal 3: Clarify the use of group notification for session activation and for new multicast data during an active session separately.

Proposal 4: Clarify that monitoring for group notifications means checking for MBS session ID in the Paging message.

Proposal 5: Clarify that the UE stops monitoring for group paging when the UE leaves the multicast session or the network releases the multicast session.

# References

- TS 23.247, Architectural enhancements for 5G multicast-broadcast services; Stage 2, SA2, Rel-17

# Annex: TPs 38.300

<TEXT OMITTED>

#### 16.10.5.2	Configuration

A UE can receive data of MBS multicast session only in RRC_CONNECTED state. If the UE which joined a multicast session is in RRC_CONNECTED state and when the multicast session is activated, the gNB sends RRCReconfiguration message with relevant MBS configuration for the multicast session to the UE.

When the multicast session is deactivated and there is temporarily no unicast data, the gNB may move the UE to RRC IDLE or INACTIVE state. When the multicast session is activated and there is temporarily no unicast nor multicast data, the gNB may move the UE to RRC_INACTIVE state. gNBs supporting MBS use a group notification mechanism to notify the UEs in RRC IDLE/INACTIVE state when a multicast session has been activated by the CN. gNBs supporting MBS use a group notification mechanism to notify the UEs in RRC_INACTIVE state when the session is activated and the gNB has multicast session data to deliver. Upon reception of the group notification, the UEs reconnect to the network. The group notification is addressed with P-RNTI on PDCCH, and the paging channels are monitored by the UE as described in clause 9.2.5. Paging message for group notification contains MBS session ID which is utilized to page all UEs in RRC IDLE and RRC INACTIVE states that joined the associated MBS multicast session, i.e., UEs are not paged individually. The UE stops monitoring for group notifications related to a specific multicast session, i.e. stops checking for the MBS session ID in the Paging message, once the UE leaves this multicast session or the network requests the UE to leave, or the network releases the multicast session.

If the UE in RRC IDLE state that joined an MBS multicast session is camping on gNB not supporting MBS, the UE may be notified about multicast session activation or data availability by CN-initiated paging where CN pages each UE individually, as described in clause 9.2.5. If the UE in RRC INACTIVE state that joined MBS multicast session is camping on gNB not supporting MBS, the UE may be notified about data availability individually by RAN-initiated paging, as described in clause 9.2.5.