3GPP TSG-RAN2 Meeting #119-e	R2-2208090

eMeeting, 15-26 August 2022

Agenda Item:	6.9.3 User Plane

Source: 	Ericsson

Title:  	PDCCH skipping in RAN1 and RAN2 specifications

Document for:	Discussion and Decision

# Introduction

Different aspects of PDCCH skipping in RAN1 and RAN2 specifications are discussed.

# Background

RAN2#118-e

During offline #074 [1] it was discussed whether the UE should ignore PDCCH skipping while drx-RetransmissionTimerUL timer is running [2, 3]. A majority of companies preferred option 1 or 2:

Option 1: the UE wakes up to monitor PDCCH when drx-RetransmissionTimerUL is running for CG regardless of the PDCCH skipping duration

Option 2: For configured grant, if UE receives PDCCH skipping command before UL HARQ reTx timer is started, UE should ignore PDCCH skipping.

Summary: 5 Companies support option 1. 5 companies support option 2. 1 company is ok with both option 1 and 2.

Proposal 3: RAN2 to discuss and agree on one of the following:

- Option 1 (6 companies support this): UE wakes up to monitor PDCCH when drx-RetransmissionTimerUL is running for CG regardless of the PDCCH skipping duration

- Option 2 (6 companies support this): For configured grant, if UE receives PDCCH skipping command before UL HARQ reTx timer is started, UE should ignore PDCCH skipping.

- Option 3 (5 companies support this): Leave it to RAN1

The issue was also discussed online, but no agreement was reached, and the chairman concluded to leave it to RAN1:

P3

- Nokia think O1 or O2 are both ok. QC think O2 or O3.

- Vivo think this is being discussed in R1.

- LGE, QC, ZTE, vivo think this is discussed in R1.

Chair: then we leave it to RAN1

RAN1#109-e

RAN1 agreed that PDCCH skipping only applies during Active Time, but did not reach agreement whether the timer is stopped or terminated when the Active Time ends:

Agreement

Down-select from the followings,

- Alt-1: The PDCCH skipping applies only in active time. The PDCCH skipping duration decrements by slot irrespective whether UE is in active time or not

- Alt-2: The PDCCH skipping applies only in active time. PDCCH skipping is terminated when UE goes into outside active time.

Furthermore RAN1 did not reach agreements concerning HARQ interaction, i.e. PDCCH skipping when drx-RetransmissionTimerUL or drx- RetransmissionTimerDL is running:

Way forward 1:

Alt-1: Adopt the following Text Proposal in TS38.213

If a UE is indicated skipping PDCCH monitoring for a duration by a DCI format 1_1 and/or DCI format 1_2 that schedules a PDSCH reception, during the duration the UE monitors PDCCH when the drx-RetransmissionTimerDL of any HARQ process is running, if a PDCCH monitoring adaptation field is received during the duration for skipping PDCCH monitoring, the indication of PDCCH monitoring adaptation field is ignored by the UE.

Alt-2: Adopt the following Text Proposal in TS38.213

If PDCCH skipping duration overlaps with drx-RetransmissionTimerDL, UE should terminate skipping PDCCH monitoring when drx-RetransmissionTimerDL is running.

Alt-3:

Conclusion: PDCCH skipping interaction with HARQ retransmission is not supported in Rel-17.

Way forward 2 (Alt 2):

Adopt the following Text Proposal in TS38.213

If PDCCH skipping duration overlaps with drx-RetransmissionTimerDL, UE should terminate skipping PDCCH monitoring when drx-RetransmissionTimerDL is running.

# Discussion

## PDCCH skipping in RAN2 specifications

PDCCH skipping is a L1 function (captured in section 10.4 in 38.213) that is configured via L3 (see pdcch-SkippingDurationList in 38.331) using UE capability (see pdcch-SkippingWithoutSSS and pdcch-SkippingWithSSSG in 38.306).

There is also a stage 2 description in chapter 11 in 38.300:

UE power saving may also be achieved through PDCCH monitoring adaptation mechanisms when configured by the network, including skipping of PDCCH monitoring and Search space set group (SSSG) switching. In this case UE does not monitor PDCCH during the PDCCH skipping duration or monitors PDCCH according to the search space sets applied in SSSG. However, in the following cases, UE ignores PDCCH skipping:

-	on all serving cells of the corresponding Cell Group when SR is sent and is pending;

-	on SpCell while contention resolution timer is running;

-	on SpCell during monitoring of the RAR/MsgB window.

However these are stage 3 details are typically not captured in 38.300.

Some companies preferred to avoid any impact of PDCCH skipping on 38.321, i.e. 38.321 only refers to 38.213 specification without mentioning PDCCH skipping:

2>	monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6];

PDCCH skipping does not impact Active Time, and for that reason does not need to be captured in 38.321. From a RAN2 perspective the UE is required to monitor PDCCH during Active Time. And in 38.213 it is specified where the UE can find the PDCCH occasions that it needs to monitor during Active Time, except when the PDCCH skipping duration timer is running. The handling of the PDCCH skipping duration timer is specified in 38.213 in details for different use cases.

Dormant BWP, similar as PDCCH skipping, is a L1 function which does not impact Active Time. However the UE is not required to monitor PDCCH on a dormant BWP, which is captured in 38.321:

Dormant BWP: The dormant BWP is one of downlink BWPs configured by the network via dedicated RRC signaling. In the dormant BWP, the UE stop monitoring PDCCH on/for the SCell, but continues performing CSI measurements, Automatic Gain Control (AGC) and beam management, if configured.

Proposal 1: Clarify in 38.321 that when PDCCH skipping is configured by RRC the UE does not monitor PDCCH for a duration as specified in TS 38.213:

When PDCCH skipping is configured by RRC the UE does not monitor PDCCH for a duration as specified in TS 38.213 [6].

Proposal 2: Move the following stage 3 requirements from 38.300 to 38.321:

However, in the following cases, UE ignores PDCCH skipping:

-	on all serving cells of the corresponding Cell Group when SR is sent and is pending;

-	on SpCell while contention resolution timer is running;

-	on SpCell during monitoring of the RAR/MsgB window.

A TP for 38.321 is provided in the Annex.

## PDCCH skipping and HARQ interaction

drx-RetransmissionTimerUL

Some companies in RAN1 thought that the UE should not skip when drx-RetransmissionTimerUL is running. Others thought that the UE skipping should be allowed when the PDCCH skipping is triggered by UL grant DCI.

drx-RetransmissionTimerDL

RAN1 could not reach agreement if the skipping duration timer is ignored or terminated when drx-RetransmissionTimerDL is running.

RAN1 also discussed as a possible outcome/alternative that PDCCH skipping interaction with HARQ retransmission is not supported in Rel-17. The implications of such outcome should be discussed further and properly understood:

Observation 1: In case the PDCCH skipping interaction with HARQ retransmission is not supported in Rel-17, it is not clear whether the UE behavior is unspecified for certain cases, or that the NW is assumed to avoid any ambiguity.

Proposal 3: Wait for the RAN1 outcome for PDCCH skipping interaction with HARQ retransmission

# Summary

RAN2 is kindly asked to discuss PDCCH skipping in RAN1 and RAN2 specifications:

Proposal 1: Clarify in 38.321 that when PDCCH skipping is configured by RRC the UE does not monitor PDCCH for a duration as specified in TS 38.213:

When PDCCH skipping is configured by RRC the UE does not monitor PDCCH for a duration as specified in TS 38.213 [6].

Proposal 2: Move the following stage 3 requirements from 38.300 to 38.321:

However, in the following cases, UE ignores PDCCH skipping:

-	on all serving cells of the corresponding Cell Group when SR is sent and is pending;

-	on SpCell while contention resolution timer is running;

-	on SpCell during monitoring of the RAR/MsgB window.

Observation 1: In case the PDCCH skipping interaction with HARQ retransmission is not supported in Rel-17, it is not clear whether the UE behavior is unspecified for certain cases, or that the NW is assumed to avoid any ambiguity.

Proposal 3: Wait for the RAN1 outcome for PDCCH skipping interaction with HARQ retransmission

# References

- R2-2206487 Report of [AT118-e][074][ePowSav] PDCCH skipping, Samsung, Report, RAN2#118-e

- R2-2204732, Discussion on PDCCH skipping, OPPO, DISC, RAN2#118-e

- R2-2205024, Remaining issues on PDCCH adaptation, Nokia, DISC, RAN2#118-e

- R1-2205278, FL summary#2 of DCI-based power saving adaptation, Moderator (vivo), Report, RAN1#109-e

# Annex: TP 38.321

<TEXT OMITTED>

## 5.7	Discontinuous Reception (DRX)

The MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CI-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, AI-RNTI, SL-RNTI, SLCS-RNTI and SL Semi-Persistent Scheduling V-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other clauses of this specification. When in RRC_CONNECTED, if DRX is configured, for all the activated Serving Cells, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this clause; otherwise the MAC entity shall monitor the PDCCH as specified in TS 38.213 [6].

NOTE 1:	Void

<TEXT OMITTED>

1>	if a DRX group is in Active Time:

2>	monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6];

2>	if the PDCCH indicates a DL transmission; or

2>	if the PDCCH indicates a one-shot HARQ feedback as specified in clause 9.1.4 of TS 38.213 [6]; or

2>	if the PDCCH indicates a retransmission of HARQ feedback as specified in clause 9.1.5 of TS 38.213 [6]:

<TEXT OMITTED>

Regardless of whether the MAC entity is monitoring PDCCH or not on the Serving Cells in a DRX group, the MAC entity transmits HARQ feedback, aperiodic CSI on PUSCH, and aperiodic SRS defined in TS 38.214 [7] on the Serving Cells in the DRX group when such is expected.

The MAC entity needs not to monitor the PDCCH if it is not a complete PDCCH occasion (e.g. the Active Time starts or ends in the middle of a PDCCH occasion).

When PDCCH skipping is configured by RRC the UE does not monitor PDCCH for a duration as specified in TS 38.213 [6]. In the following cases the UE ignores PDCCH skipping:

-	on all serving cells of the corresponding Cell Group when SR is sent and is pending;

-	on SpCell while contention resolution timer is running;

-	on SpCell during monitoring of the RAR/MsgB window.

<TEXT OMITTED>