3GPP TSG-RAN WG2 #119e	R2-2208103

Electronic meeting, 2022-08-17 - 2022-08-29

Agenda Item:	8.12.2

Source:	Ericsson

Title:	Mobility enhancements for mIAB node

Document for:	Discussion, Decision

# 1	Introduction

One of the agendas for Rel-18 Mobile IAB is to discuss the necessary mobility enhancement needed to support the mobile-IAB.

### Mobility Enhancements Enhancements for mobility of an IAB-node together with its served UEs, including aspects related to group mobility. No optimizations for the targeting of surrounding UEs. [RAN3, RAN2]

# 2	Discussion

## 2.1	Identification of Mobile IAB cell

When the UE accesses a cell, it should be able to understand what sort of cell it is. In legacy as such there is only static cells whereas now with mobile IAB the cells would be mobile. Hence, at least the new release UEs (supporting rel-18) should be able to understand whether the connectivity is via mobile IAB cell or static cell.

There can be few benefits of identifying if it is connected via mobile IAB cell such as:

- No need of handover/cell reselection if it is connected via mobile IAB cells

- Relaxed neighbour cell measurement

Further, it is possible for the UEs to keep track of serving cell signal strength and as long as the RSRP is within certain limit and with a marginal variance can understand if it is still in mobile cell. However, if there are fluctuations and serving cell signal strength is no longer with in certain range, the UE may realize that it has got out of the mobile cell (bus, train etc), and it may need to perform cell re-selection to static cell, i.e., no longer the relaxed neighbour cell measurement is applicable.

Early identification of a mobile IAB cell may help a UE to avoid the triggering of unnecessary RRC procedures.

There can be different ways to indicate whether cell is mobile or static. One simple way would be via System information broadcast. Her, also additional information useful for the UE that can be broadcast such as:

- From operator perspective, this can be additional service, i.e., to provide connectivity via mobile IAB cells. Hence, not every UE may be able to access such cell. Thus, in these operator deployments there is need for authorization, e.g., subscription-based access as not every UE should have access (e.g., only UEs who have purchased the bus ticket). This authorization should be however discussed in SA2

- In some network (operator deployment) though the mobile IAB cell could be used only for public safety or emergency use cases. Hence, only certain class of traffic type could be allowed to access such cell.  The access class category should be provided.

- The mobile IAB location and speed. This would be useful for the UEs to determine if they are still in the bus or have left.

SIB-IAB ::=                             SEQUENCE {

latitudeSign							ENUMERATED {north, south},

degreesLatitude							INTEGER (0..8388607),			-- 23 bit field

degreesLongitude						INTEGER (-8388608..8388607),	-- 24 bit field

uncertainty								INTEGER (0..127)

velocity								INTEGER (0..511),

distanceThreshold						INTEGER (0..1023),

rsrp-Threshold							INTEGER (0..127),

rsrq-Threshold							INTEGER (0..127),

accessClass								ENUMERATED {onlyPublicSafety} OPTIONAL, --Need ON

allowedAccessClass						ENUMERATED {n14, n12} OPTIONAL, --Need ON

barreddAccessClass						ENUMERATED {n11, n15} OPTIONAL, --Need ON

pathID									INTEGER(1..10)

}

Figure 1: Example of system information block for mobile IAB

According to this, the following proposal is formulated:

RAN2 to discuss the need of provide mobile IAB specific information to the UE, such as mobile IAB’s position, velocity, access class/category and any other parameters to help the UE in performing (or avoiding) certain RRC procedures.

## 2.2	Usage of pre-configurations for handling of mobile IAB node mobility

Previous release discusses the migration of static IAB node for intra-CU and inter-CU scenarios mainly because of RLF and congestion. This release needs to also address the IAB-node migration because of mobility. The IAB-node migration is considered to be a complex procedure as lot of config parameter needs to be provided again especially for inter-CU case. Further, there is also risk of service interruption etc.

However, one aspect that can be exploited and the signalling aspects can be minimized is by using the known routes/path of the mobile IAB node. If it is known that a city bus equipped with a mobile IAB cell follows a certain route and at a particular time reaches a particular destination then this known traffic pattern can be exploited such that the configurations can be stored and activated when the mobile IAB node reaches to a new cell (parent IAB node, new CU) etc. There can be some parameters which can be stored in the mobile IAB node whereas there can be some parameters that can be stored in each static parent IAB nodes which a mobile IAB would connect to.

Figure 2: Mobile IAB-Node which involves Intra-Donor, Inter-Donor (same CU) and Inter CUs.

In above figure, the mobile IAB node maintains an F1 connection to the donor (one donor at a time). In the figure, the mobile IAB connects to the following nodes in the different positions as described below:

- Position A: BH through parent node 1, F1 connection to donor node X

- Position B: BH through parent node 2, F1 connection to donor node X

- Position C: BH through parent node 3, F1 connection to donor node Y

- Position D: BH through parent node 4, F1 connection to donor node Y

The mobile IAB must change the F1 connection from donor X to donor Y when moving from position B to C, thus requiring a F1 handover. The F1 handover can be simplified by providing the needed IP addressing to connect to the new donor in advance. Similar to F1, the BAP configuration can be prestored in mIAB which would allow access to reach to parent node (for upstream traffic). The stationary node which connects to mIAB node can also store the BAP configuration to reach to the mIAB (for downstream traffic).

In a similar scenario, if the legacy procedure is used this means that the BAP configuration at the mobile IAB needs to be dynamically reconfigured every time that the mobile IAB will traverse a new parent IAB node or donor node. Nevertheless, since the features specified so far in the previous releases did not consider that an IAB node may be moving, the results are that a continuous reconfiguration (at least the F1 setup and BAP configuration) of the mobile IAB may not be efficient and may react slowly to the changes caused by the mobility.

Legacy IAB procedures may incur in high latency and overhead signalling when reconfiguring an IAB node due to its mobility.

Of course, even if the mobile IAB is moving, one aspect that can be exploited when considering F1 and RRC procedure is that its mobility may be predictable. One example is to consider that a mobile IAB mode may be mounted on public transportation vehicle such as public busses or trains. In such a case, this type of vehicle has a predictable mobility that can be exploited when considering that handovers may already been known in advance. In such a case, some F1 and RRC procedure such as the F1 setup and the BAP (re)configuration may be provided in advance (preconfigured) at the mobile IAB. An example of what the procedure would like is shown in Figure 2.

Predictable mobility may be used to determine the route of the mobile IAB node and to provide configurations in advance to the mobile IAB in order to execute F1 or RRC procedures (e.g., for providing handover commands in advance).

Figure 2: Donor nodes provide the necessary F1 and BAP configurations to be stored in advance and activated when mIAB is handed over to new parent/new Donor.

In conclusion, a solution where certain configuration a predefined (or provided in advance) at the UE would be great benefits from a latency/connectivity interruption and signalling overhead point of view. At least this is an option that it may work on top of the existing procedure for the IAB nodes.

In addition to legacy IAB procedures, RAN2 to also consider a solution where certain configurations (e.g., BAP configuration) are predefined at the mobile IAB to reduce the connectivity interruption and signalling overhead during mobility.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Early identification of a mobile IAB cell may help a UE to avoid the triggering of unnecessary RRC procedures.

Observation 2	Legacy IAB procedures may incur in high latency and overhead signalling when reconfiguring an IAB node due to its mobility.

Observation 3	Predictable mobility may be used to determine the route of the mobile IAB node and to provide configurations in advance to the mobile IAB in order to execute F1 or RRC procedures (e.g., for providing handover commands in advance).

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to discuss the need of provide mobile IAB specific information to the UE, such as mobile IAB’s position, velocity, access class/category and any other parameters to help the UE in performing (or avoiding) certain RRC procedures.

Proposal 2	In addition to legacy IAB procedures, RAN2 to also consider a solution where certain configurations (e.g., BAP configuration) are predefined at the mobile IAB to reduce the connectivity interruption and signalling overhead during mobility.

# 4	References

RP-221815, WID on Mobile IAB for NR, 3GPP TSG RAN#96, Budapest, Hungary, June 2022