3GPP TSG-RAN WG2 Meeting #119-e	R2-2208111

# Online, August 17th – 29th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0529 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on RedCap-specific initial UL BWP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_Core |  |  |  |  |  | Date: |  |  | 2022-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Regarding UE behaviour of RACH procedure, according to TS38.321, the UE performs UL carrier selection first and then BWP selection. After carrier selection, if RedCap-specific initial UL BWP is configured, then the UE will select RedCap-specific initial UL BWP to perform RACH. Extracted from clause 5.1.1: When the Random Access procedure is initiated on a Serving Cell, the MAC entity shall: *** skip non-related part*** 1> if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL: 2> select the SUL carrier for performing Random Access procedure; 2> set the PCMAX to PCMAX,f,c of the SUL carrier. 1> else: 2> select the NUL carrier for performing Random Access procedure; 2> set the PCMAX to PCMAX,f,c of the NUL carrier. NOTE 4: Void. 1> perform the BWP operation as specified in clause 5.15; Extracted from clause 5.15: Upon initiation of the Random Access procedure, after selection of the carrier for performing Random Access procedure as specified in clause 5.1.1, if the UE is a RedCap UE in RRC_IDLE or RRC_INACTIVE mode, the MAC entity shall: 1> if initialUplinkBWP-RedCap is configured: 2> perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP-RedCap. 1> else: 2> perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP. 1> if initialDownlinkBWP-RedCap is configured: 2> monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap. 1> else: 2> monitor the PDCCH on the BWP configured by initialDownlinkBWP.. Based on the conclusion in RAN#95e, a single RedCap-specific initial UL BWP is introduced, and it is applicable for NUL. RedCap-specific initial UL BWP is not supported for SUL. RAN#95e conclusion: CR R2-2204267 is approved (with the understanding that SUL operation for RedCap is not precluded by this CR) CR R2-2204268 is not pursued. However, in current TS 38.300 spec, it says: If a RedCap-specific initial UL BWP is configured, RedCap UEs in RRC_IDLE and RRC_INACTIVE shall use only the RedCap-specific initial UL BWP to perform RACH.. Whic means that as long as RedCap-specific initial UL BWP is broadcast in SIB1, the RRC_IDLE and RRC_INACTIVE RedCap UEs can only select RedCap-specific initial UL BWP to perform RACH, irrespective of NUL/SUL carrier selection. This is conflicted with TS 38.321, and will result in the UE not being able to select SUL when the RSRP is below rsrp-ThresholdSSB-SUL. So this CR is provided to update the incorrect description in TS 38.300. |
| Summary of change: |  | Clarify that RedCap UEs in RRC_IDLE and RRC_INACTIVE shall use only RedCap-specific initial UL BWP to perform RACH when NUL is selected. “If a RedCap-specific initial UL BWP is configured and NUL is selected, RedCap UEs in RRC_IDLE and RRC_INACTIVE shall use only the RedCap-specific initial UL BWP to perform RACH.” Impact analysis Impacted 5G architecture options: NR SA Impacted functionality: RACH procedure Inter-operability: - If the network implements the CR and the UE does not, in case UE’s RSRP is below rsrp-ThresholdSSB-SUL, the network assumes the UE will select SUL for RACH, but the UE will only select NUL and perform RACH on RedCap-specific initial UL BWP. RACH procedure may fail due to the poor signal. - If the UE implements the CR and the network does not, in case UE’s RSRP is below rsrp-ThresholdSSB-SUL, the network assumes the UE can only select NUL and perform RACH on RedCap-specific initial UL BWP, but the UE will select SUL to perform RACH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When RedCap-specific initial UL BWP is configured, the RedCap UEs in RRC_IDLE and RRC_INACTIVE are not able to select SUL even if the RSRP is below rsrp-ThresholdSSB-SUL. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.13.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

## 16.13	Support of Reduced Capability (RedCap) NR devices

*** ignore non-related part***

### 16.13.5	BWP operation

A RedCap UE monitors paging only in an initial BWP (default or RedCap specific) associated with CD-SSB and performs cell (re-)selection and measurements on the CD-SSB. If a RedCap-specific initial UL BWP is configured and NUL is selected, RedCap UEs in RRC_IDLE and RRC_INACTIVE shall use only the RedCap-specific initial UL BWP to perform RACH.

A RedCap UE may be configured with multiple NCD-SSBs provided that each BWP is configured with at most one SSB. NCD-SSB may be configured for a RedCap UE in RRC_CONNECTED to perform RLM, BFD, and RRM measurements when the active BWP does not contain CD-SSB.

End of change