3GPP TSG-RAN WG2 #119e	Tdoc R2-2208113

Electronic meeting, 2022-08-17 - 2022-08-29

Agenda Item:	8.10.3

Source:	Ericsson

Title:	TDM Solution for NR IDC

Document for:	Discussion, Decision

# 1	Introduction

The IDC WID has the following objectives, in were TDM solutions have the following targets:

3 Justification As a general assumption, IDC is assumed to work as follows: - The UE detects internal issue or the possibility of internal issue caused by coexistence related to usage of certain radio resources, that the UE cannot resolve by itself. - The UE provides information to the gNB to assist that the gNB may restrict radio resource usage to avoid the UE internal issue (or potential issue) caused by coexistence. The current IDC solution in NR has the following limitations: It does not support well interference mitigation between 3GPP and other RAT, as e.g. the affected frequencies cannot be adequately indicated via the NR FDM solution. Introducing a TDM solution would make it possible to handle scenarios for which alternative non-interfered frequencies are not available. The TDM solution may be used to avoid the interference caused by simultaneous uplink transmission on the UL frequencies to non-3GPP RAT. 4 Objective 4.1 Objective of SI or Core part WI or Testing part WI This WI expects to address interference between 3GPP (including various MR-DC architectures, i.e. NR-DC and EN-DC) and non-3GPP RAT (e.g. WiFi). - Enhancements to FDM solution, to allow more granular indication of affected frequencies (e.g. granularity of BWP or PRB level). (RAN2) Note: Enhancements to FDM solution is prioritized. - Introduction of TDM solution (e.g. indication of UE preferred TDM pattern for UL/DL). (RAN2, RAN4). Note: The TDM solution is considered complementary to the FDM solution. - Specify RRM requirements for TDM solution (RAN4) Note: LTE IDC solution should be considered as the baseline for the solutions developed in this WI.

In this contribution we look at TDM solutions for NR and general considerations that can be made based on the work in LTE and guidance given in the WID.

# 2	Discussion

The In-Device Coexistence (IDC) feature was introduced in LTE and NR. This feature was introduced to tackle situations when a UE is operating multiple radio technologies and the radios used for these different technologies cause interference to each other.

For the IDC TDM solution in LTE specifically, the following options have been defined:

- preferred DRX configuration reported by the UE

- preferred UL/DL sub-frame allocation reported by the UE

- Autonomous denial

As an example, the preferred DRX configuration is given as (3GPP TS 36.331):

drx-AssistanceInfo-r11              SEQUENCE {

drx-CycleLength-r11                 ENUMERATED {sf40, sf64, sf80, sf128, sf160,

sf256, spare2, spare1},

drx-Offset-r11                      INTEGER (0..255)    OPTIONAL,

drx-ActiveTime-r11                  ENUMERATED {sf20, sf30, sf40, sf60, sf80,

sf100, spare2, spare1}

},

an example of the preferred UL/DL sub-frame allocation indication:

IDC-SubframePatternList-r11 ::= SEQUENCE (SIZE (1..maxSubframePatternIDC-r11)) OF IDC-SubframePattern-r11

IDC-SubframePattern-r11 ::= CHOICE {

subframePatternFDD-r11              BIT STRING (SIZE (4)),

subframePatternTDD-r11              CHOICE {

subframeConfig0-r11                 BIT STRING (SIZE (70)),

subframeConfig1-5-r11               BIT STRING (SIZE (10)),

subframeConfig6-r11                 BIT STRING (SIZE (60))

},

And example of autonomous denial:

autonomousDenialParameters-r11      SEQUENCE {

autonomousDenialSubframes-r11           ENUMERATED {n2, n5, n10, n15,

n20, n30, spare2, spare1},

autonomousDenialValidity-r11            ENUMERATED {

sf200, sf500, sf1000, sf2000,

spare4, spare3, spare2, spare1}

}       OPTIONAL,       -- Need OR

In the Rel-18 WID for NR IDC [1], and for a TDM solution specifically the following principals can be made for the coming work on IDC:

- With enhancements to FDM solution prioritized, a TDM solution should be limited to an alternative option for important cases/situations where a FDM solution is not adequate.

- A NR TDM solution should reuse the LTE framework as a starting point.

The preferred DRX pattern giving the scheduling periods in LTE was designed based on analysis on traffic also including BT/WLAN. As a result, the properties of that can be assumed also to apply to NR. Additional means to this indication needs to be evaluated. For example, it may be so that for NR IDC needs to cover more configurations with additional DRX cycle lengths.

The preferred UL/DL sub-frame allocation indication in LTE can be reused if RAN2 concludes that a DRX pattern solution is deemed insufficient. It can be noted that a solution entailing UL/DL sub-frame allocation will come with additional complexity in that a solution based on LTE needs to be amended in how to indicate the slot granularity due to numerology dependency, subframe pattern by using asynchronous HARQ and how DL/UL interference for different frequencies impacts the solution.

For autonomous denial, as there could be large differences between NR and LTE as to the number of subframes/slots which can be dropped while still meeting performance requirements, a solution adopted for NR needs a detailed study also in RAN4.

From the above, it is concluded that it is beneficial to initially study a DRX based mechanism as a baseline starting point, as this has the basis for traffic pattern, may have less RAN4 impact and includes (LTE studied) aspects also in other technical areas. Only if this is not sufficient RAN2 should explore a solution based on UL/DL sub-frame pattern, Autonomous denial or other.

As a baseline, a TDM solution for NR IDC is using signalling of a preferred DRX pattern.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	As a baseline, a TDM solution for NR IDC is using signalling of a preferred DRX pattern.

# References

[1] RP-221281, Xiaomi, “Revised WID: In-Device Co-existence (IDC) enhancements for NR and MR-DC”.