# 3GPP TSG RAN WG2 Meeting #119-e                     	    R2-2208126 E-meeting, 17 – 26 August 2022

Agenda item:	8.2.2

Source:	Qualcomm Incorporated

Title:	Study of Sidelink Positioning Architecture, Signaling and Procedures

WID/SID:

Document for:	Discussion and Decision

# Introduction

This contribution discusses architecture, functionality, and procedures for support of UE-based and network-based sidelink positioning and ranging to address the requirements in RP-221814 Study on expanded and improved NR positioning [1].  This contribution emphasizes protocol aspects of sidelink positioning, recognizing lower-layer aspects, including positioning reference signals, will be addressed by RAN1 working group.

RP-213588 Study on expanded and improved NR positioning 4.1 Objective of SI or Core part WI or Testing part WI [text omitted] - Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

# Discussion

Sidelink Positioning for relative positioning, ranging and absolute positioning may include UE discovery, capability exchange, measurement configuration, and exchange of location information resulting from performed measurements.  Depending on the deployment scenario and operational use case, sidelink positioning and ranging may occur between UEs, or between UEs and an LMF, potentially over multiple cast types and access technologies.  To realize this capability in a manner satisfying the desired use cases (V2X, public safety, commercial and IIOT) and within the time frame and work scope of the release, RAN2 should study approaches minimizing and/or not requiring changes to the underlying sidelink transport.

Fundamental to sidelink positioning and ranging is UE peer-to-peer operation.  As such, the current 5GS architecture, using network-based location servers (user plane location servers or control plane location servers), is not sufficient for sidelink positioning.  To address this limitation UEs should be capable of calculating position and/or range based on sidelink positioning and ranging measurements, and support a UE-to-UE Sidelink Positioning Protocol (SLPP).

A possible architecture for sidelink positioning and ranging is shown in Figure 1 [2] (the illustrated architecture is for a non-roaming, same PLMN operation, but is readily extensible to inter-PLMN and roaming scenarios).  This architecture supports sidelink positioning and ranging for in-coverage, out-of-coverage, or partial-coverage operation.  A Sidelink Positioning/Ranging Function (SPRF) provides Sidelink Positioning Protocol (SLPP) between UEs to configure measurements and exchange measurement results.  The SPRF may also provide location and range computation for those UEs capable of determining location/range on behalf of other UEs.  For scenarios involving an LMF, through the SPRF, the LMF may request UE capability information, configure measurements, receive measurement results, and perform location and range computation on behalf of participating UEs.  Two or more UEs may participate in a sidelink positioning and ranging session through exchange of discovery and positioning protocol over the SR5 reference point.  SR5 is carried over the PC5 reference point and provides the interface for discovery and SLPP positioning protocol between UEs.

Observation 1: To support sidelink positioning and ranging a UE should support a sidelink positioning and ranging protocol (SLPP).

Observation 2: To support sidelink positioning and ranging a UE should be capable of being enabled with the ability to calculate position and or range based on sidelink positioning and ranging measurements.

Observation 3: A UE-based Sidelink Positioning and Ranging Function (SPRF) communicating over the SR5 reference point can be used to provide location and range computation and sidelink positioning and ranging protocol (SLPP) functionality.

Proposal 1: RAN2 should study a Sidelink Positioning and Ranging Function (SPRF) including positioning protocol functionality and location and range calculation based on sidelink positioning and ranging measurements.

Proposal 2: RAN2 should study a Sidelink Positioning Protocol (SLPP) to support UE-to-UE sidelink positioning and ranging.

Figure 1: Reference architecture for Sidelink Positioning and Ranging-based services for non-roaming and same PLMN operation

## Transport and Protocol Layering

Transport mechanisms over the SR5 interface used for discovery and sidelink positioning protocol (SLPP) for sidelink positioning and ranging should provide sufficient flexibility to address all use cases and cast types.  Over PC5, possible SLPP transport mechanisms RAN2 may consider for discovery and positioning protocol exchange include the following:

- PC5-Signaling (PC5-S):

PC5-S enables sidelink positioning and ranging over a unicast link between two UEs.  SLPP transport over PC5-S limits sidelink positioning and ranging to unicast, excluding sidelink positioning support over groupcast (NR PC5) and broadcast (NR PC5 and LTE PC5), resulting in a constrained solution excluding applications such as V2X from effectively using sidelink positioning and ranging.

- PC5-Signaling enhanced to support non-unicast cast types:

RAN2 could enhance PC5-S to support SLPP over groupcast and broadcast.  Such an enhancement would introduce significant flexibility to PC5-S, but would likely require substantial standards modification, a challenge that may be precluded considering the large scope of the current release.  This approach would also not enable sidelink positioning and ranging support for applications operating over currently deployed LTE PC5, including V2X.

- PC5 User Plane (PC5-U):

Operation over PC5 reference point as data traffic (PC5-U) enables sidelink positioning and ranging over all cast types and all PC5 Radio Access Technologies (RATs).  SLPP can be transported as data traffic with no modification to the PC5 reference point required.

The evaluation of possible PC5 transport mechanisms for SPRF SLPP over the SR5 reference point suggests transport over the PC5 user plane provides the greatest flexibility, with no modifications to the PC5 reference point needed.  A protocol layering for SPRF SLPP communication over PC5-U is shown in Figure 2.  This protocol layering is suitable for inter-UE sidelink positioning and ranging operation for in-coverage, out-of-coverage and partial coverage.  When a network LMF supporting a SPRF is a participant in a sidelink positioning and ranging session, a UE may send measurements to the LMF for the LMF to assist the UE in performing sidelink positioning and ranging, or for the LMF to compute the results of a sidelink positioning and ranging session.  SLPP sent to, or received from the LMF SPRF can be sent using existing NAS without requiring any modification to the PC5 reference point or the Uu reference point.

Observation 4: Exchange of Sidelink Positioning and Ranging Function (SPRF) messaging can be accomplished without modification to the PC5 reference point over the PC5 user plane.

Proposal 3: RAN2 should study SPRF SLPP transport over PC5 mechanisms providing flexibility in cast type and minimizing or introducing no change to the PC5 reference point.

Figure 2: SLPP protocol layering over PC5 reference point

## Architecture

The architecture described in Figure 1 supports discovery and positioning protocol carried over the SR5 reference point, over the PC5 reference point with no changes required to the PC5 reference point.  This architecture can be mapped to the RAN2 UE overall positioning architecture [2] as shown in Figure 3.  In-coverage UEs (UE A, UE B) and out-of-coverage UEs (UE C, UE D) conduct sidelink positioning and ranging over the PC5 reference point using SLPP.  Network components, such as an LMF supporting SLPP, can provide UE assistance by requesting UE capabilities, configuring measurements, receiving measurement results, and computing resultant positions or relative locations.  LMF support of SPRF, and operation as an SLPP endpoint, enables the LMF to support in parallel both sidelink positioning and Uu positioning, advancing the release objective of “covering both UE based and network based positioning”.

NOTE:	“In-coverage UEs” refer to UEs that are in coverage of a PLMN and are subscribed to receive PLMN service. “Out-of-coverage UEs” refer to UEs that are either out of coverage of any PLMN or are in coverage of one or more PLMNs for which there is no subscription to receive service.

Observation 5: A sidelink positioning and ranging network architecture supporting SPRF, discovery and SLPP can be realized without requiring enhancements to the PC5 reference point.

Proposal 4: RAN2 should study a sidelink positioning and ranging architecture supporting UE and LMF Sidelink Positioning Ranging Function (SPRF).

Figure 3: UE Positioning Overall Architecture applicable to NG-RAN

## Functionality

A SPRF can provide the following functionality in support of a sidelink positioning and ranging service:

- SLPP: SLPP messages over the SR5 reference point enable configuring and executing a sidelink positioning and ranging session. This may include capability transfer, measurement configuration and location information exchange.  SLPP can be a separate protocol from LPP, such that SLPP can be supported without having to necessarily support LPP.

- Location and range calculation: UEs capable of performing location or range calculations may do so on behalf of other UEs participating in a sidelink positioning and ranging session based on location information exchanged via SLPP.  When an LMF is a participant in the sidelink positioning and ranging session the LMF may perform location and range calculation for participating UEs through its SPRF.

An example call flow for a sidelink positioning and ranging exchange is shown in Figure 4.  Device discovery is conducted over the SR5 reference point.  After device discovery, UEs may communicate via SLPP over the SR5 reference point to perform capability transfer and measurement configuration.  Once measurement configuration is established, UEs transmit and measure respective Positioning Reference Signals (PRS), as defined by RAN1.  SLPP is used to exchange measurement results (location information), and range and position calculation is performed by participating UEs, or may be performed by one UE on behalf of another UE.

Proposal 5: RAN2 should study the SPRF SLPP between UEs over the SR5 reference point, considering capability transfer, assistance data transfer and location information transfer.

Figure 4: Sidelink positioning Device Discovery, SLPP

# Positioning Protocol Procedures

## SLPP Session and Session-less Modes

With LPP [3], a UE with NR or LTE access to 5GC and an LMF engage in an LPP session to locate the UE using position methods defined for LPP. Similarly, for SLPP, a pair of UEs or a group of UEs, and possibly one or more LMFs, could be used in a session mode in which they engage in an SLPP session to obtain absolute locations, relative locations or ranges for the participating UEs based on position methods supported in SLPP.  Participating UEs should be cognizant of the other participating UEs in order to effectively apply measurements received from other UEs.  This awareness of SLPP session participants implies benefit in having a mechanism to establish an SLPP session in some formal manner and identifying which UEs are participants in that SLPP session.  For example, consider a scenario where a UE A has discovered UE B and UE C; UE B has discovered UE A but not UE C; and UE C has discovered UE A but not UE B. Which of the UEs A, B and C should participate in an SLPP session, and how each UE is notified of the participants is beyond the capability of Discovery alone and instead suggests additional capability is required. Such session management capability could be part of SLPP. Once an SLPP session has been established, positioning can proceed.

Observation 6: UEs participating in sidelink positioning and ranging should be aware of the other participating UEs in order to effectively apply measurements received from the other UEs.

Figure 5 illustrates a possible overall procedure in which an SLPP session is created to support positioning among a group of UEs 1 to n. At step 1, the UEs 1 to n discover one another. At step 2, an SLPP session is established between UEs 1 to n based on the discovery information from step 1. At step 3, positioning is performed among UEs 1 to n using SLPP (as described in section 3.2 and 3.3). At step 4, and as an option, the SLPP session might be modified by adding and/or removing UEs (e.g. if some of UEs 1 to n are no longer in PC5 range of other UEs and/or if other UEs move within PC5 range of UEs 1 to n). For example, in a dynamic V2X environment, it is quite likely that any initial set of UEs 1 to n will not stay within PC5 range of one another for any long period of time. At step 5, also as an option, the SLPP session can be explicitly terminated.

Figure 5: SLPP Positioning based on an SLPP Session

Figure 6 provides one example of how the session establishment at step 2 of Figure 5 might operate if UE1 acting as a coordinating UE (e.g. UE1 is an RSU for V2X) were to separately discover each of the other UEs 2 to n and then start an SLPP session for all of the UEs 1 to n.

Figure 6: Starting an SLPP Session among a group of UEs 1 to n

At step 1 in Figure 6, UE1 discovers each of the other UEs 2 to n, including their capability to support SLPP. At steps 2 to 4, UE1 invites each of the UEs 2 to n to become part of an SLPP session. Possibly not all of UEs 2 to n would accept joining an SLPP session, which is one reason for performing steps 2 to 4.  At step 5, UE1 broadcasts a message to all UEs 2 to n to initiate an SLPP session.  UE1 may include the addresses and possibly UE IDs used in SLPP for each of the UEs 1 to n. The group address used might be provided to each UE at one of steps 2 to 4. The participating UEs then individually acknowledge the session start, subsequent to which the positioning within the session can start.  If the coordinating UE1 needs to modify the SLPP session after the session has started (e.g. if some of UEs 2 to n move out of range of UE1 and/or if UE1 discovers other UEs nearby which might join the session), UE1 can repeat steps similar to steps 2-4 for any new UEs and repeat steps 5 and 6 to restart the session and include addresses and IDs only for UEs which will be part of the restarted session for the repetition of step 5.

Observation 7: SLPP sessions may be used between 2 or more UEs to support sidelink positioning in a structured and coordinated manner.

The SLPP session mode described requires an explicit SLPP session between participating UEs which may result in additional signaling overhead (e.g. all the steps shown in Figure 5).  As an alternative approach, a group of UEs might perform positioning using SLPP in a session-less mode in which there is no discovery, no UE associations and no SLPP sessions. The motivation would be to minimize signaling overhead and enable instant adaptation to changing UE conditions. For example, on a busy highway, there might be many UEs near enough to one another to exchange signaling and measure PRS, but where the set of nearby UEs changes every few seconds such that SLPP sessions, if used, might have to also change, introducing additional signaling overhead. This session-less approach is illustrated in Figure 7.

Figure 7: SLPP Session-less mode between UEs 1, 2 and 3

Although Figure 7 illustrates a session-less approach for 3 UEs, the principles are extensible to any number of UEs.  At step 1, each UE broadcasts an SLPP message indicating when the UE intends to broadcast a reference signal (e.g. sidelink PRS) and a configuration for the reference signal (e.g. frequency, bandwidth, coding, duration, timing, etc).  As potential for collisions may be present, UEs could first listen for SLPP messages from other UEs and avoid reusing the same radio resources. There is also an assumption that UEs share some common time reference, which could be based on GNSS or on transmission timing from a network or nearby RSUs (in the case of V2X).  At step 2 UEs broadcast their reference signals as indicated at step 1 and measure reference signals broadcast by other UEs, also as indicated at step 1.  At step 3 UEs exchange measurements (also via broadcast) and determine RTTs (and possibly bearings) based on performed measurements and measurements received from other UEs at step 4.  Steps 1 to 4 can then be repeated.

Observation 8: SLPP could be used in a session-less mode between 2 or more UEs to support sidelink positioning with reduced signaling overhead and greater adaptivity to changing UE locations and other changing conditions.

The examples in Figure 5 to Figure 7 provide possible directions for study in RAN2 to determine their suitability. RAN2’s study could include the following aspects:

- SLPP sessions

Session establishment, modification and termination

Signaling Overhead

- Session-less SLPP

Signaling overhead versus session oriented SLPP

Means of identifying UEs

Supporting a common time reference (e.g. UTC or GNSS)

Proposal 6: RAN2 should study solutions that support SLPP sessions and session-less use of SLPP and evaluate associated benefits, limitations, overheads and impacts.

## SLPP Operation without LMF Support

The out-of-coverage requirement for sidelink positioning implies UEs be able to conduct sidelink positioning and ranging without network or LMF support.  To accommodate this mode of operation, SLPP procedures should enable UEs participating in a sidelink positioning and ranging session to configure and execute a sidelink positioning and ranging session, including sharing respective capabilities, configuring required measurements, and exchanging measurement results.  SLPP should also enable at least one or more UEs to determine the relative or absolute position for the participating UEs.

Proposal 7: Sidelink positioning protocol (SLPP) should enable sidelink positioning and ranging between UEs when no network coverage is present (no network or LMF involvement), where at least one UE may assume the role of determining relative or absolute positions of participating UEs.

Figure 8 and Figure 9 illustrate two possible example scenarios for SLPP-managed sidelink positioning and ranging.  Figure 8 illustrates a centralized mode of operation where one UE (UE B) performs the computations to determine the range and/or position of the four participating UEs, and Figure 9 illustrates a distributed (or decentralized) mode of operation, where each of the participating UEs perform range/position computations. In both centralized and distributed operation, following discovery and potential SLPP session establishment, the initiating UE (UE B ) may broadcast a request for the position and range capabilities of the participating UEs (SLPP Request Capabilities in Figure 8 and Figure 9).  The addressed UEs can subsequently respond with a their respective capabilities (SLPP Provide Capabilities), where individual UE capabilities may include details of the supported SL-PRS configurations and supported SL-PRS measurements.  Based on the received information for the participating UEs, the initiating UE (UE B) may broadcast positioning assistance data to the participating UEs (SLPP Provide Assistance Data), where examples of positioning assistance data could include a SL-PRS configuration for each of the participating UEs.  After receipt of assistance data, participating UEs may broadcast their respective SL-PRS (according to received SL-PRS configurations from the initiating UE) and perform the requested measurements of SL-PRS received from other UEs. Upon completion of measurements, UEs may broadcast measurement results (SLPP Provide Location Information). In the centralized mode of operation (Figure 8), the participating UEs (apart from the initiating UE B) would broadcast measurement results, which the initiating UE B would use to determine ranges and/or positions for all participating UEs, and potentially then distribute those results to the participating UEs.  In the distributed mode of operation (Figure 9), all UEs may broadcast measurement results, and each participating UE can perform the range and/or position computations.  Such a sequence of capability transfer, assistance data transfer and measurement/location information transfer enables UEs to conduct sidelink positioning transactions in the absence of network coverage.

Observation 9: The three fundamental transaction types from LPP (capability transfer, assistance data transfer, and location information transfer) seem also suitable for the SLPP positioning operation over sidelink.

Proposal 8: Sidelink positioning protocol (SLPP) should support centralized operation, where one UE performs position/range computation based on shared measurement/location information, and distributed operation, where all participating UEs perform position/range computation.

Figure 8: Sidelink-only Positioning – Initiator (centralized) position calculation

Figure 9: Sidelink-only Positioning – Distributed position calculation

## SLPP operation with LMF Support

The basic SLPP transaction types (capability transfer, assistance data transfer, location information transfer) suitable for supporting out-of-coverage sidelink positioning and ranging scenarios may also be applied to in-coverage and partial coverage scenarios, including LMF-assisted (MO-LR) and LMF-initiated (MT-LR) sidelink positioning and ranging.  When at least one participating UE is in network coverage and enabled via a subscription to access a PLMN, the UE may request LMF assistance for a sidelink positioning and ranging session, or may be requested by an LMF to participate in a sidelink positioning and ranging session. In this mode of operation, not all UEs may have PLMN access and be supported by, or provide support to, an LMF ( LMF support may be restricted to just some UEs).  Possible examples of LMF-assisted (MO-LR) and LMF-initiated (MT-LR) are illustrated in Figure 10 and Figure 11, respectively.

Figure 10 shows an example of LMF-assisted sidelink positioning and ranging, where the LMF acts as an adjunct to UE B which is in network coverage and initiating a sidelink positioning and ranging session. After capability transfer between the participating UEs, the initiating UE B may request positioning assistance data from the LMF (assistance data could include SL-PRS configurations) via an SLPP Request Assistance Data. To enable the LMF to determine suitable SL-PRS configurations for all UEs, UE B may also provide the obtained SL-PRS capabilities of all UEs in the group to the LMF in a SLPP Provide Capabilities message. Subsequently, UE measurements and location information transfer may follow the same behavior as for the out-of-coverge operation illustrated in Figure 8 and Figure 9, with UE B performing resultant position/range computations.  Alternatively, UE B may provide the obtained location measurements from all participating UEs to the LMF in an SLPP Provide Location Information message for the LMF to perform and distribute range/position calculations (e.g., via SLPP Provide Location Information message). The UE B initiated SLPP transactions towards the LMF might be part of a location session between UE B and the LMF (e.g. initiated by an MO-LR or a new supplementary services operation).

Observation 10: The three fundamental SLPP transaction types (capability transfer, assistance data transfer, and location information transfer) for positioning and ranging operation over sidelink are also suitable for an LMF-assisted operation mode where an LMF may act as an adjunct to the UE to assist the UE to perform sidelink positioning.

Observation 11: A MO-LR or a new supplementary services operation may be used for UE initiated SLPP transactions towards an LMF.

Figure 10: LMF-assisted Sidelink positioning (MO-LR)

Figure 11 shows an example of LMF-initiated sidelink positioning and ranging operation with a target UE (UE B) in network coverage to obtain MT-LR location results. The MT-LR could have been instigated by an external client or AF (not shown in Figure 11) which would have provided all necessary information for the MT-LR to the LMF (e.g. via a GMLC and AMF). The LMF may first request the sidelink positioning capabilities from the target UE via SLPP Request Capabilities message. The LMF can subsequently request location results from the target UE using a supplementary services operation request, which can indicate the type of location results requested (e.g. location of the target UE and/or of other UEs), identities and/or addresses of specific UEs to be involved (e.g. UEs A, C, D), or whether any UEs can be used, and whether a single set of location results is requested (immediate location), or whether deferred (e.g. periodic or triggered) location results are requested. The supplementary services request might also include an embedded SLPP Request Location Information message indicating specific SLPP location results or measurements to be provided by the target UE and/or an embedded SLPP Provide Assistance Data message to provide assistance data for the SLPP positioning to the target UE (e.g. SL-PRS configurations). The reason for using a supplementary services request in Figure 11 would be to allow inclusion of information such as UE addresses and identities, and use of immediate versus deferred location which might not be suitable for inclusion in an SLPP message. However, it is possible that an SLPP message could be used instead (e.g. an SLPP Request Location Information). The target UE may then confirm or acknowledge the supplementary services request with a supplementary services response, which may indicate whether any requested UEs are available (e.g. whether UEs A, C, D have been discovered by the target UE B). The target UE then performs SLPP positioning as in Figure 8 (or Figure 9) to obtain measurements and location results without further LMF assistance and returns the location results and/or measurements to the LMF. The LMF may then determine a location (or range and/or bearing) for the target UE and possibly for the other UEs and return location(s) to the external client or AF. For deferred (periodic or triggered) location, the SLPP positioning by the target UE and return of location results to the LMF might be repeated.

Observation 12: The three fundamental SLPP transaction types (capability transfer, assistance data transfer, and location information transfer) for positioning and ranging operation over sidelink are also suitable for an LMF initiated sidelink/SLPP MT-LR operation mode where an LMF may initiate and assist the UE to perform sidelink positioning

Observation 13: A MT-LR or a new supplementary services operation may be used for LMF-initiated SLPP transactions towards a UE.

Figure 11: LMF-initiated Sidelink positioning for an MT-LR

Figure 12 shows the same scenario as Figure 11, but where the LMF actively assists with the SLPP positioning of the four UEs as in Figure 10.  For example, the LMF might indicate in the supplementary services request whether no LMF assistance is preferred as in Figure 11 or whether assistance is preferred (or required) as in Figure 12.

Observation 14: An LMF initiated sidelink/SLPP MT-LR can be supported on the target UE side using the same SLPP procedures as applicable to UE initiated sidelink/SLPP positioning and with either no LMF support during the positioning or active LMF support during the positioning.

Figure 12: LMF-initiated Sidelink positioning for an MT-LR with additional LMF Assistance

## SLPP operation with LMF Support for Joint Sidelink and Uu positioning

The basic SLPP transaction types (capability transfer, assistance data transfer, location information transfer) suitable for supporting in-coverage, partial coverage and out-of-coverage sidelink positioning and ranging scenarios may also be applied to support joint sidelink-Uu positioning.  This could simply be achieved by jointly performing the SLPP, LPP and NRPPa procedures for the desired positioning methods as shown in Figure 13.  For Uu-only positioning, LPP and NRPPa are jointly used for UE positioning operation (e.g. for Multi-RTT). For joint Uu and SL positioning, any combination of LPP, SLPP, and NRPPa may be used, dependent on the desired positioning method(s) (UL-, DL, SL positioning).

Figure 13: Joint Uu-Sidelink Positioning

Observation 15: Hybrid Uu and SL positioning could be achieved by jointly using the SLPP, LPP, and NRPPa procedures.

# Summary

Observation 1: To support sidelink positioning and ranging a UE should support a sidelink positioning and ranging protocol (SLPP).

Observation 2: To support sidelink positioning and ranging a UE should be capable of being enabled with the ability to calculate position and or range based on sidelink positioning and ranging measurements.

Observation 3: A UE-based Sidelink Positioning and Ranging Function (SPRF) communicating over the SR5 reference point can be used to provide location and range computation and sidelink positioning and ranging protocol (SLPP) functionality.

Observation 4: Exchange of Sidelink Positioning and Ranging Function (SPRF) messaging can be accomplished without modification to the PC5 reference point over the PC5 user plane.

Observation 5: A sidelink positioning and ranging network architecture supporting SPRF, discovery and SLPP can be realized without requiring enhancements to the PC5 reference point.

Observation 6: UEs participating in sidelink positioning and ranging should be aware of the other participating UEs in order to effectively apply measurements received from the other UEs.

Observation 7: SLPP sessions may be used between 2 or more UEs to support sidelink positioning in a structured and coordinated manner.

Observation 8: SLPP could be used in a session-less mode between 2 or more UEs to support sidelink positioning with reduced signaling overhead and greater adaptivity to changing UE locations and other changing conditions.

Observation 9: The three fundamental transaction types from LPP (capability transfer, assistance data transfer, and location information transfer) seem also suitable for the SLPP positioning operation over sidelink.

Observation 10: The three fundamental SLPP transaction types (capability transfer, assistance data transfer, and location information transfer) for positioning and ranging operation over sidelink are also suitable for an LMF-assisted operation mode where an LMF may act as an adjunct to the UE to assist the UE to perform sidelink positioning.

Observation 11: A MO-LR or a new supplementary services operation may be used for UE initiated SLPP transactions towards an LMF.

Observation 12: The three fundamental SLPP transaction types (capability transfer, assistance data transfer, and location information transfer) for positioning and ranging operation over sidelink are also suitable for an LMF initiated sidelink/SLPP MT-LR operation mode where an LMF may initiate and assist the UE to perform sidelink positioning

Observation 13: A MT-LR or a new supplementary services operation may be used for LMF-initiated SLPP transactions towards a UE.

Observation 14: An LMF initiated sidelink/SLPP MT-LR can be supported on the target UE side using the same SLPP procedures as applicable to UE initiated sidelink/SLPP positioning and with either no LMF support during the positioning or active LMF support during the positioning.

Observation 15: Hybrid Uu and SL positioning could be achieved by jointly using the SLPP, LPP, and NRPPa procedures.

Proposal 1: RAN2 should study a Sidelink Positioning and Ranging Function (SPRF) including positioning protocol functionality and location and range calculation based on sidelink positioning and ranging measurements.

Proposal 2: RAN2 should study a Sidelink Positioning Protocol (SLPP) to support UE-to-UE sidelink positioning and ranging.

Proposal 3: RAN2 should study SPRF SLPP transport over PC5 mechanisms providing flexibility in cast type and minimizing or introducing no change to the PC5 reference point.

Proposal 4: RAN2 should study a sidelink positioning and ranging architecture supporting UE and LMF Sidelink Positioning Reference Function (SPRF).

Proposal 5: RAN2 should study the SPRF SLPP between UEs over the SR5 reference point, considering capability transfer, assistance data transfer and location information transfer.

Proposal 6: RAN2 should study solutions that support SLPP sessions and session-less use of SLPP and evaluate associated benefits, limitations, overheads and impacts.

Proposal 7: Sidelink positioning protocol (SLPP) should enable sidelink positioning and ranging between UEs when no network coverage is present (no network or LMF involvement), where at least one UE may assume to role of determining relative or absolute position of participating UEs.

Proposal 8: Sidelink positioning protocol (SLPP) should support centralized operation, where one UE performs position/range computation based on shared measurement/location information, and distributed operation, where all participating UEs perform position/range computation.

# References

- RP-221814, " Study on expanded and improved NR positioning ", Intel Corporation.

- TR 23.700-86

- TS 38.305

- TS 37.355