3GPP TSG-RAN WG2 Meeting #119-e	R2-2208127

Electronic, August 17 – 26, 2022

Agenda item:	8.2.3

Source: 	Qualcomm Incorporated

Title: 	Integrity of NR Positioning Technologies

Document for:		Discussion and Decision

# 1.	Introduction

At RAN#94e a new Study Item on "expanded and improved NR positioning" was approved which was revised at RAN#96 [1]. This study includes the following objective:

- Study solutions for Integrity for RAT dependent positioning techniques [RAN2, RAN1]: - Identify the error sources, [RAN1, RAN2]. - Study methodologies, procedures, signalling, etc. for determination of positioning integrity for both UE-based and UE-assisted positioning [RAN2] - Focus on reuse of concepts and principles being developed for RAT-Independent GNSS positioning integrity, where possible.

In this contribution, we review the Rel-17 GNSS integrity principles and procedures and study the applicability to the NR Positioning Technologies.

# 2.	Integrity of NR Positioning Technologies

## 2.1	General

The integrity of a location estimate refers to the confidence one can give to correctness of the estimate with respect to the true (but unknown) location. Integrity is generally defined as a measure of trust that can be placed in the correctness of information supplied by a navigation system.

Setting a protection level (PL) is a widely accepted integrity methodology for navigation systems. A PL is commonly defined as the maximum error from the true position that can occur in a specified direction (horizontal, vertical), "guaranteed" to a sufficiently high level of probability. When the PL exceeds a predefined alert limit (AL) for the corresponding application, the positioning result is declared unavailable. Since the position error is not observable, the decision of alert is done by comparing the AL specified and the PL calculated. The protection level is a measure of the integrity that allows the receiver to operate without knowing the Alert Limit (AL).

For a navigation system to fulfil its integrity requirement, it must estimate the maximum size of each error source, such that the errors are either contained within safe bounds, or that the probability is sufficiently high that a timely warning is issued when the combination of these errors exceeds safe bounds.

Integrity monitoring algorithms are well-developed and applied in the GNSS domain. These concepts are the outcome of a particularly detailed understanding of the errors and faults affecting GNSS systems as well as of a thorough mathematical framework to ensure the appropriate level of bounding.

The receivers embedded with integrity monitoring algorithms can continuously perform integrity checks and inform users in the case of unreliable positioning solutions. In the current literature, two approaches for integrity monitoring algorithms exist: snapshot and sequential. The snapshot approach is generally based on the residuals of the least-squares estimator, which uses only the measurements of the current epoch. The sequential approach is usually based on the innovations of the Kalman filter, which uses both current and past measurements and therefore, need to consider the time-correlation of the errors.

Integrity failures of NR Positioning Technologies can be due to erroneous RTDs, ARP locations, TEG margins, Beam-shape information, etc. and obviously depends on the specific NR location technology. The error sources for the NR Positioning Technologies are currently investigated by RAN1 [2] and include (at least) the following [3]:

-	RTDs;

-	TRP locations;

-	TEG margins;

-	Beam-shape information;

-	ExpectedRSTD / expectedAoD/AoA.;

-	Potential new assistance data added in Rel-18, for example, related to carrier phase positioning;

-	Potentially, the rate of time-variation of some or all of the above assistance data elements.

## 2.2	GNSS Integrity Operation and Applicability to NR Positioning

The following is a summary of the main features of GNSS integrity operation, based on or paraphrased from the description in TS 38.805 [4], that is relevant to the discussion on how to extend integrity operation to NR positioning.

When GNSS is designed to inter-work with the NG-RAN, the network assists the UE GNSS receiver to improve the performance in several respects. These performance improvements will: - allow the UE to determine and report the integrity results of the calculated location; the UE can use the integrity requirements and assistance data obtained via NG-RAN, together with its own measurements, to determine the integrity results of the calculated location. ### 8.1.1a Integrity Principle of Operation For integrity operation, the network will ensure that: P(Error > Bound for longer than TTA ¦ NOT DNU) <= Residual Risk + IRallocation for all values of IRallocation in the range irMinimum <= IRallocation <= irMaximum for all the errors which have corresponding integrity assistance data available and where the corresponding DNU flag(s) are set to false. … Where: Error: Error is the difference between the true value of a GNSS parameter (e.g. ionosphere, troposphere etc.), and its value as estimated and provided in the corresponding assistance data. Bound: Integrity Bounds provide the statistical distribution of the residual errors associated with the GNSS positioning corrections (e.g. RTK, SSR etc). Integrity bounds are used to statistically bound the residual errors after the positioning corrections have been applied. The bound formula describes a bounding model including a mean and standard deviation (e.g. paired over-bounding Gaussian). The bound may be scaled by multiplying the standard deviation by a K factor corresponding to an IRallocation, for any desired IRallocation within the permitted range. The bound is computed according to the bound formula Bound = mean + K * stdDev K = normInv(IRallocation / 2) irMinimum <= IRallocation <= irMaximum where mean and stDev are the mean and standard deviation for this specific error. Residual Risk: The residual risk is the component of the integrity risk provided in the assistance data. This may correspond to the fault case risk but the implementation is permitted to allocate this component in any way that satisfies P(Error > Bound for longer than TTA ¦ NOT DNU) <= Residual Risk + IRallocation. irMinimum, irMaximum: Minimum and maximum allowable values of IRallocation that may be chosen by the client. Provided as service parameters from the network according to Integrity Service Parameters. Correlation Times: The minimum time interval beyond which two sets of GNSS assistance data parameters for a given error can be considered to be independent from one another. ##### 8.1.2.1.8 Real-Time Integrity Real-Time Integrity assistance provides the GNSS receiver with information about the health status of a GNSS constellation (where the specific GNSS is indicated by a GNSS ID). ##### 8.1.2.1.29 Integrity Service Parameters Integrity Service Parameters provide the range of Integrity Risk (IR) for which the associated GNSS integrity assistance data is considered to be valid. ##### 8.1.2.1.30 Integrity Alerts Integrity Service Alerts provide information on whether the service can be used for integrity. A Do Not Use (DNU) flag indicates that the corresponding assistance data is not suitable for the purpose of computing integrity. If an Integrity Service Alert is issued and the DNU flag is false, then the corresponding assistance data may be used for the purpose of computing integrity. The DNU flags are defined to be applicable to the specified epoch time only.

Each error of the NR location technologies will be distributed according to some statistical distribution. In general, it is not possible to know the true distribution of the errors. Different errors may also have a different distribution (e.g., RTDs vs. ARP locations). Therefore, the actual distribution of errors is typically replaced by a simpler distribution, which is often a Gaussian distribution. The true distribution can be approximated with a model distribution that is conservative, i.e., it "overbounds" the true distribution and never under-estimates the probability of an error of a given magnitude. A probability density function p "overbounds" a probability density function p, if for any integration interval [-L, L] the integral of p is less than or equal to the respective integral of function p.

-LLpxdx≤ -LLpxdx      ∀ L

To leverage the PL definition, the idealized overbound should be Gaussian. The goal is to find a Gaussian distribution that can be used instead of "true distribution" in the integrity calculations and that will result in an upper bound of the integrity risk. One principle is to inflate the sigma, such that the error distribution (characterized by σ) overbounds all reasonable error distributions out to the probabilities assumed in the computation of the PLs. The inflation generates conservative Gaussian models, called "overbounds".

Since real error distributions tend to have non-zero mean and "fat tails" that would require a very conservative value of σ to overbound the distribution (resulting in over-inflated Protection Levels), a pair of bounds can be used, rather than a single bound, to represent the actual error distribution. The true distribution is bounded by two non-zero mean Gaussians, shifted by -𝜇 and +𝜇. This allows for a tighter bound that is still an overbound across the whole distribution.

In effect, distribution anomalies are converted into biases.

Based on the above discussion, we make the following observations:

Observation 1:	The Rel-17 GNSS integrity principle of operation does not require "hard-coding" of parameters or specifying user algorithms. It enables integrity monitoring for snapshot and sequential user algorithms.

Observation 2:	Rel-17 GNSS integrity feature allows for the following:

a)	An LMF can specify target integrity risk for which the protection level is requested, and a UE can report integrity result for the location estimate, expressed as protection level together with achieved target integrity risk.

b)	LMF can indicate bounds on the assistance data, such as mean and variance of various correction data such as clock errors, troposphere and ionosphere errors.

Observation 3: 	The Rel-17 GNSS integrity concept is rather generic and can be applied to the NR positioning technologies as well.

Observation 4:	Applying the Rel-17 GNSS integrity concept to NR positioning technologies requires a detailed understanding of the errors and faults affecting the NR positioning technologies. However, this depends on RAN1 outcome.

Observation 5:	A paired Gaussian overbounding model for the errors can also be used for the NR positioning technologies.

## 2.3	Integrity Assistance Data

The GNSS integrity assistance data comprise the following.

-	Integrity Alerts ("Do Not use" flags).

- 	Integrity Bounds. Mean and standard deviation of the errors and error rates.

-	Residual Risks. Probabilities of onset of a particular fault.

-	Integrity Correlation Times. Error and error rate correlation times.

The above assistance data can be provided in the individual SSR assistance data elements [5].

The assistance data for the NR positioning technologies currently comprise the following [5].

-	NR-DL-PRS-AssistanceData

-	NR-TRP-LocationInfo

-	NR-DL-PRS-BeamInfo

-	NR-RTD-Info

-	NR-TRP-BeamAntennaInfo

-	NR-DL-PRS-ExpectedLOS-NLOS-Assistance

-	NR-DL-PRS-TRP-TEG-Info

Each of the above assistance data element includes information affecting the integrity of the NR positioning system. For example:

-	NR-DL-PRS-AssistanceData: DL-PRS parameter matching the actual DL-PRS over the air. Expected RSTD / expected AoD/AoA.

-	NR-TRP-LocationInfo: ARP location for each DL-PRS resource compared to the true ARP location.

-	NR-DL-PRS-BeamInfo: Spatial direction information of the DL-PRS Resources compared to the true direction.

-	NR-RTD-Info: RTD of the TRPs compared to the true RTD for each DL-PRS Resource.

-	NR-TRP-BeamAntennaInfo: Beam shape information of the TRP compared to the true beam shape information for each DL-PRS Resource.

-	NR-DL-PRS-ExpectedLOS-NLOS-Assistance: Expected likelihood of a LOS propagation path compared to the true LOS/NLOS.

-	NR-DL-PRS-TRP-TEG-Info: Provided association information of DL-PRS Resources with TRP Tx TEGs compared to the true association.

For GNSS, the integrity bounds provide the statistical distribution of the residual errors associated with the GNSS positioning corrections (e.g. RTK, SSR etc). The integrity bounds are used to statistically bound the residual errors after the positioning corrections have been applied. For the NR positioning technologies, the assistance data do not provide "corrections" but rather the (assumed) "true" value of a specific parameter (e.g., RTD, ARP locations, etc.). Therefore, the bound definition must be slightly modified for the NR positioning technologies:

Bound: Integrity Bounds provide the statistical distribution of the errors associated with the assistance data. The bound is computed according to the bound formula defined in equation (A). The bound formula describes a bounding model including a mean and standard deviation (e.g. paired over-bounding Gaussian). The bound may be scaled by multiplying the standard deviation by a K factor corresponding to an IRallocation, for any desired IRallocation within the permitted range. The bound for a particular error is computed according to the following formula: Bound = mean + K × stdDev (Equation A) K = normInv(IRallocation / 2) irMinimum <= IRallocation <= irMaximum where: mean: mean value for this specific error. stdDev: standard deviation for this specific error.

Similar to A-GNSS, the Integrity Bounds, Residual Risks, and Integrity Correlation Times can be included in the above NR assistance data. For the Integrity Alerts ("Do Not use" flags), an additional assistance data element seems required.

The UE may use the integrity requirements for the specific application and the integrity assistance data to compute a PL for the current navigation solution similar to GNSS. For example:

| PL=KmdσPE+B |  |
| --- | --- |

Kmd=Q-1IR2

Q=12πu∞e-t2/2dt

where σPE is the user position error standard deviation at receiver level and B is a bias term related to all possible biases involved into the measurements. Single measurements variances can be represented as in the following:

| σi2=σA,i2+σB,i2+σC,i2+… |  |
| --- | --- |

where σx,i2 is the variance of the specific error (which depends on the NR positioning method) for TRP i.

For the validity time of the A-GNSS integrity bounds, the validity time of the SSR assistance data is reused. Specifically, the validity time of the integrity bounds is set as equal to twice the SSR update interval for the given SSR Assistance Data message. For the NR assistance data, a validity time or update interval is currently not defined, therefore, the validity period of the integrity bounds could be included in the NR assistance data. Some NR assistance data are rather long-term valid (e.g., TRP/ARP locations per resource) and some may change more frequently (e.g., RTDs, but also DL-PRS parameter (e.g,, in case of on-demand DL-PRS)). If a long-term valid assistance data element with integrity changes, a DNU message needs to be issued. The assistance data (e.g., TRP/ARP locations) may then still be used for positioning, but not for integrity related applications. Therefore, for integrity related applications, also the "integrity alerts" for the NR positioning technologies need to be provided periodically with certain intervals (e.g., related to the TTA). For this, posSIBs seem most appropriate.

The UE may also be provided with a target integrity risk for the specific application in a LPP location request message for which a PL should be determined. The UE would then determine the PL for the provided target integrity risk and report the PL along with the location estimate to the LMF. This functionality is common positioning in LPP Rel-17 [5] (CommonIEsRequestLocationInformation, CommonIEsProvideLocationInformation) and can also be used for NR positioning.

Observation 6:	Similar to A-GNSS, the integrity bounds, residual risks, integrity correlation times for the errors of each NR positioning technology could be added to the existing NR assistance data.

Observation 7:	The validity time of the integrity bounds could be added to the NR integrity assistance data.

Observation 8:	The integrity alerts ("Do Not use" flags) require new assistance data elements.

Observation 9:	A new Assistance Data Element is required to provide the minimum and maximum allowable values of IRallocation that may be chosen by the client.

Observation 10:	The Common Positioning Information (targetIntegrityRisk in CommonIEsRequestLocationInformation and IntegrityInfo in CommonIEsProvideLocationInformation) can also be applicable to the NR positioning technologies.

# 3.	Summary

In this contribution, we reviewed the Rel-17 GNSS integrity principles and procedures and studied the applicability to the NR Positioning Technologies. The following observations were made.

Observation 1:	The Rel-17 GNSS integrity principle of operation does not require "hard-coding" of parameters or specifying user algorithms. It enables integrity monitoring for snapshot and sequential user algorithms.

Observation 2:	Rel-17 GNSS integrity feature allows for the following:

a)	An LMF can specify target integrity risk for which the protection level is requested, and a UE can report integrity result for the location estimate, expressed as protection level together with achieved target integrity risk.

b)	LMF can indicate bounds on the assistance data, such as mean and variance of various correction data such as clock errors, troposphere and ionosphere errors.

Observation 3: 	The Rel-17 GNSS integrity concept is rather generic and can be applied to the NR positioning technologies as well.

Observation 4:	Applying the Rel-17 GNSS integrity concept to NR positioning technologies requires a detailed understanding of the errors and faults affecting the NR positioning technologies. However, this depends on RAN1 outcome.

Observation 5:	A paired Gaussian overbounding model for the errors can also be used for the NR positioning technologies.

Observation 6:	Similar to A-GNSS, the integrity bounds, residual risks, integrity correlation times for the errors of each NR positioning technology could be added to the existing NR assistance data.

Observation 7:	The validity time of the integrity bounds could be added to the NR integrity assistance data.

Observation 8:	The integrity alerts ("Do Not use" flags) require new assistance data elements.

Observation 9:	A new Assistance Data Element is required to provide the minimum and maximum allowable values of IRallocation that may be chosen by the client.

Observation 10:	The Common Positioning Information (targetIntegrityRisk in CommonIEsRequestLocationInformation and IntegrityInfo in CommonIEsProvideLocationInformation) can also be applicable to the NR positioning technologies.

Therefore, the following proposal is made.

Proposal 1:	For studying the integrity of NR Positioning Technologies, reuse the Rel-17 definitions and principle of operation of GNSS integrity.

The Observations and Proposal 1 are implemented in the Text Proposal shown in Annex A.

Proposal 2:	Endorse the Text Proposal for TR 38.859 in the Annex of this contribution.

# References

[1]	RP-221814, "Revised SID on Study on expanded and improved NR positioning", Intel Corporation.

[2]	RP-221591, "Status Report to TSG".

[3]	R1-2205039, "Integrity for RAT dependent positioning", Qualcomm Incorporated.

[4]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[5]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

# Annex: Text Proposal for TR 38.859

## 6.1	Integrity for RAT-Dependent Positioning Techniques

### 6.1.1	Identification of error sources

### 6.1.2	Methodologies, procedures and signalling for determination of positioning integrity

#### 6.1.2.1	Integrity Principle of Operation

For integrity operation, the network will ensure that:

P(Error > Bound for longer than TTA | NOT DNU) <= Residual Risk + IRallocation               (Equation 6.1.2.1-1)

for all values of IRallocation in the range irMinimum <= IRallocation <= irMaximum

for all the errors in Table 6.1.4-1, which have corresponding integrity assistance data available and where the corresponding DNU flag(s) are set to false.

The integrity risk probability is decomposed into a constant Residual Risk component provided in the assistance data as well as a variable IRallocation component that corresponds to the contribution from the Bound according to the Bound formula in Equation 6.1.2.1-2. IRallocation may be chosen freely by the client based on the desired Bound, therefore, the network should ensure that Equation  6.1.2.1-1 holds for all possible choices of IRallocation. The Residual Risk and IRallocation components may be mapped to fault and fault-free cases respectively, but the implementation is free to choose any other decomposition of the integrity risk probability into these two components.

Equation  6.1.2.1-1 holds for all assistance data that has been provided that is still within its validity period. If this condition cannot be met then the corresponding DNU flag must be set.

Equation  6.1.2.1-1 holds at any epochs for which Assistance Data is provided. Providing Assistance Data without the Integrity Alerts is interpreted as a DNU=FALSE condition. For any bound that is still valid (within its validity time), the network ensures that the Integrity Alert IEs are also included in the provided Assistance Data if needed to satisfy the condition in Equation  6.1.2.1-1. It is up to the implementation how to handle epochs for which integrity results are desired but there are no DNU flag(s) available, e.g. the Time To Alert (TTA) may be set such that there is a "grace period" to receive the next set of DNU flags.

Only those TRPs for which the integrity assistance data are provided are monitored by the network and can be used for integrity related applications.

Where:

Error: Error is the difference between the true value of a parameter (e.g. ARP location, RTD, etc.), and its value as provided in the corresponding assistance data as per Table 6.1.4-1.

Bound: Integrity Bounds provide the statistical distribution of the errors. The bound is computed according to the Bound formula defined in Equation  6.1.2.1-2. The bound formula describes a bounding model including a mean and standard deviation (e.g. paired over-bounding Gaussian). The bound may be scaled by multiplying the standard deviation by a K factor corresponding to an IRallocation, for any desired IRallocation within the permitted range.

The bound for a particular error is computed according to the following formula:

Bound = mean + K * stdDev																	(Equation  6.1.2.1-2)

K = normInv(IRallocation / 2)

irMinimum <= IRallocation <= irMaximum

where:	mean: mean value for this specific error, as per Table 6.1.4-1

stdDev: standard deviation for this specific error, as per Table 6.1.4-1

Time-to-Alert (TTA): The maximum allowable elapsed time from when the Error exceeds the Bound until a DNU flag must be issued.

DNU: The DNU flag(s) corresponding to a particular error as per Table 6.1.4-1. Where multiple DNU flags are specified, the DNU condition in Equation  6.1.2.1-1 is present when any of the flags are true (logical OR of the flags).

Residual Risk: The residual risk is the component of the integrity risk provided in the assistance data as per  6.1.2.1-1. This may correspond to the fault case risk, but the implementation is permitted to allocate this component in any way that satisfies Equation  6.1.2.1-1.

The Residual Risk is the Probability of Onset which is defined per unit of time and represents the probability that the feared event begins. Each Residual Risk is accompanied by a Mean Duration which represents the expected mean duration of the corresponding feared event and is used to convert the Probability of Onset to a probability that the feared event is present at any given time, i.e.

P(Feared Event is Present) = Mean Duration * Probability of Onset of Feared Event		(Equation  6.1.2.1-3)

irMinimum, irMaximum: Minimum and maximum allowable values of IRallocation that may be chosen by the client. Provided as service parameters from the Network according to Integrity Service Parameters.

Correlation Times: The minimum time interval beyond which two sets of GNSS assistance data parameters for a given error can be considered to be independent from one another.

### 6.1.3	Summary of Evaluation Results for Integrity for RAT-Dependent Positioning Techniques

### 6.1.4	Potential Specification Impact for Integrity for RAT-Dependent Positioning Techniques

The potential specification impact for the integrity of NR Positioning Technologies comprises the following.

-	Specification of a new Alert Assistance Data element ("DNU") for each error source in Table 6.1.4-1.

-	Specification of a new Integrity Service Assistance Data Element to provide the minimum and maximum allowable values of IRallocation that may be chosen by the client.

-	Specification of the integrity bounds (mean and standard deviation of errors and error rates) for each error source in Table 6.1.4-1.

-	Specification of the residual risk component for each error source in Table 6.1.4-1.

-	Specification of the minimum time interval beyond which two sets of NR assistance data parameters for a given error can be considered to be independent from one another (Integrity Correlation Times).

-	Specification of the validity time of the integrity assistance data.

Table 6.1.4-1 shows the mapping between the integrity fields and the NR assistance data.

Table 6.1.4-1: Mapping of Integrity Parameters

| Error | NR Assistance Data | Integrity Fields |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Integrity Alerts | Integrity Bounds (Mean) | Integrity Bounds (StdDev) | Residual Risks | Integrity Correlation Times |
| FFS | DL-PRS-Assistance Data | FFS | FFS | FFS | FFS | FFS |
|  | TRP-Location Info |  |  |  |  |  |
|  | DL-PRS Beam Info |  |  |  |  |  |
|  | RTD Info |  |  |  |  |  |
|  | TRP Beam Antenna Info |  |  |  |  |  |
|  | DL-PRS Expected LOS/NLOS Assistance |  |  |  |  |  |
|  | DL-PRS TRP TEG Info |  |  |  |  |  |