3GPP TSG-RAN WG2 #119e	Tdoc R2-2208139

# Electronic, 17 - 26 Aug, 2022

Agenda Item:	5.1.3.1.1

Source:	Ericsson

Title:	NS_55/57 in NR CA

Document for:	Discussion, Decision

# 1	Introduction

In this document, we discuss how to resolve the NS value issues for UL CA that have been identified for the n77 band extensions in the US and Canada.

In this document, we use the case of NS_55 in the US, but the discussion and proposal are also applicable to n77/NS_57 in Canada.

# 2	Discussion

The solution to the extended use of n77 sub-bands in the USA included introduction of:

- A new UE capability (extendedBand-n77-r16) indicated by UE that support both the original n77 sub-range (3700–3980 MHz, C-band) and the added range (3450-3550 MHz, DoD-band) in the USA.

- A new NS value (NS_55), indicated in system information in DoD-band cells to prevent idle mode UE that do not indicate support of the extendedBand-n77-r16 to camp and access a DoD-band cell. NS_55 defines no actual A-MPR requirements, that is, equivalent as for NS_01.

For NR CA, the field description of the field additionalSpectrumEmission states the following:

FrequencyInfoUL field descriptions

absoluteFrequencyPointA Absolute frequency of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A. Note that the lower edge of the actual carrier is not defined by this field but rather in the scs-SpecificCarrierList (see TS 38.211 [16], clause 4.4.4.2).

additionalSpectrumEmission The additional spectrum emission requirements to be applied by the UE on this uplink. If the field is absent, the UE uses value 0 for the additionalSpectrumEmission (see TS 38.101-1 [15], table 6.2.3.1-1A, and TS 38.101-2 [39], table 6.2.3.1-2). Network configures the same value in additionalSpectrumEmission for all uplink carrier(s) of the same band with UL configured. The additionalSpectrumEmission is applicable for all uplink carriers of the same band with UL configured.

<cut>

Furthermore, gNB is expected to signal the same values in dedicated signalling to UE (ServingCellConfigCommon) as is signalled in SIB1 (ServingCellConfigCommonSIB):

#### –	ServingCellConfigCommon

The IE ServingCellConfigCommon is used to configure cell specific parameters of a UE's serving cell. The IE contains parameters which a UE would typically acquire from SSB, MIB or SIBs when accessing the cell from IDLE. With this IE, the network provides this information in dedicated signalling when configuring a UE with a SCells or with an additional cell group (SCG). It also provides it for SpCells (MCG and SCG) upon reconfiguration with sync.

Obviously, since a DoD-cell would indicate NS_55 in SIB1 and a C-band cell would indicate NS_01, requirements for UL CA in TS38.331 are violated.

In UL CA with one serving cell in C-band and one serving cell in DoD-band violates TS 38.331.

A possible solution is to introduce exception for the rules for UL CA in TS 38.331. This has in earlier RAN2 discussions also been favored by companies. There are two obvious options:

- Allow the nw to indicate different NS values for the UL carriers (NS_01 and NS_55)

- Mandate the network to indicate the same value (NS_01) for all UL carriers (despite NS_55 is indicated by DoD-band Cell in SIB1).

Our preference is to introduce option 1, as this allows for the gNb  to set the NS value for each UL carrier (additionalSpectrumEmission)  in the same way as for UL CA in other bands (i.e., to the same value as in SIB1 for each UL carrier). One scenario which we believe requires special consideration is the scenario where the PCell is in the DoD-band and hence the network broadcasts NS_55 for the cell, and the SCell is in the C-band and hence the NW broadcasts NS_01. In this scenario it may not be possible for the gNB to avoid having different NS-values in the cells. But we can also understand that other companies may prefer option 2.

We assume that for UE implementations, the solution to select is not so critical, since this is anyway an exception from legacy behaviour that need to be supported by n77 UEs, but UE vendors need to confirm this.

As a way forward, we propose that for the DoD-band cell, the network may indicate either NS_01 or NS_55 (up to nw impl), but indicate NS_01 for the C-band cell.

For UL CA in n77 with at least one cell in DoD-band and at least one cell in C-band, the network may configure either NS_55 or NS_01 for UL carrier(s) in DoD-band, and NS_01 for the remaining uplink carrier(s) in this band.

A draft Rel-17 CR that covers both the US and Canada cases for band n77 extension.

Agree to the text proposal to TS 38.331 in Annex.

# Conclusion

In the previous sections we made the following observations:

Observation 1	In UL CA with one serving cell in C-band and one serving cell in DoD-band violates TS 38.331.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For UL CA in n77 with at least one cell in DoD-band and at least one cell in C-band, the network may configure either NS_55 or NS_01 for UL carrier(s) in DoD-band, and NS_01 for the remaining uplink carrier(s) in this band.

Proposal 2	Agree to the text proposal to TS 38.331 in Annex.

# Annex A, Draft TP to TS 38.331

### 6.3.2	Radio resource control information elements

#### <First change>

#### –	FrequencyInfoUL

The IE FrequencyInfoUL provides basic parameters of an uplink carrier and transmission thereon.

FrequencyInfoUL information element

-- ASN1START

-- TAG-FREQUENCYINFOUL-START

FrequencyInfoUL ::=                 SEQUENCE {

frequencyBandList                   MultiFrequencyBandListNR                                OPTIONAL,   -- Cond FDD-OrSUL

absoluteFrequencyPointA             ARFCN-ValueNR                                           OPTIONAL,   -- Cond FDD-OrSUL

scs-SpecificCarrierList             SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

additionalSpectrumEmission          AdditionalSpectrumEmission                              OPTIONAL,   -- Need S

p-Max                               P-Max                                                   OPTIONAL,   -- Need S

frequencyShift7p5khz                ENUMERATED {true}                                       OPTIONAL,   -- Cond FDD-TDD-OrSUL-Optional

...

}

-- TAG-FREQUENCYINFOUL-STOP

-- ASN1STOP

FrequencyInfoUL field descriptions

absoluteFrequencyPointA Absolute frequency of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A. Note that the lower edge of the actual carrier is not defined by this field but rather in the scs-SpecificCarrierList (see TS 38.211 [16], clause 4.4.4.2).

additionalSpectrumEmission The additional spectrum emission requirements to be applied by the UE on this uplink. If the field is absent, the UE uses value 0 for the additionalSpectrumEmission (see TS 38.101-1 [15], table 6.2.3.1-1A, and TS 38.101-2 [39], table 6.2.3.1-2). Network configures the same value in additionalSpectrumEmission for all uplink carrier(s) of the same band with UL configured, except for NS_55/NS_57. If NS_55/NS_57 (see TS 38.101-1 [15], table 6.2.3.1-1) is applicable for at least one uplink carrier, the network may configure either NS_55/NS_57 or NS_01 for this uplink carrier, and NS_01 for the remaining uplink carrier(s) of the same band. The additionalSpectrumEmission is applicable for all uplink carriers of the same band with UL configured.

frequencyBandList List containing only one frequency band to which this carrier(s) belongs. Multiple values are not supported.

frequencyShift7p5khz Enable the NR UL transmission with a 7.5 kHz shift to the LTE raster. If the field is absent, the frequency shift is disabled.

p-Max Maximum transmit power allowed in this serving cell. The maximum transmit power that the UE may use on this serving cell may be additionally limited by p-NR-FR1 (configured for the cell group) and by p-UE-FR1 (configured total for all serving cells operating on FR1). If absent, the UE applies the maximum power according to TS 38.101-1 [15] in case of an FR1 cell or TS 38.101-2 [39] in case of an FR2 cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39]. Value in dBm. This field is ignored by IAB-MT, the IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [63].

scs-SpecificCarrierList A set of carriers for different subcarrier spacings (numerologies). Defined in relation to Point A. The network configures a scs-SpecificCarrier at least for each numerology (SCS) that is used e.g. in a BWP (see TS 38.211 [16], clause 5.3).

| Conditional Presence | Explanation |
| --- | --- |
| FDD-OrSUL | The field is mandatory present if this FrequencyInfoUL is for the paired UL for a DL (defined in a FrequencyInfoDL) or if this FrequencyInfoUL is for a supplementary uplink (SUL). It is absent, Need R, otherwise (if this FrequencyInfoUL is for an unpaired UL (TDD). |
| FDD-TDD-OrSUL-Optional | The field is optionally present, Need R, if this FrequencyInfoUL is for the paired UL for a DL (defined in a FrequencyInfoDL), or if this FrequencyInfoUL is for an unpaired UL (TDD) in certain bands (as defined in clause 5.4.2.1 of TS 38.101-1 and in clause 5.4.2.1 of TS 38.104 [12]), or if this FrequencyInfoUL is for a supplementary uplink (SUL). It is absent, Need R, otherwise. |

#### <End of change