3GPP TSG-RAN WG2 Meeting #119-e		R2-2208159

Electronic, 17th August – 29th August 2022

Source:	Qualcomm Incorporated

Title:	Miscellaneous SON MDT enhancements

Agenda Item:	8.13.3

Document for:	Discussion and Decision

# Introduction & Background

Rel-18, SON/MDT WID [1] has the following as one of the objectives,

Support of SON/MDT enhancements for [RAN3, RAN2]:

- MR-DC CPAC

- Successful PSCell change report

- Successful Handover Report (e.g., inter-RAT)

- NPN

- RACH report

- fast MCG recovery

- NR-U (MRO and UL MLB)

This paper discusses SON/MDT enhancements for the aforementioned use cases.

# Discussion

- SON/MDT enhancements for MR-DC CPAC

Rel-17 SON/MDT WI developed the solution for MRO for NR-DC SCG failures [2]. In Rel-17, SON/MDT WI considered primarily two SCG failure scenarios as follows:

- SCG RLF, and

- Legacy PSCell change or addition failure.

Observation 1: In Rel-17, SCGFailureInformation was enhanced for reporting relevant MRO parameters for SCG failures.

In Rel-17, MR-DC WI further developed conditional PSCell addition or change (CPAC) solutions for improving the success rate of PSCell change or addition. Similar to the conditional handover, in the CPAC configuration, the network can configure multiple (up to 8) PSCells for PSCell addition or change having different execution conditions. UE upon CPAC configuration reception starts evaluating the configured execution conditions. If the configured execution conditions for a PSCell meet, UE performs the PSCell change or addition procedure at that selected PSCell. Note that the CPAC procedure may fail if is performed too late, or too early, or  UE selects the wrong PSCell to perform CPA or CPC procedure. Upon the failure of the CPA or CPC procedure, UE sends SCGFailureInformation to the MN indicating the failure of the CPA or CPC failure while providing relevant information.

Observation 2: Upon CPA and CPC failure, SCGFailureInformation is used to report CPA or CPC failures.

In Rel-17, Legacy PSCell change failure scenarios were captured in TS 37.340 [3], as the following,

-	Too late PSCell change: an SCG failure occurs after the UE has stayed for a long period of time in the PSCell; a suitable different PSCell is found based on the measurements reported from the UE.

-	Too early PSCell change: an SCG failure occurs shortly after a successful PSCell change from a source PSCell to a target PSCell or a PSCell change failure occurs during the PSCell change procedure; source PSCell is still the suitable PSCell based on the measurements reported from the UE.

-	Triggering PSCell change to the wrong PSCell: an SCG failure occurs shortly after a successful PSCell change from a source PSCell to a target PSCell or a PSCell change failure occurs during the PSCell change procedure; a suitable PSCell different from the source PSCell or target PSCell is found based on the measurements reported from the UE.

For CPA and CPC, the aforementioned failure scenarios can be further extended where RAN2 can study the optimization of CPAC configurations in addition to the optimized selection of target cells. The following scenario can be considered,

-	Too late CPC: a UE receives CPC configuration, but SCG failure is detected before the execution of the CPC command.

-	Too early CPC: an SCG failure occurs shortly after a successful CPC procedure, or a CPC failure occurs during the CPC procedure; source PSCell is still the suitable PSCell based on the measurements reported from the UE.

-	Wrong PSCell CPC or CPA: an SCG failure occurs shortly after a successful CPA or CPC procedure, or a CPA or CPC failure occurs during the CPA or CPC procedure; a suitable PSCell different from the source PSCell or target PSCell is found based on the measurements reported from the UE.

Note that as there is a previous PSCell for CPA procedure, therefore only wrong PSCell CPA is a relevant failure scenario for CPA. SCGFailureInformation can be enhanced to capture relevant information for the above CPA and CPC failure scenarios.

Proposal 1: RAN2 should consider enhancements of SCGFailureInformation for the following CPA and CPC failure scenarios,

- NO CPC execution and SCG RLF

- Failed CPC or CPA executions

- Successful CPC or CPA execution but RLF at target PSCell immediately after Successful CPC or CPA execution

As previously discussed in rel-17, SCGFailureInformation is a mandatory report. Therefore, we should limit extending the size of SCGFailureInformation by only including the information not available on the network. We can take rel-17 discussions on CHO as the baseline, and SCGFailureInformation can be enhanced to include relevant information.

Proposal 2: Minimize the impact on SCGFailureInformation size by only including the relevant information for CPAC failure not available or retrieved at the network.

Proposal 3: Include the following information in the SCGFailure for CPAC optimization:

- Indicate which configured execution event has been met if multiple execution conditions are configured at the UE

- The time gap between the reception of CPA or CPC configuration until execution or SCGFailure

- The time gap between configured execution conditions if multiple execution conditions are configured at the UE and CPA or CPC execution is performed

Note that there may have coexisting MR-DC failure scenarios such as legacy HOF, CHO failure, or PCell RLF with CPAC failures. However, RAN2 can deprioritize studying co-existing scenarios until RAN2 makes initial agreements on standalone CPAC failure scenarios.

Proposal 4: RAN2 should deprioritize studying coexisting failure scenarios such as legacy HOF, CHO failure, or PCell RLF with CPAC failures, at least until RAN2 makes initial agreements on standalone CPAC failure scenarios.

- SON/MDT enhancements for successful PSCell change (SPC) report

- SPC report configuration

In Rel-17, the successful handover report (SHR) was introduced for reporting any lower layer issue during a successful handover procedure [4]. For the recording of SHR, several trigger conditions were defined, for example, the T310 threshold for PCell, T312 threshold for PCell, T304 threshold for PCell handover, and source RLF for DAPS HO. RAN2 should consider similar trigger conditions generating the SPC report, where the corresponding T310/T312/T304 threshold can be defined for PSCell.

Proposal 5: Define SCG T310, T312, and T304 thresholds for generating the SPC report.

Note that the status of the PCell link can impact the PSCell change or addition procedure, i.e., even without no issue at the PSCell, the PSCell change or addition may still fail due to MCG RLF. Therefore, additionally, MCG T310 and T312 can be defined as the trigger condition for generating an SPC report.

Observation 3: PSCell change or addition procedure can fail due to the MCG RLF.

Proposal 6: Define MCG T310 and T312 thresholds for generating the SPC report.

- SPC reporting considerations

In Rel-17, the successful handover report was introduced for reporting the relevant parameters during a successful handover procedure when triggered conditions meet. For the reporting of the SPC report, RAN2 can have the following options:

- Nested PSCell change report within SHR: The latest successful PSCell change report is nested within the SHR report.

- Independent PSCell change report: An independent successful PSCell change report can be introduced to report lower layer issues during the successful PSCell change or addition.

Note that while the nested PSCell change report provides reporting of PCell SHR and PSCell SPC together, it is also less flexible where the network may not request SHR and SPC independently. Therefore, RAN2 should study the benefit and consequences of these reporting options.

Proposal 7: For the reporting of the SPC report, RAN2 can have the following options:

- Nested PSCell change report within SHR: The latest successful PSCell change report is nested within the SHR report.

- Independent PSCell change report: An independent successful PSCell change report can be introduced to report lower layer issues during the successful PSCell change or addition.

Note that for SHR in Rel-17, a UE variable was introduced to store relevant measurements and information for the successful handover report. UE variable is released if the report is not retrieved by the network within 48 hours. Note that for MN-initiated CPC and CPA procedures, the execution conditions, measurement gaps, and others are determined by the MN. In another scenario, e.g., SN-initiated CPC or CPA, the execution conditions, measurement gaps, and others are determined by the source SN, where the scope. Therefore, for the SPC report, we can have the following two options for clearing the successful PSCell change report,

- Clear after 48 hours if not retrieved by network: UE generates the PSCell change report when configured trigger condition meets. Send the availability indicator in RRCReconfigurationComplete (containing RRCReconfigurationWithSYNC for SCG), and other RRC complete messages. Clear after 48 hours if not retrieved by the network.

- Clear upon PCell change: UE generates the PSCell change report when configured trigger condition meets. Send the availability indicator in RRCReconfigurationComplete (containing RRCReconfigurationWithSYNC for SCG) messages. Clear after UE changes PCell.

Proposal 8: RAN2 should consider the following two options for clearing the successful PSCell change report,

- Clear after 48 hours if not retrieved by network: UE generates the PSCell change report when configured trigger condition meets. Send the availability indicator in RRCReconfigurationComplete (containing RRCReconfigurationWithSYNC for SCG), and other RRC complete messages. Clear after 48 hours if not retrieved by the network.

- Clear upon PCell change: UE generates the PSCell change report when configured trigger condition meets. Send the availability indicator in RRCReconfigurationComplete (containing RRCReconfigurationWithSYNC for SCG) messages. Clear after UE changes PCell.

- SON/MDT enhancements for successful handover report

In rel-17, SHR is considered only for intra-NR handovers. SHR for intra-LTE and inter-RAT handovers was not supported in Rel-17. Rel-18 SON/MDT WID considers supporting enhancements for SHR (e.g., inter-RAT). The following scenarios can be considered for enhancements in Rel-18.

| Handover scenarios | Reporting consideration |
| --- | --- |
| Intra-LTE handover | LTE SHR is generated - Reports SHR to LTE cell, no need for cross-RAT reporting |
| Intra-system inter-RAT handover (NR to LTE) | UE generates NR SHR report - Reports SHR when UE comeback to NR, similar to RLF report (no need for cross RAT reporting) |
| Intra-system inter-RAT SHR (LTE to NR) | UE generates LTE SHR - Reports SHR when UE comeback to LTE (no need for cross RAT reporting) |
| Inter-system inter-RAT SHR (LTE to NR) | UE generated LTE SHR - Reports SHR when UE comeback to LTE (No need for cross RAT reporting) |
| Inter-system inter-RAT SHR (NR to LTE) | UE generated NR SHR - Reports SHR when UE comeback to NR, similar to RLF report (No need for cross RAT reporting) |

Note that successful handover reports were mainly introduced in NR to optimize RLM/BFD configurations by knowing whether a threshold of the configured RLM/BFD timers (e.g., T310 or T312) has been crossed at the UE during successful handovers. LTE does not intrinsically support a beam-based structure, there is little benefit in trying to catch near-failures during successful handovers. Also, there is little motivation to further optimize handovers in LTE as MRO mechanisms have been there since Rel-9 for LTE. Moreover, we are now moving towards more NR SA deployments.

Observation 4: There is little benefit and motivation to optimize RLM configurations and successful handovers in LTE due to the absence of beam-based structure in LTE and the industry’s move towards more NR SA deployments.

According to Rel-17 SHR [2], SHR is stored at the UE for 48 hours or until retrieval. Therefore, there is no significant requirement or benefit in cross-RAT retrieval of SHR as the SHR can always be reported once UE is back on the same RAT.

Observation 5:  There is no significant requirement or benefit in cross-RAT retrieval of SHR as the SHR can always be reported once UE is back on the same RAT.

Proposal 9: Cross-RAT SHR reporting is not needed for inter-RAT handover scenarios.

- SON/MDT enhacements for NPN

- MDT for the non-public network (NPN)

3GPP in Rel-16 introduced support for Non-Public Networks (NPN), including SNPN and PNI-NPN. To manage an SNPN, the standalone SNPN management system needs a dedicated NPN identifier. The combination of a PLMN ID and Network Identifier (NID) is used to uniquely identify an SNPN.

MDT has been so far only supported for PLMNs. Recently SA5 has started looking into how to support trace-related data collection from UE in Non-Public Networks via Rel-18 WI OAM_NPN. SA5 TR in 28.557 outlines some high-level requirements and procedures for creating MDT collection tasks in NPNs and collecting MDT reports from NPNs.

NPN Service Customer (NPN-SC): a Communication Service Customer (CSC) which consumes communication services for non-public use, i.e., communication services offered over NPNs. An NPN-SC is the realization of the CSC role in NPN environments.

NPN Service Provider (NPN-SP): a Communication Service Provider (CSP) which provides communication services for non-public use, i.e., communication services offered over NPNs. An NPN-SP is the realization of the CSP role in NPN environments.

NPN Operator (NPN-OP): a Network Operator (NOP) whose management scope is limited to 5G networks for non-public use, i.e., NPNs. An NPN operator is the realization of the NOP role in NPN environments.

The following are steps for the NPN MDT procedure [5],

- Based on the pre-defined agreements, NPN-SC sends a "Create MDT collection task" request to NPN-SP/OP.

- The NPN-SP/OP sends a Trace Session activation request to the NE. This request includes the parameters for configuring MDT data collection such as area, job type, and list of measurements.

- After receiving the MDT collection request, NE performs the UE selection based on the input information derived from NPN-SP/OP, such as device capability information and area scope.

- NE shall activate the MDT functionality and send configuration information to the selected UEs.

- When UE receives the MDT activation, it shall start the MDT functionality based on the received configuration parameters. The MDT-related measurements are then reported to NE.

- Then NE reports the related data to NPN-SP/OP

- According to pre-defined agreements among the NPN roles, some specific UE-related data can be provided to authorized NPN customers such data may be processed or masked based on collected data such as MDT or trace. For example, GNSS information can be extracted from MDT to locate assets in NPN.

Observation 6: Rel-18 SI FS_OAM_NPN has studied some high-level requirements and procedures in TR 28.557 to enable MDT data collection from UEs in Non-Public networks (both SNPN and PNI-NPN).

Proposal 10: Both Signalling and management based MDT and for NPNs should be supported.

Proposal 11: Both immediate MDT and logged MDT for NPNs should be supported.

- MDT for the non-public network (NPN)

MRO has been supported for PLMN to optimize handover failures, radio link failures, and connection establishment/resume failures. Rel-18 should support MRO for NPNs. For MRO, the unique identification of the cells is important. For example, in the PLMN, RLF report, UE reports the global cell identity of the previous/failed/reestablished/reconnect cells. For the NPN MRO, unique identification for the NPN cell will be required. Therefore, the CGI report should be enhanced to include the NID such that NPN cells can be uniquely identified in the UE SON/MDT reports.

Proposal 12: Enhance NID in the CGI report to uniquely identify NPN cells.

In rel-17, the availability indicator for the PLMN SON report and actual SON report is sent to the gNB only after checking if the registered PLMN lies within the PLMN identity list stored in the UE variable for the SON report. Ran2 should follow a similar mechanism for reporting of availability indicator and SON report for NPN.

Proposal 13: UE report NPN SON reports availability and report only after checking if registered SNPN lies within the NPN identity list stored in the UE variable for the corresponding SON report.

- SON/MDT enhancements for fast MCG recovery

In rel-16, fast MCG recovery was introduced to increase the resiliency of the MCG link, whereupon the detection of MCG RLF, a UE in RRC_CONNECTED sends the MCGFailureInfomration to recover the MCG link. UE also stores the relevant radio link information in the varRLF-report. Upon transmission of MCGFailureInfomration, UE starts timer T316 and waits for RRCReconfiguration from the network. Upon T316 expiry, UE performs the connection re-establishment procedure. Note that while T316 is running, the SCG failure may happen, and in such as scenario UE will not receive RRCReconfiguration from the network.

In rel-16/rel-17, the RLF report is sent to the network upon successful MCG recovery procedure or upon establishment procedure. If the fast MCG recovery procedure succeeds, then UE indicates the RLF availability in the following RRCReconfigurationComplete and other following RRC complete messages. If fast MCG recovery fails, then UE indicates the RLF availability in the following RRCReestablishmentComplete and other following RRC complete messages. Note that, in the scenario, fast MCG recovery fails, UE can provide additional information that can help enhance the fast MCG recovery procedure.

Proposal 14: Upon the detection of fast MCG recovery failure, enhance the RLF report to include the following additional information,

- Upon t316 expiry, include an indicator to indicate fast MCG recovery failure

- Introduce an indicator to indicate if SCG Failure is detected during the MCG recovery procedure

- Include the cause of SCG failure, if SCG failure detected before the t316 expiry

- Include configured SCG RRM measurements in the RLF report, if SCG RLF is detected during the fast MCG recovery procedure

# Conclusion

Observation 1: In Rel-17, SCGFailureInformation was enhanced for reporting relevant MRO parameters for SCG failures.

Observation 2: Upon CPA and CPC failure, SCGFailureInformation is used to report CPA or CPC failures.

Proposal 1: RAN2 should consider enhancements of SCGFailureInformation for the following CPA and CPC failure scenarios,

- NO CPC execution and SCG RLF

- Failed CPC or CPA executions

- Successful CPC or CPA execution but RLF at target PSCell immediately after Successful CPC or CPA execution

Proposal 2: Minimize the impact on SCGFailureInformation size by only including the relevant information for CPAC failure not available or retrieved at the network.

Proposal 3: Include the following information in the SCGFailure for CPAC optimization:

- Indicate which configured execution event has been met if multiple execution conditions are configured at the UE

- The time gap between the reception of CPA or CPC configuration until execution or SCGFailure

- The time gap between configured execution conditions if multiple execution conditions are configured at the UE and CPA or CPC execution is performed

Proposal 4: RAN2 should deprioritize studying coexisting failure scenarios such as legacy HOF, CHO failure, or PCell RLF with CPAC failures, at least until RAN2 makes initial agreements on standalone CPAC failure scenarios.

Proposal 5: Define SCG T310, T312, and T304 thresholds for generating the SPC report.

Observation 3: PSCell change or addition procedure can fail due to the MCG RLF.

Proposal 6: Define MCG T310 and T312 thresholds for generating the SPC report.

Proposal 7: For the reporting of the SPC report, RAN2 can have the following options:

- Nested PSCell change report within SHR: The latest successful PSCell change report is nested within the SHR report.

- Independent PSCell change report: An independent successful PSCell change report can be introduced to report lower layer issues during the successful PSCell change or addition.

Proposal 8: RAN2 should consider the following two options for clearing the successful PSCell change report,

- Clear after 48 hours if not retrieved by network: UE generates the PSCell change report when configured trigger condition meets. Send the availability indicator in RRCReconfigurationComplete (containing RRCReconfigurationWithSYNC for SCG), and other RRC complete messages. Clear after 48 hours if not retrieved by the network.

- Clear upon PCell change: UE generates the PSCell change report when configured trigger condition meets. Send the availability indicator in RRCReconfigurationComplete (containing RRCReconfigurationWithSYNC for SCG) messages. Clear after UE changes PCell.

Observation 4: There is little benefit and motivation to optimize RLM configurations and successful handovers in LTE due to the absence of beam-based structure in LTE and the industry’s move towards more NR SA deployments.

Observation 5:  There is no significant requirement or benefit in cross-RAT retrieval of SHR as the SHR can always be reported once UE is back on the same RAT.

Proposal 9: Cross-RAT SHR reporting is not needed for inter-RAT handover scenarios.

Observation 6: Rel-18 SI FS_OAM_NPN has studied some high-level requirements and procedures in TR 28.557 to enable MDT data collection from UEs in Non-Public networks (both SNPN and PNI-NPN).

Proposal 10: Both Signalling and management based MDT and for NPNs should be supported.

Proposal 11: Both immediate MDT and logged MDT for NPNs should be supported.

Proposal 12: Enhance NID in the CGI report to uniquely identify NPN cells.

Proposal 13: UE report NPN SON reports availability and report only after checking if registered SNPN lies within the NPN identity list stored in the UE variable for the corresponding SON report.

Proposal 14: Upon the detection of fast MCG recovery failure, enhance the RLF report to include the following additional information,

- Upon t316 expiry, include an indicator to indicate fast MCG recovery failure

- Introduce an indicator to indicate if SCG Failure is detected during the MCG recovery procedure

- Include the cause of SCG failure, if SCG failure detected before the t316 expiry

- Include configured SCG RRM measurements in the RLF report, if SCG RLF is detected during the fast MCG recovery procedure

# Reference

[1] RP-221825, “Revised WID: Further enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity)”

[2] TS 38.331, “Radio Resource Control (RRC) protocol specification (Release 17)”

[3] TS 37.340, “Multi-connectivity; Stage 2 (Release 17)”

[4] TS 38.300, “NR and NG-RAN Overall description; Stage 2 (Release 17)”