3GPP TSG-RAN WG2 Meeting #119-e	R2-2208205

# Electronic, 17th - 29th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.323 | CR |  | rev |  | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of UDC in the description of Data field |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_UDC-Core |  |  |  |  |  | Date: |  |  | 2022-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A U-plane PDCP Data PDU consists of a PDCP PDU header and Data field. When UDC is configured the applicable Data PDU formats are: - 6.2.14 User plane PDCP Data PDU with long PDCP SN (12 bits) for UDC - 6.2.15 User plane PDCP Data PDU with extended PDCP SN (15 bits) for UDC - 6.2.16 User plane PDCP Data PDU with further extended PDCP SN (18 bits) for UDC Both UDC header and UDC Data Block are considered as PDCP SDU and this was captured in the description of the Data field in 6.3.3 by the informative note below. NOTE: If UDC is configured, uncompressed/compressed PDCP SDU comprises UDC header and UDC data block. With CR0277 “Specify UDC Header is part of Data Field” (R2-1915657, RAN2#108, Nov 2019) the above informative note in clause 6.3.3 was replaced in V15.5.0 by the bullet point below. - UDC header and UDC Data Block if UDC is configured. Furthermore, with CR0297r2 “Clarification on Security Coverage” (R2-2111648, RAN2#116-e, Nov 2021) the below note was added in V15.7.0, clause 6.3.3. NOTE: All fields other than PDCP PDU header and MAC-I belong to Data field.‎ However, this new note makes the description of UDC in clause 6.3.3 redundant but it was missed to remove the concerned bullet point at that time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The redundant bullet point saying “UDC header and UDC Data Block if UDC is configured.” has been removed from the description of Data field. Impact analysis Impacted functionality: Uplink Data Compression Inter-operability: There are no interoperability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Redundant description of UDC in the description of Data field remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

### 6.3.3	Data

Length: Variable

The Data field may include either one of the following:

-	Uncompressed PDCP SDU (user plane data, or control plane data); or

-	Compressed PDCP SDU (user plane data only).

NOTE:	All fields other than PDCP PDU header and MAC-I belong to Data field.‎

End of changes