3GPP TSG-RAN WG2 Meeting #119-e	R2-2208208

# Online, 17 – 29 August, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4855 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of overheating for NR SCG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN2#116e, a CR in R2-2111610 was agreed to correct the UE indication when the UE no longer experiences overheating for NR SCG in EN-DC case. The CR replaced the “overheatingAssistanceForSCG” with its parent IE “overheatingAssistance-v1610” since the former was a mandatory IE. In the coversheet of the above CR, the following Note was also captured to clarify how the UE can implement this: Note: not including the overheatingAssistance-v1610 signifies that UE can implement it by not including parent IE, e.g. UEAssistanceInformation-v1610-IEs or UEAssistanceInformation-v1530-IEs, the NW interpretation in both cases is that “UE did not include overheatingAssistance-v1610” However, the CR and thus the Rel-16 specification does not allow such UE implementation as the procedural text says that the UE will perform “do not include” in “OverheatingAssistance IE”. This means that the only option is to send an empty “OverheatingAssistance IE”. This has caused a different understanding between UE and network which was observed in the field. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarifying the expected UE behavior when no overheating is experienced by the UE for the MCG and SCG independently to avoid different interpretations. Impact Analysis: Impacted 5G architecture options: EN-DC Impacted functionality: SCG overheating Interoperability issue: - If the Network is implemented according to the CR and the UE is not, there is no inter-operability issue. - If the UE is implemented according to the CR and the Network is not, the Network may not interpret the UE indication for NR SCG overheating resolution correctly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There may be inter-operability issues when the UE no longer experiences overheating for NR SCG in EN-DC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.10.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Changes

#### 5.6.10.3	Actions related to transmission of UEAssistanceInformation message

The UE shall set the contents of the UEAssistanceInformation message for power preference indications:

1>	if configured to provide power preference indication and if the UE prefers a configuration primarily optimised for power saving:

2>	set powerPrefIndication to lowPowerConsumption;

1>	else if configured to provide power preference indication:

2>	set powerPrefIndication to normal;

The UE shall set the contents of the UEAssistanceInformation message for SPS assistance information:

1>	if configured to provide SPS assistance information:

2>	if there is any traffic for V2X sidelink communication which needs to report SPS assistance information:

3>	include trafficPatternInfoListSL in the UEAssistanceInformation message;

2>	if there is any traffic for uplink communication which needs to report SPS assistance information:

3>	include trafficPatternInfoListUL in the UEAssistanceInformation message;

The UE shall set the contents of the UEAssistanceInformation message for bandwidth preference indications:

1>	set bw-Preference to its preferred configuration;

The UE shall set the contents of the UEAssistanceInformation message for delay budget report:

1>	if configured to provide delay budget report:

2>	if the UE prefers an adjustment in the connected mode DRX cycle length:

3>	set delayBudgetReport to type1 according to a desired value;

2>	else if the UE prefers coverage enhancement configuration change:

3>	set delayBudgetReport to type2 according to a desired value;

The UE shall set the contents of the UEAssistanceInformation message for the RLM report:

1>	if configured to provide RLM report:

2>	if T314 has expired:

3>	set rlm-event to earlyOutOfSync;

2>	if T315 has expired:

3>	set rlm-event to earlyInSync;

3>	if configured to report rlmReportRep-MPDCCH:

4>	set excessRep-MPDCCH to the value indicated by lower layers;

The UE shall set the contents of the UEAssistanceInformation message for overheating assistance indication:

1>	if configured to provide overheating assistance indication:

2>	if the UE experiences internal overheating:

3>	if the UE prefers to temporarily reduce its DL category and UL category:

4>	include reducedUE-Category in the OverheatingAssistance IE;

4>	set reducedUE-CategoryDL to the number to which the UE prefers to temporarily reduce its DL category;

4>	set reducedUE-CategoryUL to the number to which the UE prefers to temporarily reduce its UL category;

3>	if the UE prefers to temporarily reduce the number of maximum secondary component carriers:

4>	include reducedMaxCCs in the OverheatingAssistance IE;

4>	set reducedCCsDL to the number of maximum SCells the UE prefers to be temporarily configured in downlink;

4>	set reducedCCsUL to the number of maximum SCells the UE prefers to be temporarily configured in uplink;

3>	if configured to provide overheating assistance indication for NR SCG:

4>	include overheatingAssistanceForSCG in the OverheatingAssistance IE;

4>	if configured with serving cells operating on FR2-2 for NR SCG

5>	include overheatingAssistanceForSCG-FR2-2 in the OverheatingAssistance IE;

4>	set overheatingAssistanceForSCG and if applicable, overheatingAssistanceForSCG-FR2-2, in accordance with clause 5.7.4.3a as specified in TS 38.331 [82];

2>	else (if the UE no longer experiences an overheating condition):

3>  if the UE had a preference for the OverheatingAssistance IE:

4> do not include reducedUE-Category, reducedMaxCCs in OverheatingAssistance IE;

3>  if the UE had a preference for the overheatingAssistanceForSCG IE:

4>  do not include overheatingAssistance-v1610 in the UEAssistanceInformation-v1610 IE;

4> if configured with serving cells operating on FR2-2 for NR SCG

5> do not include OverheatingAssistance-v1710 in the UEAssistanceInformation-v1710 IE;

NOTE 0:	The UE may send UEAssistanceInformation-v1610 IE and/or UEAssistanceInformation-v1710 IE even when not configured to provide overheating assistance indication for NR SCG in case the NR SCG was released due to overheating.

The UE shall set the contents of the UEAssistanceInformation message for NR SCG deactivation:

1>	if configured to provide its preference for NR SCG deactivation;

2>	if the UE prefers NR SCG to be deactivated

3>	include the scg-DeactivationPreference and set it to scgDeactivationPreferred:

2>	else:

3>	include the scg-DeactivationPreference and set it to noPreference:

The UE shall:

1>	if the UE is configured with a deactivated NR SCG and there are uplink data to send on a DRB for which rlc-Config is not configured in drb-ToAddModList: and

1>	if the UE previously did not have any uplink data to send for any SCG RLC entity:

2>	include uplinkData in the UEAssistanceInformation message;

1>	if the procedure was triggered to provide SPS assistance information and the related configuration was provided by an RRCConnectionReconfiguration message that was received embedded within an NR RRCReconfiguration message:

2>	submit the UEAssistanceInformation message via SRB1 embedded in NR RRC message ULInformationTransferIRAT as specified in TS 38.331 [82];

1>	else:

2>	submit the UEAssistanceInformation message to lower layers for transmission.

NOTE 1:	It is up to UE implementation when and how to trigger SPS assistance information.

NOTE 2:	It is up to UE implementation to set the content of trafficPatternInfoListSL and trafficPatternInfoListUL.

NOTE 3:	Traffic patterns for different Destination Layer 2 IDs are provided in different entries in trafficPatternInfoListSL.

NOTE 4:	Although not recommended, UE may start or restart the following timers whenever it sends the UEAssistanceInformation message (i.e. even if the message was not triggered for the concerned feature): T340, T341, T342, T343, T344 and T345.

End of Changes