3GPP TSG-RAN WG2 Meeting #119e	R2-2208230

Online, 17th Aug– 29th Aug, 2022

Title:		Discussion on FDM enhancement

Source:	Huawei, HiSilicon

Agenda item:	8.10.2

Document for:	Discussion and Decision

# Introduction

In Rel-15, the IDC Frequency Division Multiplexing (FDM) solution was introduced to support the EN-DC. In Rel-16, the IDC FDM solution was introduced to support the standalone NR. However, the current IDC solution in NR has the following limitations

- The affected frequencies cannot be adequately indicated via the NR FDM solution

- TDM solution as in LTE is not available

In RAN#94e, the new work item for In-Device Co-existence (IDC) enhancements for NR and MR-DC was approved [1]. In the WID, the following objective was agreed:

This WI expects to address interference between 3GPP (including various MR-DC architectures, i.e. NR-DC and EN-DC) and non-3GPP RAT (e.g. WiFi). - Enhancements to FDM solution, to allow more granular indication of affected frequencies (e.g. granularity of BWP or PRB level). (RAN2) Note: Enhancements to FDM solution is prioritized. - Introduction of TDM solution (e.g. indication of UE preferred TDM pattern for UL/DL). (RAN2, RAN4). Note: The TDM solution is considered complementary to the FDM solution. - Specify RRM requirements for TDM solution (RAN4) Note: LTE IDC solution should be considered as the baseline for the solutions developed in this WI.

This paper first discusses the scenarios where the FDM solution enhancement is needed to address the IDC interference in Rel-18 followed by the details of the FDM solution enhancements for these scenarios respectively.

# Discussion

## 2.1 Scenarios for IDC interference in Rel.18

The scenarios for IDC interference can be categorized into two main scenarios that needs to be addressed in Rel.18. These scenarios are

#### 2.1.1 Scenario 1: Adjacent channel interference between NR and non-3GPP

In this scenario IDC interference will be caused by the NR Transmission (Tx) to Non-3GPP Reception (Rx) or by the Non-3GPP Transmission (Tx) to NR Reception (Rx) due to the operational NR bands are adjacent to the ISM bands. An example of this scenario is as shown in Figure 1.

Figure 1 - Example of adjacent channel interference between NR and non-3GPP

Depending on whether the NR node is operating in standalone mode or as MN of NR-DC or as SN of MR-DC, this scenario can be further categorized into following two sub scenarios

Scenario 1-1: Adjacent channel interference between NR Stand Alone (SA) or MN of NR-DC and non-3GPP

Scenario 1-2: Adjacent channel interference between SN (NR) of MR-DC and non-3GPP

#### 2.1.2 Scenario 2: Intermodulation Distortion (IMD) interference from simultaneous Tx in MR-DC to non-3GPP

In this scenario the inter modulation distortion interference will be caused by the simultaneous transmission (Tx) in the uplink within MCG and SCG (including NR-DC and EN-DC case) to non-3GPP due to the combination of the operational bands used during MR-DC operation. An example of this scenario is as shown in Figure 2.

Figure 2 - Example of IMD interference from simultaneous Tx in MR-DC to non-3GPP

Depending on whether the MR-DC configuration is EN-DC or NR-DC, this scenario can also be further categorized into following two sub scenarios

Scenario 2-1: IMD interference from simultaneous Tx in EN-DC to non-3GPP

Scenario 2-2: IMD interference from simultaneous Tx in NR-DC to non-3GPP

For all of the above scenarios, the FDM solution to address the IDC issue has be supported by the current specification, so we think that all of these scenarios should be considered for the FDM enhancement in Rel.18. Hence, we propose the following

Proposal 1: The following scenarios are considered for FDM enhancements

- Scenario 1: Adjacent channel interference between NR and non-3GPP that includes the following sub scenarios

  - Scenario 1-1: Adjacent channel interference between NR Stand Alone (SA) or MN of NR-DC and non-3GPP

  - Scenario 1-2: Adjacent channel interference between SN (NR) of MR-DC and non-3GPP

- Scenario 2: Intermodulation Distortion (IMD) interference from simultaneous Tx in MR-DC to non-3GPP that includes the following sub scenarios

  - Scenario 2-1: IMD interference from simultaneous Tx in EN-DC to non-3GPP

  - Scenario 2-2: IMD interference from simultaneous Tx in NR-DC to non-3GPP

In the following sections we discuss the FDM enhancements needed for each of these scenarios

## 2.2 FDM Enhancement for Scenario 1-1

The current IDC FDM solution for NR is the same as the one used for LTE IDC:

- gNB configures the UE with the candidate serving frequency list, which indicates the center frequency around which the UE is requested to report IDC issues;

- On detecting the IDC issue, the UE reports the affected carrier frequency list, which indicate the center of affected carrier frequencies to the gNB.

However, since the bandwidth of the carrier is large in NR, the actually affected frequency range may only be part of entire carrier, hence the UE reporting  the center of affected carrier frequency does not provide enough information to help the gNB to address the IDC issue. That is also the motivation for FDM solution enhancement in Rel.18, i.e. to allow more granular indication of affected frequencies to the gNB. We think two aspects needs to be considered for FDM enhancement.

#### 2.2.1 gNB configuration for IDC

If the UE needs to report more granular indication of the affected frequencies, the gNB should also provide more granular indication of the candidate serving frequencies for which the UE is requested  to report the IDC issue. Otherwise, what the UE reports may be not what the gNB  is interested  in knowing.

So, in addition to configuring the candidate serving frequency list (center frequency), the gNB should also configure the candidate bandwidth for each of the candidate serving frequency, which is used to indicate the frequency range the UE is requested to report IDC issues as shown in Figure 3.

Figure 3 - Enhanced gNB configuration for IDC including candidate serving frequency list and candidate bandwidth

Proposal 2-1: For each candidate serving frequency (center frequency), the gNB will configure the candidate bandwidth, the combination of which is used to indicate the frequency range of the corresponding candidate serving frequency the UE is request to report IDC issues

#### 2.2.2 IDC UE assistance information reporting

For the granular indication of affected frequencies, two examples given in the WID are PRB and BWP. If the affected frequency is the UE’s serving frequency, then there is no problem for the UE to report the actually affected PRB or BWP. However, if the affected frequency is the UE’s non-serving frequency, the UE will not be able to report the actually affected PRB or BWP since UE has no information on the PRB or BWP of the non-serving frequency. To have a common design for both serving frequency and non-serving frequency, we prefer that the UE report the actually affected bandwidth and the central frequency of this bandwidth for each affected frequency as shown in Figure 4. Of course, the frequency range that is determined by the affected bandwidth and central frequency are within the frequency range configured by the gNB for IDC.

Figure 4 - Enhanced UE reporting for IDC including actual affected bandwidth and the central frequency of this bandwidth

In addition, considering that the actually affected frequency range for the interference from NR TX to non-3GPP RX and the interference from non-3GPP TX to NR RX could be different, the UE should be allowed to report two different frequency range for each candidate serving frequency

Proposal 2-2: For each affected frequency, if the actually affected frequency range is within the frequency range that is configured by the gNB, UE reports the actually affected bandwidth and the central frequency of this bandwidth for each interference direction i.e. interference from NR TX to non-3GPP RX and non-3GPP TX to NR RX respectively.

## 2.3 FDM Enhancement for Scenario 1-2

In case the NR node is operating as a SN within the MR-DC configuration and in case only SN frequency is affected by IDC problem, the same enhancements for the Scenario 1-1 described above for NR Stand Alone (SA) mode can be applied.

There are two ways to support the FDM enhancement of scenario 1-1 in scenario 1-2 depending upon whether SRB3 is configured or not as shown in Figure 5.

Figure 5 - Enhanced IDC configuration from SN and UE reporting to SN

Case 1 - SRB3 is not configured: SN configures the enhanced IDC related configuration via SRB1 by using the RRCReconfiguration container to the UE and UE reports the enhanced IDC assistance information via SRB1, i.e. by reporting UAI in the ULInformationTransferMRDC message which is then transferred to SN using RRCTransfer message

Case 2 - SRB3 is configured: SN configures the enhanced IDC related configuration to the UE via SRB3 and UE reports the IDC assistance information directly via SRB3

By using these  signaling procedures, we can avoid further specification impact on top of the FDM solution for scenario 1-1, especially avoid impact to 36.331 in case of EN-DC.

Proposal 3: To support the FDM enhancement as in Proposal 2-1 and 2-2 for scenario 1-2,

- If SRB3 is not configured: SN configures the enhanced IDC related configuration as given in proposal 2-1 via SRB1 by using the RRCReconfiguration container to the UE, and UE reports the IDC assistance information with more granular/enhanced FDM information as given in proposal 2-2 to SN via SRB1 by reporting UAI in the 	ULInformationTransferMRDC message

- if SRB3 is configured: SN configures the enhanced IDC related configuration as given in proposal 2-1 to the UE via SRB3, and UE reports the IDC assistance information with more granular/enhanced FDM information as given in proposal 2-2 to SN via SRB3

## FDM Enhancement for Scenario 2-1

For the EN-DC, Figure 6 shows the involved procedures for the current FDM solution. For the scenario 2-1, the same enhancements for the Scenario 1-1 described above can be applied on top of the current procedures shown in Figure 6. To be specific:

- When the SN provide configurations for IDC to the MN via the CG-Config, the SN includes the enhanced IDC related configuration as given in proposal 2-1;

- When the MN configures otherConfig for the IDC to the UE, the MN includes the above information provided by the SN in the CG-Config for IDC

- When the UE detects the IDC issues for EN-DC, for the affected NR frequency, the UE reports the IDC assistance information with more granular/enhanced FDM information as given in proposal 2-2 to the MN;

- The MN transfer the IDC assistance information for EN-DC to the SN by initiating the SN modification procedure and include the information that the UE reports as above in the CG-ConfigInfo

UE

MN

SN

1: Information of Candidate SN serving Frequency List

2: Information of Candidate SN serving Frequency List

(In CG-Config)

(In Other-Config)

3: Information of actually affected SN serving Frequency List

4: Information of actually affected SN serving Frequency List

(In CG-ConfigInfo)

(In IDC assistance information)

Figure 6 – EN-DC procedure for the current FDM based solution

Proposal 4: To support the FDM enhancement as in Proposal 2-1 and 2-2 for scenario 2-1:

- When the SN provide configurations for IDC to the MN via the CG-Config, the SN includes the enhanced IDC related configurations as given in proposal 2-1, and the MN provides such configurations to the UE in the otherConfig

- When the UE detects the IDC issues for EN-DC, for the affected NR frequency, the UE reports the IDC assistance information with more granular/enhanced FDM information as given in proposal 2-2 to the MN, and MN transfer such information to the SN in the CG-ConfigInfo.

## 2.5 FDM Enhancement for Scenario 2-2:

For the NR-DC, currently there is no FDM based solution in place, in the current specification, for reporting of Intermodulation Distortion interference arising from simultaneous Tx during NR-DC operation. This is because the affectedCarrierFreqCombList IE in the UEAssistanceInformation message currently supports the reporting of list of NR carrier frequency combinations that are affected by IDC problems due to Inter-Modulation Distortion and harmonics from NR when configured with only uplink carrier aggregation UL CA.

So, for this scenario, the FDM solution should be introduced firstly, and then the same enhancements as in proposal 4 can be applied.

To support the FDM solution for NR-DC, the current affectedCarrierFreqCombList in the UAI message for IDC should be extended to cover the NR-DC case. In other words, in case Intermodulation Distortion interference is caused by simultaneous TX on UL frequency to non-3GPP during the NR-DC operation then it is necessary that the UE reports the combination of such NR carrier frequencies to the network when configured with NR-DC.

Additionally, in the CG-configInfo, the affectedCarrierFreqCombMRDC should also be extended to support the NR-DC case

Hence for this scenario, we propose the following

Proposal 5-1: To introduce the FDM solution for the Scenario 2-2 firstly by extending the affectedCarrierFreqCombList in the UAI message for IDC and the affectedCarrierFreqCombMRDC in the CG-configInfo to cover the NR-DC case.

Proposal 5-2: To further support the FDM enhancement as in Proposal 2-1 and 2-2 for scenario 2-2:

- When the SN provide configurations for IDC to the MN via the CG-Config, the SN includes the enhanced IDC related configurations as given in proposal 2-1, and the MN provides such configurations to the UE in the otherConfig

- When the UE detects the IDC issues for NR-DC, for the affected NR frequency, the UE reports the IDC assistance information with more granular/enhanced FDM information as given in proposal 2-2 to the MN, and MN transfer such information to the SN in the CG-ConfigInfo.

## 2.6 gNB actions on receiving enhanced assistance information

On receiving the enhanced IDC assistance information for the UE about the actually affected bandwidth and the central frequency of this bandwidth for each interference direction separately, the gNB can apply scheduling restrictions to not schedule the UE on the actual affected frequency ranges or on the PRBs within the actual affected frequency ranges or to reconfigure the BWP for the UE to resolve the IDC issue depending upon its implementation.

Proposal 6: gNB can apply scheduling restrictions to not schedule the UE on the actual affected frequency ranges or on the PRBs within the actual affected frequency ranges or to reconfigure the BWP for the UE to resolve the IDC issue depending upon its implementation

# Conclusions

In this paper we discuss various scenarios that needs to be addressed for IDC interference in Rel-18 followed by the details of the enhancements required for the IDC FDM solutions to address these scenarios and have the following proposals.

Scenarios for IDC interference in Rel.18

Proposal 1: The following scenarios are considered for FDM enhancements

- Scenario 1: Adjacent channel interference between NR and non-3GPP that includes the following sub scenarios

  - Scenario 1-1: Adjacent channel interference between NR Stand Alone (SA) or MN of NR-DC and non-3GPP

  - Scenario 1-2: Adjacent channel interference between SN (NR) of MR-DC and non-3GPP

- Scenario 2: Intermodulation Distortion (IMD) interference from simultaneous Tx in MR-DC to non-3GPP that includes the following sub scenarios

  - Scenario 2-1: IMD interference from simultaneous Tx in EN-DC to non-3GPP

  - Scenario 2-2: IMD interference from simultaneous Tx in NR-DC to non-3GPP

FDM Enhancement for Scenario 1-1

Proposal 2-1: For each candidate serving frequency (center frequency), the gNB will configure the candidate bandwidth, the combination of which is used to indicate the frequency range of the corresponding candidate serving frequency the UE is request to report IDC issues

Proposal 2-2: For each affected frequency, if the actually affected frequency range is within the frequency range that is configured by the gNB, UE reports the actually affected bandwidth and the central frequency of this bandwidth for each interference direction i.e. interference from NR TX to non-3GPP RX and non-3GPP TX to NR RX respectively.

FDM Enhancement for Scenario 1-2

Proposal 3: To support the FDM enhancement as in Proposal 2-1 and 2-2 for scenario 1-2,

- If SRB3 is not configured: SN configures the enhanced IDC related configuration as given in proposal 2-1 via SRB1 by using the RRCReconfiguration container to the UE, and UE reports the IDC assistance information with more granular/enhanced FDM information as given in proposal 2-2 to SN via SRB1 by reporting UAI in the 	ULInformationTransferMRDC message

- if SRB3 is configured: SN configures the enhanced IDC related configuration as given in proposal 2-1 to the UE via SRB3, and UE reports the IDC assistance information with more granular/enhanced FDM information as given in proposal 2-2 to SN via SRB3

FDM Enhancement for Scenario 2-1

Proposal 4: To support the FDM enhancement as in Proposal 2-1 and 2-2 for scenario 2-1:

- When the SN provide configurations for IDC to the MN via the CG-Config, the SN includes the enhanced IDC related configurations as given in proposal 2-1, and the MN provides such configurations to the UE in the otherConfig

- When the UE detects the IDC issues for EN-DC, for the affected NR frequency, the UE reports the IDC assistance information with more granular/enhanced FDM information as given in proposal 2-2 to the MN, and MN transfer such information to the SN in the CG-ConfigInfo.

FDM Enhancement for Scenario 2-2

Proposal 5-1: To introduce the FDM solution for the Scenario 2-2 firstly by extending the affectedCarrierFreqCombList in the UAI message for IDC and the affectedCarrierFreqCombMRDC in the CG-configInfo to cover the NR-DC case.

Proposal 5-2: To further support the FDM enhancement as in Proposal 2-1 and 2-2 for scenario 2-2:

- When the SN provide configurations for IDC to the MN via the CG-Config, the SN includes the enhanced IDC related configurations as given in proposal 2-1, and the MN provides such configurations to the UE in the otherConfig

- When the UE detects the IDC issues for NR-DC, for the affected NR frequency, the UE reports the IDC assistance information with more granular/enhanced FDM information as given in proposal 2-2 to the MN, and MN transfer such information to the SN in the CG-ConfigInfo.

gNB actions on receiving enhanced assistance information

Proposal 6: gNB can apply scheduling restrictions to not schedule the UE on the actual affected frequency ranges or on the PRBs within the actual affected frequency ranges or to reconfigure the BWP for the UE to resolve the IDC issue depending upon its implementation

# References

- RP-213589 New WI: In-Device Co-existence (IDC) enhancements for NR and MR-DC

- TS 38.331 NR Radio Resource Control (RRC) protocol specification v 17.1.0

- TS 36.331 LTE Radio Resource Control (RRC) protocol specification v 17.1.0