3GPP TSG-RAN WG2 Meeting #119-electronic	   R2-2208265

Online, August 17th - August 29th, 2022

Agenda item:		5.1.3.1.2

Source:	vivo

Title:	Discussion on SI-request Period Issues

Document for:		Discussion and Decision

1 Introduction

For Rel-15 Msg-1 based SI request, si-RequestPeriod can be configured to indicate there is one RACH association period that can be used for Msg-1 based SI request within si-RequestPeriod association periods.

1> if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]); and 1> if ra-AssociationPeriodIndex and si-RequestPeriod are configured: 2> determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB in the association period given by ra-AssociationPeriodIndex in the si-RequestPeriod permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6] corresponding to the selected SSB).

However, in practice, we figure out there are two issues (i.e. starting point ambiguity and 1024 SFN boundary-crossing) regarding si-RequestPeriod. In this contribution, we would like to go into the issues in detail and provide our solutions to these two issues with corresponding TP in Annex.

2 Discussion

According to the RRC spec [1], currently, only one parameter called si-RequestPeriod is used to configure the SI-request period.  However, this parameter only indicates the varied period length in the unit of RA associate period. Whether time offset is used in deriving the SI-request period is not clear. To get rid of the potential ambiguity, we think RAN2 should clarify that the SI-request period is started from frame 0, considering that the associate period is started from frame 0, as specified in section 8.1 in 38.213 [2].

Proposal 1: RAN2 clarifies SI request period is starting from SFN0.

SI-RequestConfig information element

-- ASN1START

-- TAG–SI-REQUESTCONFIG-START

SI-RequestConfig ::=                SEQUENCE {

rach-OccasionsSI                    SEQUENCE {

rach-ConfigSI                       RACH-ConfigGeneric,

ssb-perRACH-Occasion                ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

}                                                                                                       OPTIONAL,   -- Need R

si-RequestPeriod                    ENUMERATED {one, two, four, six, eight, ten, twelve, sixteen}       OPTIONAL,   -- Need R

si-RequestResources                 SEQUENCE (SIZE (1..maxSI-Message)) OF SI-RequestResources

}

SI-RequestResources ::=             SEQUENCE {

ra-PreambleStartIndex               INTEGER (0..63),

ra-AssociationPeriodIndex           INTEGER (0..15)                                                     OPTIONAL,   -- Need R

ra-ssb-OccasionMaskIndex            INTEGER (0..15)                                                     OPTIONAL    -- Need R

}

-- ASN1STOP

-- TAG–SI-REQUESTCONFIG-STOP

SI-RequestConfig field descriptions

rach-OccasionsSI Configuration of dedicated RACH Occasions for SI. If the field is absent, the UE uses the corresponding parameters configured in rach-ConfigCommon of the initial uplink BWP.

si-RequestPeriod Periodicity of the SI-Request configuration in number of association periods.

si-RequestResources If there is only one entry in the list, the configuration is used for all SI messages for which si-BroadcastStatus is set to notBroadcasting. Otherwise the 1st entry in the list corresponds to the first SI message in schedulingInfoList for which si-BroadcastStatus is set to notBroadcasting, 2nd entry in the list corresponds to the second SI message in schedulingInfoList for which si-BroadcastStatus is set to notBroadcasting and so on. Change of si-RequestResources should not result in system information change notification.

Next, we would like to give more details regarding the 1024 SFN boundary-crossing issue. Taking the following Figure 1 as an example (assuming si-RequestPeriod = n4), we can figure that the SI-request period herein equals 120ms, by which cannot be divided by 1024SFN (i.e. 10240ms). As a result, if ra-AssociationPeriodIndex is configured to 3 for this case, it means the UE cannot initiate the RA procedure for Msg-1 based SI request as there is no next available ROs, when 1024 SFN boundary-crossing issue occurs for SI-request period. To get rid of this issue, we think the simplest way to require the network to ensure the SI-request period is restricted to be a 2μ multiple of 10ms, where μ is a non-negative integer up to the maximum value 4 (i.e. the maximum length of SI-request period is not expected to be larger than the maximum value of RA associate pattern period (i.e. 160ms at most)).

Figure 1: 1024 SFN boundary-crossing issue for SI-request period

Proposal 2: Network ensures the configured period is restricted to be a 2μ multiple of 10ms, where μ is a non-negative integer up to the maximum value 4.

Last but not least, we have prepared the corresponding text proposal for 38.331.

Proposal 3: If Proposal 2 is agreeable, RAN2 adopts the text proposal in Annex.

3 Conclusion

In this contribution, we have discussed two issues regarding si-RequestPeriod and provided our solutions with corresponding TP in Annex. All the proposals are summarized as follows,

Proposal 1: RAN2 clarifies SI request period is starting from SFN0.

Proposal 2: Network ensures the configured period is restricted to be a 2μ multiple of 10ms, where μ is a non-negative integer up to the maximum value 4.

Proposal 3: If Proposal 2 is agreeable, RAN2 adopts the text proposal in Annex.

4 References

[1] 3GPP TS 38.331: “NR; Radio Resource Control (RRC) protocol specification”.

[1] 3GPP TS 38.213: “NR; Radio Resource Control (RRC) protocol specification”.

Annex

Text proposal for TS 38.331:

START OF the CHANGE

––	SI-RequestConfig

The IE SI-RequestConfig contains configuration for Msg1 based SI request.

SI-RequestConfig information element

-- ASN1START

-- TAG–SI-REQUESTCONFIG-START

SI-RequestConfig ::=                SEQUENCE {

rach-OccasionsSI                    SEQUENCE {

rach-ConfigSI                       RACH-ConfigGeneric,

ssb-perRACH-Occasion                ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

}                                                                                                       OPTIONAL,   -- Need R

si-RequestPeriod                    ENUMERATED {one, two, four, six, eight, ten, twelve, sixteen}       OPTIONAL,   -- Need R

si-RequestResources                 SEQUENCE (SIZE (1..maxSI-Message)) OF SI-RequestResources

}

SI-RequestResources ::=             SEQUENCE {

ra-PreambleStartIndex               INTEGER (0..63),

ra-AssociationPeriodIndex           INTEGER (0..15)                                                     OPTIONAL,   -- Need R

ra-ssb-OccasionMaskIndex            INTEGER (0..15)                                                     OPTIONAL    -- Need R

}

-- ASN1STOP

-- TAG–SI-REQUESTCONFIG-STOP

SI-RequestConfig field descriptions

rach-OccasionsSI Configuration of dedicated RACH Occasions for SI. If the field is absent, the UE uses the corresponding parameters configured in rach-ConfigCommon of the initial uplink BWP.

si-RequestPeriod Periodicity of the SI-RequestConfig in number of association periods, starting from frame 0. Network ensures the configured period is restricted to be a 2μ multiple of 10ms, where μ is a non-negative integer up to the maximum value 4.

si-RequestResources If there is only one entry in the list, the configuration is used for all SI messages for which si-BroadcastStatus is set to notBroadcasting. Otherwise the 1st entry in the list corresponds to the first SI message in schedulingInfoList for which si-BroadcastStatus is set to notBroadcasting, 2nd entry in the list corresponds to the second SI message in schedulingInfoList for which si-BroadcastStatus is set to notBroadcasting and so on. Change of si-RequestResources should not result in system information change notification.

SI-RequestResources field descriptions

ra-AssociationPeriodIndex Index of the association period in the si-RequestPeriod in which the UE can send the SI request for SI message(s) corresponding to this SI-RequestResources, using the preambles indicated by ra-PreambleStartIndex and rach occasions indicated by ra-ssb-OccasionMaskIndex.

ra-PreambleStartIndex If N SSBs are associated with a RACH occasion, where N > = 1, for the i-th SSB (i=0, …, N-1) the preamble with preamble index = ra-PreambleStartIndex + i is used for SI request; For N < 1, the preamble with preamble index = ra-PreambleStartIndex is used for SI request.

END OF the CHANGE