3GPP TSG-RAN WG2 Meeting #119-electronic	R2-2208266

# Online, August 17th - August 29th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1377 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on CG-SDT Transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Core |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - During the ongoing CG-SDT procedure, 2-step RA may be initiated, and consequently, Absolute Timing Advance Command will be received, which should have triggered the restart of the cg-SDT-TimeAlignmentTimer. - cg-SDT-RetransmissionTimer can be configured to autonomously retransmit the initial MAC PDU of CG-SDT with CCCH message, which is not captured in the current MAC spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - During the ongoing CG-SDT procedure, cg-SDT-TimeAlignmentTimer is restarted upon the reception of Absolute Timing Advance Command. - Definition and usage of cg-SDT-RetransmissionTimer is added in sub-clause 5.8.2. Impact analysis Impacted 5G architecture options: Standalone Impacted functionality: CG-SDT Inter-operability: If the UE is implemented according to this CR but the network is not, or vice versa, there is no inter-operability issue foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CG-SDT functionality is broken. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 5.8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF THE CHANGE

5.2	Maintenance of Uplink Time Alignment

RRC configures the following parameters for the maintenance of UL time alignment:

-	timeAlignmentTimer (per TAG) which controls how long the MAC entity considers the Serving Cells belonging to the associated TAG to be uplink time aligned;

-	inactivePosSRS-TimeAlignmentTimer which controls how long the MAC entity considers the Positioning SRS transmission in RRC_INACTIVE in clause 5.26 to be uplink time aligned;

-	cg-SDT-TimeAlignmentTimer which controls how long the MAC entity considers the uplink transmission for CG-SDT to be uplink time aligned.

The MAC entity shall:

1>	when a Timing Advance Command MAC CE is received, and if an NTA (as defined in TS 38.211 [8]) has been maintained with the indicated TAG:

2>	apply the Timing Advance Command for the indicated TAG;

2>	if inactivePosSRS-TimeAlignmentTimer is configured and there is ongoing Positioning SRS Transmission in RRC_INACTIVE as in clause 5.25:

3>	start or restart the inactivePosSRS-TimeAlignmentTimer associated with the indicated TAG.

2>	if CG-SDT procedure triggered as in clause 5.27 is ongoing:

3>	start or restart the cg-SDT-TimeAlignmentTimer associated with the indicated TAG.

2>	start or restart the timeAlignmentTimer associated with the indicated TAG.

1>	when a Timing Advance Command is received in a Random Access Response message for a Serving Cell belonging to a TAG or in a MSGB for an SpCell:

2>	if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble:

3>	apply the Timing Advance Command for this TAG;

3>	start or restart the timeAlignmentTimer associated with this TAG.

2>	else if the timeAlignmentTimer associated with this TAG is not running:

3>	apply the Timing Advance Command for this TAG;

3>	start the timeAlignmentTimer associated with this TAG;

3>	when the Contention Resolution is considered not successful as described in clause 5.1.5; or

3>	when the Contention Resolution is considered successful for SI request as described in clause 5.1.5, after transmitting HARQ feedback for MAC PDU including UE Contention Resolution Identity MAC CE:

4>	stop timeAlignmentTimer associated with this TAG.

3>	when the Contention Resolution is considered not successful as described in clause 5.1.5:

4>	if CG-SDT procedure triggered as in clause 5.27 is ongoing:

5>	set the NTA value to the value before applying the received Timing Advance Command as in TS 38.211 [8].

3>	when the Contention Resolution is considered successful for Random Access procedure while the CG-SDT procedure is ongoing:

4>	stop timeAlignmentTimer associated with this TAG;

4>	start or restart the cg-SDT-TimeAlignmentTimer associated with this TAG.

2>	else:

3>	ignore the received Timing Advance Command.

1>	when an Absolute Timing Advance Command is received in response to a MSGA transmission including C-RNTI MAC CE as specified in clause 5.1.4a:

2>	apply the Timing Advance Command for PTAG;

2>	start or restart the timeAlignmentTimer associated with PTAG;

2>	if CG-SDT procedure triggered as in clause 5.27 is ongoing:

3>	start or restart the cg-SDT-TimeAlignmentTimer associated with the indicated TAG.

1>	when the indication is received from upper layer for stopping the inactivePosSRS-TimeAlignmentTimer:

2>	stop the inactivePosSRS-TimeAlignmentTimer.

1>	when the indication is received from upper layer for starting the inactivePosSRS-TimeAlignmentTimer:

2>	start or restart the inactivePosSRS-TimeAlignmentTimer.

1>	when instruction from the upper layer has been received for starting the cg-SDT-TimeAlignmentTimer:

2>	start the cg-SDT-TimeAlignmentTimer.

1>	when instruction from the upper layer has been received for stopping the cg-SDT-TimeAlignmentTimer:

2>	consider the cg-SDT-TimeAlignmentTimer as expired.

1>	when instruction from the upper layer has been received for starting the TimeAlignmentTimer associated with PTAG:

2>	start the TimeAlignmentTimer associated with PTAG.

1>	when a timeAlignmentTimer expires:

2>	if the timeAlignmentTimer is associated with the PTAG:

3>	flush all HARQ buffers for all Serving Cells;

3>	notify RRC to release PUCCH for all Serving Cells, if configured;

3>	notify RRC to release SRS for all Serving Cells, if configured;

3>	clear any configured downlink assignments and configured uplink grants;

3>	clear any PUSCH resource for semi-persistent CSI reporting;

3>	consider all running timeAlignmentTimers as expired;

3>	maintain NTA (defined in TS 38.211 [8]) of all TAGs.

2>	else if the timeAlignmentTimer is associated with an STAG, then for all Serving Cells belonging to this TAG:

3>	flush all HARQ buffers;

3>	notify RRC to release PUCCH, if configured;

3>	notify RRC to release SRS, if configured;

3>	clear any configured downlink assignments and configured uplink grants;

3>	clear any PUSCH resource for semi-persistent CSI reporting;

3>	maintain NTA (defined in TS 38.211 [8]) of this TAG.

1>	when the inactivePosSRS-TimeAlignmentTimer expires:

2>	notify RRC to release Positioning SRS for RRC_INACTIVE configuration(s).

1>	when the cg-SDT-TimeAlignmentTimer expires:

2>	clear any configured uplink grants;

2>	if a PDCCH addressed to the MAC entity's C-RNTI after initial transmission for the CG-SDT with CCCH message has not been received:

3>	consider ongoing CG-SDT procedure as terminated;

3>	indicate the expiry of cg-SDT-TimeAlignmentTimer to the upper layer.

2>	flush all HARQ buffers;

2>	maintain NTA (defined in TS 38.211 [8]) of this TAG.

When the MAC entity stops uplink transmissions for an SCell due to the fact that the maximum uplink transmission timing difference between TAGs of the MAC entity or the maximum uplink transmission timing difference between TAGs of any MAC entity of the UE is exceeded, the MAC entity considers the timeAlignmentTimer associated with the SCell as expired.

The MAC entity shall not perform any uplink transmission on a Serving Cell except the Random Access Preamble and MSGA transmission when the timeAlignmentTimer associated with the TAG to which this Serving Cell belongs is not running and CG-SDT procedure is not ongoing. Furthermore, when the timeAlignmentTimer associated with the PTAG is not running and CG-SDT procedure is not ongoing, the MAC entity shall not perform any uplink transmission on any Serving Cell except the Random Access Preamble and MSGA transmission on the SpCell. The MAC entity shall not perform any uplink transmission except the Random Access Preamble and MSGA transmission when the cg-SDT-TimeAlignmentTimer is not running during the ongoing CG-SDT procedure as triggered in clause 5.27.

NEXT CHANGE

5.8.2	Uplink

There are two types of transmission without dynamic grant:

-	configured grant Type 1 where an uplink grant is provided by RRC, and stored as configured uplink grant;

-	configured grant Type 2 where an uplink grant is provided by PDCCH, and stored or cleared as configured uplink grant based on L1 signalling indicating configured uplink grant activation or deactivation.

Type 1 and Type 2 are configured by RRC for a Serving Cell per BWP. Multiple configurations can be active simultaneously in the same BWP. For Type 2, activation and deactivation are independent among the Serving Cells. For the same BWP, the MAC entity can be configured with both Type 1 and Type 2.

Only configured grant Type 1 can be configured for CG-SDT. CG-SDT can only be configured on initial BWP.

RRC configures the following parameters when the configured grant Type 1 is configured:

-	cs-RNTI: CS-RNTI for retransmission;

-	cg-SDT-RSRP-ThresholdSSB: an RSRP threshold configured for SSB selection for CG-SDT;

-	periodicity: periodicity of the configured grant Type 1;

-	timeDomainOffset: Offset of a resource with respect to SFN = timeReferenceSFN in time domain;

-	timeDomainAllocation: Allocation of configured uplink grant in time domain which contains startSymbolAndLength (i.e. SLIV in TS 38.214 [7]) or startSymbol (i.e. S in TS 38.214 [7]);

-	nrofHARQ-Processes: the number of HARQ processes for configured grant;

-	harq-ProcID-Offset: offset of HARQ process for configured grant configured with cg-RetransmissionTimer for operation with shared spectrum channel access;

-	harq-ProcID-Offset2: offset of HARQ process for configured grant not configured with cg-RetransmissionTimer;

-	timeReferenceSFN: SFN used for determination of the offset of a resource in time domain. The UE uses the closest SFN with the indicated number preceding the reception of the configured grant configuration.

RRC configures the following parameters when the configured grant Type 2 is configured:

-	cs-RNTI: CS-RNTI for activation, deactivation, and retransmission;

-	periodicity: periodicity of the configured grant Type 2;

-	nrofHARQ-Processes: the number of HARQ processes for configured grant;

-	harq-ProcID-Offset: offset of HARQ process for configured grant configured with cg-RetransmissionTimer for operation with shared spectrum channel access;

-	harq-ProcID-Offset2: offset of HARQ process for configured grant not configured with cg-RetransmissionTimer.

RRC configures the following parameter when retransmissions on configured uplink grant is configured:

-	cg-RetransmissionTimer: the duration after a configured grant (re)transmission of a HARQ process when the UE shall not autonomously retransmit that HARQ process.

RRC configures the following parameter when retransmissions on configured grant Type 1 for CG-SDT is configured:

-	cg-SDT-RetransmissionTimer: the duration after a configured grant (re)transmission of a HARQ process for the initial transmission of CG-SDT with CCCH message when the UE shall not autonomously retransmit that HARQ process.

Upon configuration of a configured grant Type 1 for a BWP of a Serving Cell by upper layers, the MAC entity shall:

1>	store the uplink grant provided by upper layers as a configured uplink grant for the indicated BWP of the Serving Cell;

1>	initialise or re-initialise the configured uplink grant to start in the symbol according to timeDomainOffset, timeReferenceSFN, and S (derived from SLIV or provided by startSymbol as specified in TS 38.214 [7]), and to reoccur with periodicity.

After an uplink grant is configured for a configured grant Type 1, the MAC entity shall consider sequentially that the Nth (N >= 0) uplink grant occurs in the symbol for which:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)
	+ (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
	(timeReferenceSFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot
	+ timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity)
	modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)

For an uplink grant configured for configured grant Type 1 for CG-SDT on the selected uplink carrier as in clause 5.27, when CG-SDT is triggered and not terminated, for each configured uplink grant valid according to TS 38.214 [7] for which the above formula is satisfied, the MAC entity shall:

1>	if, after initial transmission for CG-SDT with CCCH message has been performed according to clause 5.4.1, PDCCH addressed to the MAC entity's C-RNTI has not been received, and the SSB corresponding to the configured UL grant has the same SSB index as the SSB selected for initial transmission for CG-SDT with CCCH message (i.e., retransmission of initial transmission of CG-SDT):

2>	indicate the SSB index corresponding to the configured uplink grant to the lower layer;

2>	consider this configured uplink grant as valid.

1>	else if at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB is available:

2>	if at least one SSB corresponding to the configured uplink grant with SS-RSRP above the cg-SDT-RSRP-ThresholdSSB is available:

3>	if this is the initial transmission of CG-SDT with CCCH message after the CG-SDT procedure is initiated as in clause 5.27 (i.e., initial transmission for CG-SDT):

4>	select an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB amongst the SSB(s) associated with the configured grant.

3>	else if PDCCH addressed to C-RNTI has been received after the initial transmission of CG-SDT with CCCH message (i.e., subsequent new transmission for CG-SDT):

4>	if SS-RSRP of the SSB selected for the previous transmission for CG-SDT is above cg-SDT-RSRP-ThresholdSSB and this SSB is associated with this configured grant:

5>	select this SSB.

4>	else:

5>	select an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB amongst the SSB(s) associated with the configured grant.

3>	indicate the SSB index to the lower layer;

3>	consider this configured uplink grant as valid.

1>	else if PDCCH addressed to C-RNTI after the initial transmission of the CG-SDT with CCCH message has been received:

2>	initiate Random Access procedure in clause 5.1.

After an uplink grant is configured for a configured grant Type 2, the MAC entity shall consider sequentially that the Nth (N >= 0) uplink grant occurs in the symbol for which:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)
	+ (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
	[(SFNstart time × numberOfSlotsPerFrame × numberOfSymbolsPerSlot
	+ slotstart time × numberOfSymbolsPerSlot + symbolstart time) + N × periodicity]
	modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)

where SFNstart time, slotstart time, and symbolstart time are the SFN, slot, and symbol, respectively, of the first transmission opportunity of PUSCH where the configured uplink grant was (re-)initialised.

If cg-nrofPUSCH-InSlot or cg-nrofSlots is configured for a configured grant Type 1 or Type 2, the MAC entity shall consider the uplink grants occur in those additional PUSCH allocations as specified in clause 6.1.2.3 of TS 38.214 [7].

NOTE:	In case of unaligned SFN across carriers in a cell group, the SFN of the concerned Serving Cell is used to calculate the occurrences of configured uplink grants.

When the configured uplink grant is released by upper layers, all the corresponding configurations shall be released and all corresponding uplink grants shall be cleared.

The MAC entity shall:

1>	if at least one configured uplink grant confirmation has been triggered and not cancelled; and

1>	if the MAC entity has UL resources allocated for new transmission:

2>	if, in this MAC entity, at least one configured uplink grant is configured by configuredGrantConfigToAddModList:

3>	instruct the Multiplexing and Assembly procedure to generate a Multiple Entry Configured Grant Confirmation MAC CE as defined in clause 6.1.3.31.

2>	else:

3>	instruct the Multiplexing and Assembly procedure to generate a Configured Grant Confirmation MAC CE as defined in clause 6.1.3.7.

2>	cancel all triggered configured uplink grant confirmation(s).

For a configured grant Type 2, the MAC entity shall clear the configured uplink grant(s) immediately after first transmission of Configured Grant Confirmation MAC CE or Multiple Entry Configured Grant Confirmation MAC CE which confirms the configured uplink grant deactivation.

Retransmissions use:

-	repetition of configured uplink grants; or

-	received uplink grants addressed to CS-RNTI; or

-	configured uplink grants with cg-RetransmissionTimer configured; or

-	configured uplink grants with cg-SDT-RetransmissionTimer configured.

END OF THE CHANGE