3GPP RAN WG2 Meeting #119-e	R2-2208277

eMeeting August 17th – 26th, 2022

Agenda Item:	8.7.4

Source:	InterDigital

Title:	RRC Idle/Inactive measurement, mobility, and service continuity

Document for:	Discussion, Decision

# Introduction

Adaptations to NR mobility must address the following issues in NTN: 1) increased signalling latency due to large propagation delay; 2) frequent cell change due to satellite movement; and 3) lower robustness of measurement-based mobility due to reduced signal strength variation between cell center and cell edge and large regions of cell overlap. Rel-17 NR NTN partially addressed these issues via enhancements to conditional handover, measurement reporting, and measurement rules for cell (re)selection.

An objective for Rel-18 NR NTN is to continue enhancements for both NTN-NTN and NTN-TN mobility and service continuity. Work will consider existing methods from NR TN and Rel-17 NR NTN as baseline for further enhancements, and will target the following specific objectives:

- Specify NTN-TN and NTN-NTN measurement/mobility and service continuity enhancements [RAN2,RAN3,RAN4]

  - For NTN-NTN mobility, specify cell reselection enhancements for earth moving cell, the timing based and location-based cell reselection for quasi-earth fixed cell in Rel-17 can be considered as the starting point. [RAN2, RAN3, RAN4]

  - Specify NTN-NTN handover enhancement for RRC_CONNECTED UEs in the quasi-earth-fixed cell and earth-moving cell to reduce the signalling overhead. [RAN2, RAN3]

  - Specify cell reselection enhancements for RRC_IDLE/INACTIVE UEs to reduce UE power consumption (NTN-TN mobility is prioritized). [RAN2, RAN3, RAN4]

  - Study and, if needed, specify enhancement to Xn[/NG] signalling to support feeder link switch-over, CHO, e.g. exchange of necessary information between gNBs. [RAN3]

This document focuses on the RRC_IDLE/INACTIVE case (i.e. bullets 2 and 4 above). First an overview of non-terrestrial networks is provided, describing the supported deployment scenarios and Rel-17 enhancements for cell reselection. Possible enhancements for both the NTN-NTN and NTN-TN case are then proposed.

# Release-17 non-terrestrial networks

## NTN deployment scenarios

Rel-17 NTN deployment scenarios are generally classified by three characteristics: 1) the satellite payload configuration; 2) the satellite orbit; and 3) the service link type. Currently NR only supports a transparent payload configuration, where an NTN payload (satellite) forwards the radio protocol received from the UE (via the service link) to the NTN gateway (via the feeder link) and vice-versa.

Two orbital classfications are supported: Geosynchronous orbit (GSO) and Non-Geosynchronous orbit (NGSO). NGSO orbits include Low Earth Orbit at altitude between approximately 300 km and 1500 km, and Medium Earth Orbit at altitude between approximately 7000 km and 25000 km. GSO altitutes are relatively fixed at approximately 35786km. Round trip propagation delays range from approximately 25.77ms for NGSO at 600km, to 541.46ms for GSO.

Three types of service links are supported:

- Earth-fixed: provisioned by beam(s) continuously covering the same geographical areas all the time;

- Quasi-Earth-fixed: provisioned by beam(s) covering one geographic area for a limited period and a different geographic area during another period;

- Earth-moving: provisioned by beam(s) whose coverage area slides over the Earth surface.

For NGSO satellites, the gNB can provide either quasi-Earth-fixed cell coverage or Earth-moving cell coverage, while gNB operating with GSO satellite can provide Earth fixed cell coverage. Typical beam footprint diameters for NGSO deployments range in diameter from 100 – 1000 km, and GEO deployments range from 200 – 3500 km diameter.

## Rel-17 IDLE/INACTIVE enhancements

Rel-17 enhancements to IDLE/INACTIVE cell reselection focus primarly on new measurement rules. Two key enhancements have been specified based on UE distance from a cell reference point, and on the time a quasi-Earth cell will stop serving the current area, indicated by t-Service. The cell reference point, t-Service and distanceThresh (the parameter used to evaluate the distance condition) are optionally broadcast in SIB19, a new system information block which carriers NTN-specific information.

The location-based enhancement enables measurement relaxation when a UE is located within a threshold distanceThresh from a cell reference point (usually the cell center). A UE may not perform intra-frequency measurements, measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority if all following conditions are satisfied:

- The serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ;

- The UE has valid UE location information (it’s up to UE implementation to have available UE location information); and

- the distance between the UE and the serving cell reference location is shorter than distanceThresh.

The time-based enhancement requires a UE to perform cell re-selection measurements in a quasi-earth fixed cell at some time prior to t-Service (the exact timing is up to UE implementation). The UE shall perform intra-frequency, inter-frequency, or inter-RAT measurements before t-Service regardless of the distance between the UE and serving cell measurement or whether the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, or Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ.

These new distance and time-based measurement rules do not affect measurement of higher-priority NR inter-frequency or inter-RAT frequencys. The UE shall perform these measurements according to [2] regardless of remaining service time or distance from the cell reference point.

# Release-18 enhancements

## Application of Rel-17 enhancements to earth-moving cells

Release 17 cell (re)selection enhancements only apply to quasi-earth fixed cells, and are disabled for earth moving cells (e.g. by not broadcasting relevant parameters like t-Service and distanceThresh in SIB19). As per the WID, these enhancements are to be taken as a starting point for Rel-18 work on earth-moving cells.

Unlike the quasi-earth fixed case, the cell stop time in the earth moving case depends on the UE location within the cell and direction/speed of cell movement. Broadcasting a singular t-Service value would not work, and would instead need to be done in a UE-specific manner.

Observation 1:	Broadcasting a singular t-Service does not work for earth moving beams as the cell stop time depends on the UE location within the cell and direction/speed of cell movement

A UE-specific t-Service would require dedicated signaling and periodic updates as the cell/UE move, so is unsuitable for IDLE/INACTIVE UEs. It is therefore proposed that RAN2 prioritize location-based cell reselection solutions for earth-moving beams.

Proposal 1:	Prioritize location-based cell (re)selection enhancements for earth-moving cells.

One issue when adapting the existing location-based cell reselection enhancement to earth-moving cells is that the cell reference point used for evaluation of distanceThresh will continuously change. Although the broadcasted value could be periodically updated, having the UE frequently re-aquire SIB19 is undesirable from a power consumption perspective.

Observation 2:	The cell reference point used for Rel-17 location-based cell reselection enhancements will continuously change for earth-moving cells. This may increase power consumption if the UE must frequently re-aquire SIB19 to updated the value.

RAN2 should study ways to ensure the cell reference point remains valid without frequent SIB re-acquisition. For example, the UE may use satellite ephemeris data to help predict future values of the cell reference point.

Proposal 2:	FFS how to update/maintain an accurate cell reference point for earth-moving cells.

## Location-based cell (re)selection enhancements

RAN2 has also discussed other candidate location-based cell reselection enhancements in Rel-17 such as limiting the number of candidate cells to only those with a reference point within a certain distance from the UE. The following options were discussed, but ultimately not agreed due to limited time left in the release:

- Option 1: Introduce a distance threshold. Cell ranked on R-criterion first and then the distance threshold applies to down scope the candidate cells for reselection.

- Option 2: Introduce a distance threshold. Distance threshold applies to decide the candidate cells and then rank the candidate cells based on R-criterion to decide the target cell for reselection.

- Option 3: Cell ranked on R-criterion first and then the distance criteria applies to decide the target cell for reselection

Although helpful to limit the number of candidate cells in the quasi-earth fixed case, such solutions would be particularly useful for earth-moving cells. For example, as the cells move the distance between the UE and neighbouring cells could be tracked over time (or combined with ephemeris information) to detect whether a cell is approaching or moving away from the UE. This could help reduce the number of cell reselections by having the UE prioritize incoming cells, or ensure a UE does not reselect to a neighbouring cell which will shortly disappear due to being too far away.

Observation 3:	The distance between a UE and neighbour cell reference point(s) may be tracked over time to determine whether a cell is incoming or receding in an earth moving scenario.

Since broadcasting neighbour cell assistance information (e.g. ephemeris, validity duration) via SIB19 is already supported, only minor system changes are required to include the neighbour cell reference point.

Proposal 3:	Cell reselection enhancements based on the distance between a UE and neighbouring cell reference point(s) are supported in Rel-18.

## NTN-TN cell (re)selection

Terrestrial and non-terrestrial cells offer different advantages based on their respective characteristics. For example, the large cell sizes in NTN may be beneficial to reduce the number of cell reselections or RNA updates. NTN cells are also beneficial for paging since one NTN cell may cover the same geographic area as many TN cells. Alternatively, terrestrial cells offer latency advantages and are better suited for fast connection setup or paging response.

Observation 4:	Camping on NTN vs TN cells offer different advantages: TN has faster connection setup, whereas NTN may reduce the number of cell reselections due to larger cells sizes.

Since a given network type may be better suited to a particular scenario, UE may prefer to camp on a particular network type based on UE-circumstances (e.g. UE mobility or traffic profile). So far no enhancements are specified regarding prioritization between terrestrial and non-terrestrial cells, so the UE must rely on measurement-based cell ranking regardless of network type. Including other non-measurement-based metrics into the cell reselection procedure such as the network type (TN vs. NTN) would assist the UE to select the best cell under the given circumstances.

Proposal 4:	Non-measurement-based factors (e.g. network type) are incorporated into the cell reselection procedure to assist TN/NTN prioritization.

However, such solutions are only useful if the UE is under both NTN and TN coverage simultaneously. In case a UE is far outside of terrestrial coverage, it would be beneficial from a power-saving perspective to disable or reduce searching for terrestrial cells.

Observation 5:	If an NTN-camped UE is far outside of terrestrial coverage, it would be beneficial to disable or reduce searching for terrestrial cells during cell reselection.

There are two key issues when considering such a solution: 1) How the UE can detect it is out of/within terrestrial coverage; and 2) How the UE will reduce or eliminate terrestrial cells from consideration. When discussing candidate solutions for the first issue, RAN2 should consider UE power consumption and detection accuracy the primary evaluation metrics.

Proposal 5:	FFS how an NTN-camped UE can detect it is within terrestrial coverage (e.g. when NTN network type is prioritized).

When evaluating candidate solutions for the second issue, RAN2 should consider how such solutions impact terrestrial cells belonging to high-priority frequencies or RATs, and how long it will take to resume terrestrial cell evaluation when the UE is back within TN coverage.

Proposal 6:	FFS how to minimize terrestrial cell search when a UE is out of terrestrial coverage.

## Discontinuous coverage

Discontinuous coverage in NTN refers to temporary and predictable coverage gaps caused by non-continuous coverage in NGSO deployments. Although not an issue if continuous coverage is available globally, this may not be the case in early NTN deployments or in deeply rural areas. Unlike IoT NTN, NR NTN has not addressed the discontinuous coverage scenario in Release 17.

Based on the current specification, if a coverage gap occurs the UE will have no idea it was due to discontinuous coverage, causing additional power consumption due to e.g. unnecessary cell search. Due to deterministic satellite movement, IoT NTN has introduced additional assistance information (e.g. satellite ephemeris and coverage parameters such footprint radius, cell reference points or elevation angles) to predict the duration of a coverage gap. Once the UE has determined it is out of coverage, the UE may suspend IDLE mode tasks and adapt timers to decrease UE power consumption during the coverage gap.

Observation 6:	Allowing the UE to suspend idle mode tasks (e.g. measurements) during a discontinuous coverage gap will improve power saving performance.

Similar solutions can be adopted in NR NTN to improve idle-mode power consumption. Work done in Rel-17 IoT NTN may be taken as baseline.

Proposal 7:	Discontinuous coverage scenario is included in Rel-18 NR NTN service continuity work.

# Conclusion

In this contribution the following observations and proposals were made concerning Idle/Inactive measurement, mobility, and service continuity enhancements in Rel-18 NR NTN:

Observation 1:	Broadcasting a singular t-Service does not work for earth moving beams as the cell stop time depends on the UE location within the cell and direction/speed of cell movement

Observation 2:	The cell reference point used for Rel-17 location-based cell reselection enhancements will continuously change for earth-moving cells. This may increase power consumption if the UE must frequently re-aquire SIB19 to updated the value.

Observation 3:	The distance between a UE and neighbour cell reference point(s) may be tracked over time to determine whether a cell is incoming or receding in an earth moving scenario.

Observation 4:	Camping on NTN vs TN cells offer different advantages: TN has faster connection setup, whereas NTN may reduce the number of cell reselections due to larger cells sizes.

Observation 5:	If an NTN-camped UE is far outside of terrestrial coverage, it would be beneficial to disable or reduce searching for terrestrial cells during cell reselection.

Observation 6:	Allowing the UE to suspend AS procedures (e.g. measurements) during a discontinuous coverage gap will improve power saving performance.

Proposal 1:	Prioritize location-based cell (re)selection enhancements for earth-moving cells.

Proposal 2:	FFS how to update/maintain an accurate cell reference point for earth-moving cells.

Proposal 3:	Cell reselection enhancements based on the distance between a UE and neighbouring cell reference point(s) are supported in Rel-18.

Proposal 4:	Non-measurement-based factors (e.g. network type) are incorporated into the cell reselection procedure to assist TN/NTN prioritization.

Proposal 5:	FFS how an NTN-camped UE can detect it is within terrestrial coverage (e.g. when NTN network type is prioritized).

Proposal 6:	FFS how to minimize terrestrial cell search when a UE is out of terrestrial coverage.

Proposal 7:	Discontinuous coverage scenario is included in Rel-18 NR NTN service continuity work.

# References

RP-221819 – 3GPP Work Item Description: NR NTN enhancements (Rel-18)

3GPP TS 38.133: Requirements for support of radio resource management v17.6.0

3GPP TS 38.300: NR and NG-RAN overall description; Stage 2 v17.1.0

3GPP TS 38.304: User Equipment (UE) procedures in idle mode and in RRC Inactive state v17.1.0

3GPP TS 38.331: Radio Resource Control (RRC) protocol specification v17.1.0