3GPP TSG-RAN2 Meeting #119-e	TDoc R2-2208305

# Electronic meeting, August 17th - 26th 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.304 | CR | 0852 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of value infinity for coverage based paging carrier hysteresis timer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh4_LTE_eMTC6-Core |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The impact on paging strategy can be reduced if it would be possible for the network to configure the coverage based paging carrier hysteresis timer with the value “infinity” in one direction, i.e., to avoid that the UE switches from a legacy paging carrier to a coverage-based paging carrier. The hysteresis timer is configured as it is currently specified before the UE is released to idle mode, but once the UE switches to a legacy (non-coverage based) paging carrier due to a change in the serving cell NRSRP, it stays there, i.e., no further switching, if the UE can be configured with the timer value “infinity” in one direction. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR clarifies that WUS groups are first to be incremented within a WUS resource and the WUS resource is incremented after. Impact analysis Impacted functionality: Configuration of coverage based paging carrier hysteresis timer for NB-IoT UEs Inter-operability: No impact on the UE or the network is expected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The timer is configured by the network, but it is not possible for the network to keep track of the paging carrier the UE uses once the timer expires after the UE is released to idle mode. This will have an impact on the paging strategy since the network would not be able to know whether the UE uses a coverage based or a non-coverage based paging carrier at the time for paging. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS 36.331 CR YYYY |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 7.7	Coverage based paging

Coverage-based paging carrier selection is only used in the cell in which the UE most recently entered RRC-IDLE triggered by:

-	reception of RRCEarlyDataComplete or RRCConnectionRelease;

-	and the message includes cbp-Index.

Coverage-based paging is enabled when at least one DL carrier in dl-ConfigList is configured with cbp-Index.

When coverage-based paging is used, the UE shall:

-	if cbp-HystTimer is not running:

-	if Srxlev > nrsrpMin in cbp-ConfigList indexed by value of cbp-Index:

-	use the list of carriers in dl-ConfigList configured with pcch-Config-r17 and with cbp-Index equal to the value of cbp-Config for carrier selection as described in clause 7.1.

-	use the nB and ue-SpecificDRX-CycleMin configured in the entry of cbp-ConfigList corresponding to the value of cbp-Index.

-	else:

-	use the list of carriers in dl-ConfigList configured with pcch-Config-r14 for carrier selection as described in clause 7.1.

-	if cbp-HystInfinity is set:

-	set cbp-HystTimer to infinity

-	else:

-	continue using list of DL carriers previously selected for carrier selection as described in clause 7.1.

-	when UE switches between paging carriers configured with pcch-Config-r14 and paging carriers configured with pcch-Config-r17 for carrier selection:

-	start cbp-HystTimer;