3GPP TSG-RAN WG2 Meeting #119-e	R2-2208326

Online, Aug 17 – Aug 26, 2022

Agenda Item		:	8.4.2.2

Source		:	LG Electronics Inc.

Title		:	General aspects of L1L2 based inter-cell mobility

Document for		:	Discussion

1.	Introduction

In Rel-18, L1/L2 based inter-cell mobility is one of the important features for further NR mobility enhancements, which is introduced in order to reduce latency, overhead and interruption time [1]. In this paper, we discuss general aspects of the L1/L2 based inter-cell mobility.

2.	Discussion

One of the main objectives of this WI is to reduce the mobility latency, signalling overhead, and interruption time by introducing a new concept of the candidate serving cell and by lowering the layer that commands the mobility execution. If L1/L2 signal is used to transmit the execution command, the RRC configuration on the candidate serving cell should be provided to UE in advance of the mobility execution as in conditional mobility procedures. Therefore, the following can be considered as a baseline of the L1/L2 based mobility:

Proposal 1. RAN2 considers the following procedure as a baseline for L1/L2 based mobility:

- Step 1. UE receives the configuration on a candidate serving cell.

- Step 2. UE receives a mobility command from network via L1/L2 signalling.

- Step 3. UE switches the serving cell to the candidate serving cell.

- Step 4. UE indicates the mobility completion to the NW.

In the legacy reconfiguration with sync, UE initiates a random access procedure to a target cell upon receiving mobility command from network. During the RA, UE obtains the Timing Advance (TA) for the target cell and selects the beam for downlink and uplink.

In [2], we show that the random access procedure takes a significant part of the mobility latency. If UE can skip RA procedure during mobility execution, e.g. by performing an early RA (i.e., RA prior to the mobility execution using available idle periods in the source cell), the mobility latency could be considerably reduced.

If UE performs TA maintenance and beam management of the candidate serving cell by performing such an early RA, the UE can immediately access to the target candidate serving cell using the TA and maintained beams upon receiving the execution command without RA.

Proposal 2. RAN2 considers the maintenance of TA of the candidate serving cell before mobility execution.

Proposal 3. RAN2 considers the beam management of the candidate serving cell before mobility execution.

Maintenance of TA and beam status of a candidate serving cell imposes several considerations. First, the validity of the TA obtained by the early RA may expire before the mobility execution, and the beams selected during the early RA may not be the best beam when the mobility is executed. Therefore, it may be desirable to bound the time gap between mobility execution based on the maintained TA and beam and the time UE acquired the TA and beam information. Second, the maintenance of TA and beam status of candidate serving cells should not impose prohibitive complexity to UEs in particular when more than one candidate serving cell is configured. Given the considerations, RAN2 needs to discuss how to reduce the UE efforts for the beam management and the TA maintenance for candidate serving cells, to achieve the RACH-less mobility using the candidate serving cell.

Since the full configuration on the mobility target cell is provided in advance, if the execution condition is configured as in the CHO, UE could perform the switching autonomously among the candidate serving cells. However, the benefit of using the candidate serving cell in mobility is that the latency of the network command based mobility can be reduced by using L1/L2 signal to command the mobility execution. Even if the autonomous switching among the candidate serving cells is supported, it could not provide the better mobility performance than the existing conditional mobility, but it would impose further works on RAN2. Therefore, unless it is shown that the autonomous switching using the candidate serving cell has the better mobility performance, it should not be supported.

Proposal 4. Do not support UE autonomous mobility among the candidate serving cells based on the pre-configured execution condition.

Unlike RRC message, since the L1/L2 signal is neither ciphered nor integrity-protected, the concern about the security of the mobility command could be raised. However, L2 signal is already used in very similar case, i.e. SCell activation/deactivation. Once RRC provides the configurations on SCell, the SCell can be activated or deactivated via MAC CE. The L1/L2 mobility command and the SCell Activation/Deactivation MAC CE are very similar in the respect that they are used just to order to apply the configurations provided via RRC message, which are ciphered and integrity-protected. Therefore, RAN2 doesn’t need to have concern about the security to using L1/L2 signal as an execution command in the mobility using the candidate serving cell.

Proposal 5. No enhancements to ensure security of L1/L2 signalling for mobility are pursued.

If UE already has TA of the candidate serving cell and beam information applicable for the candidate cell upon mobility, the UE can transmit the Scheduling Request to the target cell immediately completion of the L1/L2 based mobility. Furthermore, if the network can provide the UL grant applicable to the target cell via the L1/L2 mobility command, the UE could perform the PUSCH transmission to the target cell even without transmitting SR. In dynamic switching scenario, the latency reduction is important for downlink as well as for uplink. Therefore, RAN2 also needs to discuss how to reduce the latency of the uplink transmission using the candidate serving cell.

Proposal 6. RAN2 needs to discuss how to reduce the latency of the uplink transmission for L1/L2 mobility.

In the existing inter-cell mobility, when the mobility is successfully completed, the UE releases the configurations on the source cell and stops maintaining the TA. If another mobility happens to the previous serving cell, the network should re-provide the configurations on the previous serving cell, and UE should perform random access to the previous serving cell to re-obtain the TA.

If UE keeps maintaining the TA and the configurations of the previous serving cell, the previous serving cell could be a good candidate serving cell, especially, in the dynamic switch scenario among candidate serving cells. The dynamic switching using candidate serving cells could be useful even when the serving cell quality doesn’t become bad, e.g. in cell-ON/OFF scenario for network energy saving or for load balancing among candidate serving cells. In such scenarios, the switching to the previous serving cell would frequently occur, and it is advantageous for UE to keep the previous serving cell as a candidate serving cell after successful mobility execution. Therefore, it is proposed to discuss how to handle the previous serving cell after inter-cell mobility using the candidate serving cell.

Proposal 7. RAN2 considers keeping the previous serving cell as a new candidate serving cell after L1/L2 mobility based on a candidate cell

3.	Conclusion

In this paper, we have made the following proposals:

Proposal 1. RAN2 considers the following procedure as a baseline for L1/L2 based mobility:

- Step 1. UE receives the configuration on a candidate serving cell.

- Step 2. UE receives a mobility command from network via L1/L2 signalling.

- Step 3. UE switches the serving cell to the candidate serving cell.

- Step 4. UE indicates the mobility completion to the NW.

Proposal 2. RAN2 considers the maintenance of TA of the candidate serving cell before mobility execution.

Proposal 3. RAN2 considers the beam management of the candidate serving cell before mobility execution.

Proposal 4. Do not support UE autonomous mobility among the candidate serving cells based on the pre-configured execution condition.

Proposal 5. No enhancements to ensure security of L1/L2 signalling for mobility are pursued.

Proposal 6. RAN2 needs to discuss how to reduce the latency of the uplink transmission for L1/L2 mobility.

Proposal 7. RAN2 considers keeping the previous serving cell as a new candidate serving cell after L1/L2 mobility based on a candidate cell.

4.	References

[1] RP-221799 Revised WID on Further NR mobility enhancements, 3GPP TSG RAN Meeting #96, Budapest, Hungary, 6 – 9 June, 2022

[2] R2-2208522 L1/L2 mobility scenarios and latency, LG Electronics Inc.