3GPP TSG-RAN WG2 Meeting #119 electronic	R2-2208352

Online, Aug 17-29, 2022

Agenda Item:	5.2.3

Source: 	ASUSTeK

Title:  	Discussion on UL skipping and SL BSR

Document for:	Discussion and Decision

- Introduction

In email discussion [AT118-e][703] in the previous meeting [1], there was a discussion on a change in CR R2-2205125[2] regarding whether the UE should not trigger UL transmission for periodic SL BSR and padding BSR. Due to lack of time and need for further clarification on the scenario, the change was postponed with below agreement:

Proposal 1: The first change in R2-2205125 is postponed. RAN2 to discuss whether UL grant skipping can be supported with sidelink UE in the next meeting. … - All proposals above are agreed.

In this contribution, we discuss and clarify the scenario and re-propose the solution for the above issue.

- Discussion

Whether UL grant skipping can be supported with sidelink UE

In the current specification, if the MAC entity is configured with uplink grant skipping, periodic BSR with no data and padding BSR would not trigger any UL transmission. In the previous CR [2] proposed that similar principle should also apply to periodic SL-BSR, SL data and padding SL-BSR with proposed change below:

The MAC entity shall: 1> if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and/or the periodic SL-BSR and there is no data and/or SL data available for any LCG, or the MAC PDU includes only the padding BSR and/or the padding SL-BSR: 3> not generate a MAC PDU for the HARQ entity.

As raised by other companies in the email discussion [1], the first thing that need to be discussed is that whether UL skipping can be supported with sidelink UE (i.e. enhancedSkipUplinkTxDynamic set to true). In our view, since it was not discussed if uplink skipping can be configured with SL, we assume that both can be supported naturally at the same time. In addition, if UL skipping is not supported with sidelink UE, the UE will transmit unnecessary MAC PDU (e.g. MAC PDU with only padding or periodic SL-BSR when there’s no SL data available). It is beneficial to enable UL skipping for sidelink UE to reduce power consumption.

On the other hand, if UL skipping is not allowed to be enabled in SL UE, a clarification of such limitation should be made clear in order to avoid further confusion and discussion in the future.

Proposal 1A: 	RAN2 confirm UL skipping can be supported with sidelink UE.

Proposal 1B: 	RAN2 conclude that UL skipping is not allowed to be enabled in sidelink UE and capture the conclusion in meeting minutes.

Whether SL data should be applied when considering whether there is no data available in UL skipping

Another issue raised by companies in the email discussion is that whether UL skipping should consider SL data available. In our view, while SL data is not transmitted on Uu, for the purpose of periodic SL-BSR, if there is SL data available for transmission, the UE should trigger UL transmission for transmitting periodic BSR. In other words, the UE is allowed to skip the UL transmission for periodic SL-BSR when there are no SL data available so that periodic SL-BSR can be omitted.

In conclusion, we propose the text proposal in an accompanied CR for clarification on UL skipping and SL BSR operation to avoid unnecessary UL transmission when UL skipping is supported for SL UE.

Proposal 2: 	If UL skipping can be supported with sidelink UE, adopt the changes in R2-2208353 and R2-2208354.

- Conclusion

We have the following proposals for SL BSR and UL skipping in NR V2X:

Proposal 1A: 	RAN2 confirm UL skipping can be supported with sidelink UE.

Proposal 1B: 	RAN2 conclude that UL skipping is not allowed to be enabled in sidelink UE and capture the conclusion in meeting minutes.

Proposal 2: 	If UL skipping can be supported with sidelink UE, adopt the changes in R2-2208353 and R2-2208354.

- Reference

[1] R2-2206295	[AT118-e][703][V2X/SL] MAC corrections (ASUSTeK)	ASUSTeK

[2] R2-2205125	Corrections on SL configured grant and SL BSR	ASUSTeK