3GPP TSG-RAN WG2 Meeting #119 electronic	R2-2208361

Online, August 17 – 29, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.351 | CR | 0008 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SRAP data PDU discard examination |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ASUSTeK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_Relay-Core |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | gNB may initiate local UE ID update with the remote UE and the relay UE for assigning a new sl-LocalIdentity of the remote UE. However, the remote UE may have generated SRAP data PDUs with the old sl-LocalIdentity and these SRAP data PDUs to be sent to the relay UE may be still buffered in lower layers of the remote UE. Similarly, the relay UE may receive some SRAP data PDUs with the old sl-LocalIdentity of the remote UE from the remote UE and these SRAP data PDUs to be sent to gNB may be still buffered in lower layers of the relay UE. If these SRAP data PDUs with the old sl-LocalIdentity of the remote UE are buffered in the remote UE and then sent to the relay UE, the relay UE will discard these SRAP data PDUs based on the current discard examination since the relay UE just recognizes the new sl-LocalIdentity for the remote UE. This situation causes SL resource waste unnecessarily. Similarly, if the SRAP data PDUs with the old sl-LocalIdentity of the remote UE are buffered in the relay UE and then sent to gNB, gNB would discard these SRAP data PDUs since gNB would not recognize the sl-LocalIdentity contained in the SRAP data PDUs. This situation also causes Uu resource waste. Therefore, it is beneficial for the remote UE or the relay UE not to send these SRAP data PDUs with the old sl-LocalIdentity when the new sl-LocalIdentity for the remote UE is allocated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 5.4, for both U2N Remote UE and U2N Relay UE, when receiving a new sl-LocalIdentity for the U2N Remote UE from RRC, the SRAP entity shall inform the lower layers to discard the SRAP Data PDU(s) with SRAP header set to the old sl-LocalIdentity of the U2N Remote UE. Impact analysis Impacted functionality(ies): Handling of unknown, unforeseen, and erroneous protocol data. Inter-operability: - If the UE is implemented according to this CR but the network is not, there is no inter-operability issue foreseen. - If the network is implemented according to this CR but the UE is not, there is no inter-operability issue foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SL/Uu resources for transmissions of the SRAP data PDUs with the old sl-LocalIdentity for the remote UE are wasted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * * *

## 5.4	Handling of unknown, unforeseen, and erroneous protocol data

For U2N Remote UE, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which is not included in sl-SRAP-Config-Remote is received, the SRAP entity shall:

-	discard the received SRAP Data PDU.

For U2N Relay UE, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which is not included in sl-SRAP-Config-Relay is received except that the SRAP Data PDU from SL-RLC1 as specified in TS 38.331 [3] is the first SRAP Data PDU received from a U2N Remote UE, or when a SRAP Data PDU that contains a UE ID which does not match the concerned sl-LocalIdentity corresponding to sl-L2Identity-Remote of the ingress link is received by U2N Relay UE, the SRAP entity shall:

-	discard the received SRAP Data PDU.

For U2N Remote UE or U2N Relay UE, when receiving a new sl-LocalIdentity for the U2N Remote UE from RRC, the SRAP entity shall:

-	inform the lower layers to discard the SRAP Data PDU(s) with SRAP header set to the old sl-LocalIdentity of the U2N Remote UE.

* * * End of Changes * * * *