3GPP TSG-RAN WG2 Meeting #119-e                                  R2-2208369

Online, August 17-29, 2022

Agenda item:		6.3.3

Source:				Samsung

Title:					Further discussion on re-establishment handling while T346g timer is running

Document for:		Discussion & Decision

1	Introduction

This contribution discusses whether to trigger re-establishment procedure while T346g timer is running, and whether to stop it if triggered.

2	Discussion

In [1] it is specified that UE releases musim-LeaveAssistanceConfig if configured upon initiation of re-establishment procedure. It is observed that the UE does NOT stop timer T346g, if running

5.3.7.2	Initiation

<Only relevant procedure texts are copied below>

2>	release musim-LeaveAssistanceConfig, if configured;

Observation 1: Upon initiation of re-establishment procedure, the UE does NOT stop timer T346g, if running.

Before delving into the details, it would be good to confirm whether T346g timer can be running upon initiation of re-establishment procedure. At RAN2#117-e meeting, the following agreement was made:

RAN2 will not specify any new behaviour if the wait timer for switching notification to leave RRC connected state is running, and UE detects RLF, triggers re-establishment, receives HO command or triggers CHO. No specification changes are needed.

Our understanding on yellow-marked agreement is that we keep to following existing UE behavior. Since detection of RLF is independent of any configured wait timer (e.g. prohibit timer), it ends up with NOT specifying anything in clause 5.3.10 (radio link failure related actions). In other words, UE detects RLF as in legacy while T346g timer is running and triggers re-establishment if RLF is detected.

Observation 2: T346g timer can be running during re-establishment procedure.

If UE does not stop running timer T346g during re-establishment procedure, there may be an RRC state mismatch i.e. the UE may successfully fallback to RRC establishment during re-establishment procedure and enter RRC_IDLE autonomously upon T346g expiry. From a network point of view, it is not a good idea to leave the RRC state mismatch problem open regardless of whether the concerned scenario is rare or not.

Observation 3: RRC state mismatch may occur if UE does not stop running timer T346g during re-establishment procedure.

During previous RAN2 meetings, some companies argued that the stopping condition of timer T346g is not needed by pointing the green-marked agreement out. But we think such argumentation is not valid considering that current procedure text already specifies legacy UE behavior on handling of timer T346g in other scenarios i.e.

5.3.5.3	Reception of an RRCReconfiguration by the UE

<Only relevant procedure texts are copied below>

2>	if reconfigurationWithSync was included in masterCellGroup or secondaryCellGroup:

3>	if the UE initiated transmission of a UEAssistanceInformation message for the corresponding cell group during the last 1 second, and the UE is still configured to provide the concerned UE assistance information for the corresponding cell group; or

3>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution, and the UE is configured to provide UE assistance information for the corresponding cell group, and the UE has initiated transmission of a UEAssistanceInformation message for the corresponding cell group since it was configured to do so in accordance with 5.7.4.2:

4>	initiate transmission of a UEAssistanceInformation message for the corresponding cell group in accordance with clause 5.7.4.3 to provide the concerned UE assistance information;

4>	start or restart the prohibit timer (if exists) or the leave without response timer for the MUSIM associated with the concerned UE assistance information with the timer value set to the value in corresponding configuration;

Observation 4: Upon reception of HO command or execution of CHO, the UE starts/restarts/stops timer T346g as in legacy, as described in clause 5.3.5.3.

Some companies prefer to have UE implementation flexibility i.e. some UEs may want to enter RRC_IDLE during/after re-establishment procedure without waiting network response so that MUSIM operation works properly. But it does not mean that UE is allowed to enter RRC_IDLE before T346g expiry, if T346g timer is running. Therefore, to avoid any potential negative consequences, we think that the UE stops timer T346g if running if UE triggers re-establishment.

Proposal 1: Upon initiation of re-establishment procedure, the UE stops timer T346g, if running, as in the TP in Annex A.1.

One way forward to address concerns on UE implementation flexibility is to add NOTE i.e. it is up to UE implementation whether to initiate the re-establishment procedure while timer T346g is running.

Proposal 2: RAN2 to discuss whether to add the following NOTE in clause 5.3.7.2 as follows:

NOTE: It is up to UE implementation whether to initiate the procedure while T346g timer is running.

Note that TPs for Proposals 1 and 2 are given in Annex part.

3	Conclusion

In section 2, the following observations are made:

Observation 1: Upon initiation of re-establishment procedure, the UE does NOT stop timer T346g, if running.

Observation 2: T346g timer can be running during re-establishment procedure.

Observation 3: RRC state mismatch may occur if UE does not stop running timer T346g during re-establishment procedure.

Observation 4: Upon reception of HO command or execution of CHO, the UE starts/restarts/stops timer T346g as in legacy, as described in clause 5.3.5.3.

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Proposal 1: Upon initiation of re-establishment procedure, the UE stops timer T346g, if running.

Proposal 2: RAN2 to discuss whether to add the following NOTE in clause 5.3.7.2 as follows:

NOTE: It is up to UE implementation whether to initiate the procedure while T346g timer is running.

4	Reference

[1] 3GPP TS 38.331 v17.1.0	Radio Resource Control (RRC) protocol specification

Annex A: Text Proposal

A.1	TP for Proposal 1

START OF CHANGE

5.3.7.2	Initiation

<Only relevant procedure texts are copied below>

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	stop timer T312, if running;

1>	stop timer T304, if running;

1>	start timer T311;

1>	stop timer T316, if running;

1>	if UE is not configured with attemptCondReconfig:

2>	reset MAC;

2>	release spCellConfig, if configured;

2>	suspend all RBs, and BH RLC channels for IAB-MT, and Uu Relay RLC channels for L2 U2N Relay UE, except SRB0 and broadcast MRBs;

2>	release the MCG SCell(s), if configured;

2>	if MR-DC is configured:

3>	perform MR-DC release, as specified in clause 5.3.5.10;

2>	release delayBudgetReportingConfig, if configured and stop timer T342, if running;

2>	release overheatingAssistanceConfig, if configured and stop timer T345, if running;

2>	release idc-AssistanceConfig, if configured;

2>	release btNameList, if configured;

2>	release wlanNameList, if configured;

2>	release sensorNameList, if configured;

2>	release drx-PreferenceConfig for the MCG, if configured and stop timer T346a associated with the MCG, if running;

2>	release maxBW-PreferenceConfig for the MCG, if configured and stop timer T346b associated with the MCG, if running;

2>	release maxCC-PreferenceConfig for the MCG, if configured and stop timer T346c associated with the MCG, if running;

2>	release maxMIMO-LayerPreferenceConfig for the MCG, if configured and stop timer T346d associated with the MCG, if running;

2>	release minSchedulingOffsetPreferenceConfig for the MCG, if configured stop timer T346e associated with the MCG, if running;

2>	release rlm-RelaxationReportingConfig for the MCG, if configured and stop timer T346j associated with the MCG, if running;

2>	release bfd-RelaxationReportingConfig for the MCG, if configured and stop timer T346k associated with the MCG, if running;

2>	release releasePreferenceConfig, if configured stop timer T346f, if running;

2>	release onDemandSIB-Request if configured, and stop timer T350, if running;

2>	release referenceTimePreferenceReporting, if configured;

2>	release sl-AssistanceConfigNR, if configured;

2>	release obtainCommonLocation, if configured;

2>	release musim-GapAssistanceConfig, if configured and stop timer T346h, if running;

2>	release musim-LeaveAssistanceConfig, if configured and stop timer T346g, if running;

2>	release ul-GapFR2-PreferenceConfig, if configured;

2>	release scg-DeactivationPreferenceConfig, if configured, and stop timer T346i, if running;

2>	release propDelayDiffReportConfig, if configured;

2>	release rrm-MeasRelaxationReportingConfig, if configured;

NEXT CHANGE

5.3.7.3	Actions following cell selection while T311 is running

<Only relevant procedure texts are copied below>

1>	else:

2>	if UE is configured with attemptCondReconfig:

3>	reset MAC;

3>	release spCellConfig, if configured;

3>	release the MCG SCell(s), if configured;

3>	release delayBudgetReportingConfig, if configured and stop timer T342, if running;

3>	release overheatingAssistanceConfig , if configured and stop timer T345, if running;

3>	if MR-DC is configured:

4>	perform MR-DC release, as specified in clause 5.3.5.10;

3>	release idc-AssistanceConfig, if configured;

3>	release btNameList, if configured;

3>	release wlanNameList, if configured;

3>	release sensorNameList, if configured;

3>	release drx-PreferenceConfig for the MCG, if configured and stop timer T346a associated with the MCG, if running;

3>	release maxBW-PreferenceConfig for the MCG, if configured and stop timer T346b associated with the MCG, if running;

3>	release maxCC-PreferenceConfig for the MCG, if configured and stop timer T346c associated with the MCG, if running;

3>	release maxMIMO-LayerPreferenceConfig for the MCG, if configured and stop timer T346d associated with the MCG, if running;

3>	release minSchedulingOffsetPreferenceConfig for the MCG, if configured and stop timer T346e associated with the MCG, if running;

3>	release rlm-RelaxationReportingConfig for the MCG, if configured and stop timer T346j associated with the MCG, if running;

3>	release bfd-RelaxationReportingConfig for the MCG, if configured and stop timer T346k associated with the MCG, if running;

3>	release releasePreferenceConfig, if configured and stop timer T346f, if running;

3>	release onDemandSIB-Request if configured, and stop timer T350, if running;

3>	release referenceTimePreferenceReporting, if configured;

3>	release sl-AssistanceConfigNR, if configured;

3>	release obtainCommonLocation, if configured;

3>	release scg-DeactivationPreferenceConfig, if configured, and stop timer T346i, if running;

3>	release musim-GapAssistanceConfig, if configured and stop timer T346h, if running;

3>	release musim-LeaveAssistanceConfig, if configured and stop timer T346g, if running;

3>	release propDelayDiffReportConfig, if configured;

3>	release ul-GapFR2-PreferenceConfig, if configured;

3>	release rrm-MeasRelaxationReportingConfig, if configured;

3>	suspend all RBs, and BH RLC channels for the IAB-MT, except SRB0;

END OF CHANGE

A.2	TP for Proposal 2

START OF CHANGE

5.3.7.2	Initiation

The UE initiates the procedure when one of the following conditions is met:

1>	upon detecting radio link failure of the MCG and t316 is not configured, in accordance with 5.3.10; or

1>	upon detecting radio link failure of the MCG while SCG transmission is suspended, in accordance with 5.3.10; or

1>	upon detecting radio link failure of the MCG while PSCell change or PSCell addition is ongoing, in accordance with 5.3.10; or

1>	upon detecting radio link failure of the MCG while the SCG is deactivated, in accordance with 5.3.10; or

1>	upon re-configuration with sync failure of the MCG, in accordance with clause 5.3.5.8.3; or

1>	upon mobility from NR failure, in accordance with clause 5.4.3.5; or

1>	upon integrity check failure indication from lower layers concerning SRB1 or SRB2, except if the integrity check failure is detected on the RRCReestablishment message; or

1>	upon an RRC connection reconfiguration failure, in accordance with clause 5.3.5.8.2; or

1>	upon detecting radio link failure for the SCG while MCG transmission is suspended, in accordance with clause 5.3.10.3 in NR-DC or in accordance with TS 36.331 [10] clause 5.3.11.3 in NE-DC; or

1>	upon reconfiguration with sync failure of the SCG while MCG transmission is suspended in accordance with clause 5.3.5.8.3; or

1>	upon SCG change failure while MCG transmission is suspended in accordance with TS 36.331 [10] clause 5.3.5.7a; or

1>	upon SCG configuration failure while MCG transmission is suspended in accordance with clause 5.3.5.8.2 in NR-DC or in accordance with TS 36.331 [10] clause 5.3.5.5 in NE-DC; or

1>	upon integrity check failure indication from SCG lower layers concerning SRB3 while MCG is suspended; or

1>	upon T316 expiry, in accordance with clause 5.7.3b.5; or

1>	upon detecting sidelink radio link failure by L2 U2N Remote UE in RRC_CONNECTED, in accordance with clause 5.8.9.3; or

1>	upon reception of NotificationMessageSidelink including indicationType by L2 U2N Remote UE in RRC_CONNECTED, in accordance with clause 5.8.9.10; or

1>	upon PC5 unicast link release indicated by upper layer at L2 U2N Remote UE in RRC_CONNECTED.

NOTE x: It is up to UE implementation whether to initiate the procedure while T346g timer is running.

END OF CHANGE