3GPP TSG-RAN WG2 Meeting #119-e			R2-2208370

E-meeting, 17 Aug – 26 Aug, 2022

Agenda item:	6.24.1

Source:	Huawei, HiSilicon

Title:	Discussion on the update to endorsed CRs for DC location report for more than 2CC

Document for:	Discussion

- Introduction

The baseline CR for DC location report has been endorsed in [1] in last RAN2 meeting and A LS [2] is further received from RAN4 regarding remaining issues for DC location reporting for more than 2 CC.

In this contribution, we will further discuss the update to endorsed CR based on the LS from RAN4.

- Discussion

Offset range

RAN4 has agreed the offset range as below:

The maximum offset range is ±300 MHz for FR1 and ±1200 MHz for FR2, and the granularity is the lowest subcarrier spacing used in the CA configuration.

Considering the minimum SCS (15K for FR1 and 60K for FR2) and 275 RBs limitation per cell and maximum cell bandwidth (50M for FR1/15K SCS and 200M for FR2/60K SCS), the required offset value for FR1 would be 275*12*(300/50)=19800. And we can get the same result for FR2.

Proposal 1: The offset value is update to (-19800 to 19800).

RAN4 suggests that the lowest SCS used in the CA configuration is used as the granularity for report. However, considering that the offset is signaled per CC group, it is suggest that the lowest SCS used in the reported CC group is used as the offset granularity for offset report.

Proposal 2: The lowest SCS in the reported CC group is used as the offset granularity for offset report.

Applicable scenarios for the new DC location report

In the LS, it is clarified that the scenarios where R17 DC location can be applied as below:

Applicability update

In addition to >2CC case, the R17 DC location reporting scheme is also applicable for 2CCs and single UL carrier within CA. The single UL carrier within CA at least includes the following cases:

- non-CA case in UL (i.e., UL SCCs not exist or removed by network, only DL CA is configured)

- UL CA are configured, but only one UL CC is activated.

DL CC(s) clarification

RAN4 would like to inform that when UE is configured for one UL CC but more than one DL CC, the DC location maybe positioned in DL CC and for FR2 UE needs to be able to signal the location of the DC on any configured DL CC.

In addition, for FR2, TX LO cannot locate in the DL only spectrum, and the DL only spectrum here means the spectrum described in TS 38.101-2 clause 5.3A.4 and defined by Fsd according to Table 5.3A.4-3, and DL CC locate in DL-only frequency spectrum will not be used to calculate the default DC location.

As summary, the Re1-17 DC location can be applied for one UE’s CA configuration containing an intra-band carrier aggregation component, where one, two or more UL CCs are configured and more than two CCs are configured. This somehow overlaps with R16 DC location report which is applied to the scenario where two uplink CCs are configured. RAN4 suggest to only use one report scheme for one CA configuration as below：

Compatibility with R15/R16 reporting scheme

The new reporting scheme is optional and for cases where multiple reporting scheme are applicable and UE supports these capabilities, it is up to the network to decide which scheme will be used.

For a UL CA configuration in one band combination, if multiple DCs exist there is only one scheme will be used for these DC location reporting, for example, only Rel-17 reporting scheme can be used throughout different UL CC groups on the same band.

Proposal 3: only one DC location report scheme among R15/16/17 is used for a CA configuration and the network indicate which scheme is to be used based on UE’s capability.

CC group report

In the LS, it is suggested to limit the CC groups so that each group contains only one UL CC as blow:

CC group for multiple DCs reporting (mentioned in LS R4-2206602)

RAN4 would like to share more details as follows:

- CC groups do not always consist of multiple CCs, but it may consist of single CC.

- Considering R17 spec restriction, limit the CC groups so that each group contains only one UL CC

- In R17, since there are only two UL CCs in case NW deactivate or deconfigure UL CC, there is no second DC location.

However, in RAN2 signaling design, CC group concept is not only used for intra-band CC combinations with two DC locations, but also used for an intra-band CC combination with one DC location. Therefore this limitation is not valid for an intra-band CC combination with two uplink carrier and one DC location.

Proposal 4: we shall not limit the CC groups so that each group contains only one UL CC as suggested by RAN4, but we shall update the CR to allow that a CC group only contains only CC.

RAN4 also suggests to only report one DC location in case NW deactivates or de-configures UL CC as below, and this is based the assumption that only two UL CCs are supported in Rel-17.

  - In R17, since there are only two UL CCs in case NW deactivate or deconfigure UL CC, there is no second DC location.

However consider that there might be more CCs supported in future, we would suggest to update this as:

The UE shall report only one DC location for an intra-band CC combination with one active uplink carrier in case the offset is derived from active CC/BWP.

Proposal 5: The UE shall report only one CC group/DC location for an intra-band CC combination with one active uplink carrier in case the default DC location is derived from active CC/BWP.

In addition, RAN4 also confirm that that shift7dot5kHz is still needed for Rel-17 DC location report signaling.

Proposal 6: add shift7dot5kHz to the baseline CR.

- Conclusion

Based on the above discussion, we recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: The offset value is update to (-19800 to 19800).

Proposal 2: The lowest SCS in the reported CC group is used as the offset granularity for offset report.

Proposal 3: only one DC location report scheme among R15/16/17 is used for a CA configuration and the network indicate which scheme is to be used based on UE’s capability.

Proposal 4: we shall not limit the CC groups so that each group contains only one UL CC as suggested by RAN4, but we shall update the CR to allow that a CC group only contains only CC.

Proposal 5: The UE shall report only one CC group/DC location for an intra-band CC combination with one active uplink carrier in case the default DC location is derived from active CC/BWP.

Proposal 6: add shift7dot5kHz to the baseline CR.

Reference

- R2-2206650  Introduction of DC location reporting for more than 2CCs Huawei, HiSilicon

- R4-2210782 LS on DC location for intra-band CA