3GPP TSG-RAN WG2#119-e	R2-2208390

Online, 17-29 August 2022

Agenda Item:	8.1.2

Source: 	MediaTek Inc.

Title:  	Identification and authorization of network-controlled repeaters

Document for:	Discussion, decision

- Introduction

In [1], the last objective is to study the following aspects of smart repeater management

- Identification and authorization of smart repeaters [RAN2, RAN3]

  - NOTE2: Coordination with SA3 may be needed.

This paper will introduce our views about identification and authorization of network-controlled repeaters (NCR).

# 2	Discussion

2.1	Identification

A repeater can be deployed by operator to extend network coverage, there are two NCR scenarios. One is without SIM card, the identification solution is up to network implementation, for example, network can use C-RNTI for UE identification. The other scenario is NCR with SIM card, then, legacy UE procedures can be followed.

Proposal 1: RAN2 to study both NCR with and without SIM card scenarios.

2.2	Authorization

For authorization, several cadicate solutions are proposed, one is reuse IAB-UE based solution, the other one is legacy UE solution. However, from TS 23.501 [2], subclause 5.35.2, it states that

-- TS 23.501 subclause 5.35.2 --

In IAB operation, the IAB-UE interacts with the 5GC using procedures defined for UE. The IAB-node gNB-DU only interacts with the IAB-donor-CU and follows the CU/DU design defined in TS 38.401.

For the IAB-UE operation, the existing UE authentication methods as defined in TS 33.501 applies. Both USIM based methods and EAP based methods are allowed, and NAI based SUPIs can be used.

The following aspects are enhanced to support the IAB operation:

-	the Registration procedure as defined in clause 4.2.2.2 of TS 23.502 is enhanced to indicate IAB-node's capability to the AMF;

-	The IAB-node provides an IAB-indication to the IAB-donor-CU when the RRC connection is established as defined in TS 38.331. When the IAB-indication is received, the IAB-donor-CU selects an AMF that supports IAB and includes the IAB-indication in the N2 INITIAL UE MESSAGE as defined in TS 38.413 so that the AMF can perform IAB authorization.

-	the UE Subscription data as defined in clause 5.2.3 of TS 23.502 is enhanced to include the authorization information for the IAB operation;

-	Authorization procedure during the UE Registration procedure is enhanced to perform verification of IAB subscription information;

-	UE Context setup/modification procedure is enhanced to provide IAB authorized indication to NG-RAN.

After registered to the 5G system, the IAB-node remains in CM-CONNECTED state. In the case of radio link failure, the IAB-UE uses existing UE procedure to restore the connection with the network. The IAB-UE uses Deregistration Procedure as defined in clause 4.2.2.3 of TS 23.502 to disconnect from the network.

--

From our point of view, both IAB-UE solution and legacy UE solution are the same. Therefore, it is obviously that NCR can reuse the same solution for authorization as baseline. As a NCR, there is no need to perform function like routing, flow control feedback signalling, BH RLF notification as IAB-DU does [3]. In our opinions, NCR can be treated as a UE-like device with further enhancement on signalling reduction.

Proposal 2: From RAN2 point of view, legacy UE authorization method can be a baseline, RAN2 to further study how to enhance signalling reduction.

# 3	Conclusion

We have the following observations and proposals:

Proposal 1: RAN2 to study both NCR with and without SIM card scenarios.

Proposal 2: From RAN2 point of view, legacy UE authorization method can be a baseline, RAN2 to further study how to enhance signalling reduction.

# 4	References

[1]	RP-213592, New SI: Study on NR Smart Repeaters, ZTE, RAN Meeting #94e

[2]	TS 23.501

[3]	TS 38.300