3GPP TSG-RAN WG2 Meeting #119-e	R2-2208410

Online, Aug 17th – 26th, 2022

Source: 			ZTE Corporation, Sanechips

Title: 	Discussion on NR-DC with selective activation of the cell groups

Agenda item:		8.4.3

Document for:		Discussion and Decision

- Introduction

3GPP TSG RAN Meeting #94e has approved a new work item aimed at “Further NR Mobility Enhancements”. Detailed work item description (WID) can be found in [1]. The objectives of NR-DC with selective activation of the cell groups are shown as follows:

- To specify mechanism and procedures of NR-DC with selective activation of the cell groups (at least for SCG) via L3 enhancements: - To allow subsequent cell group change after changing CG without reconfiguration and re-initiation of CPC/CPA [RAN2, RAN3, RAN4] Note 4: A harmonized RRC modelling approach for objectives 1 and 2 could be considered to minimize the workload in RAN2.

In this contribution, we discussed NR-DC with selective activation of the cell groups.

- Discussion

  - 2.1 Potential aspects to be improved

- In CHO and CPAC procedure, multiple candidate cell configurations are pre-configured by the NW. But the UE will release all stored candidate cell configurations after completing random access towards the target SpCell, i.e. after completing one CHO/CPAC execution. Thus the subsequent cell group (CG) change needs to re-configure and re-initiate the conditional reconfiguration procedure, which increases the delay for the CG change and the signaling overhead. So it’s preferred to keep the stored candidate CG configuration after completing one CG change (at least for SCG change). However, the pre-configured candidate CG configuration is usually the delta configuration based on the source CG (i.e. the serving CG when the UE receives RRCReconfiguration message including multiple candidate CG configurations) configuration, to reduce the signalling overhead. When the UE switches to the target CG, the stored candidate CG configuration may become invalid. Thus, one key issue to allow subsequent CG change is how to handle the delta configuration for candidate CGs to keep the stored candidate CG configurations are still valid after completion of one CG change.

- Observation 1: In case that the candidate CG configuration is the delta configuration based on the source CG configuration, the stored candidate CG configuration may become invalid after completion of one CG change.

- After completion of one CG change, the configuration for source CG can also be kept as a candidate CG for the subsequent CG change. But for the UE based triggering case, if the candidate node takes the source CG as a candidate CG for the subsequent CG change, the candidate node should also provide the execution condition(s) for the source CG.

- Observation 2: The source CG can also be considered as a candidate CG for the subsequent CG change.

- Besides, the ping-pong switch between several CGs may happen during subsequent CG change. For example, the UE performs the first CG change from the CG_1 to the CG_2, and then performs the second CG change to switch back to the CG_1. In case that the CG change across CUs (i.e. inter-CU case), if the security key of CG_1 is not refreshed after the first CG change, the UE will reuse the security key to transmit the new date packet with the CG_1 when switching back to the CG_1, which may cause the security issue due to the reuse of the same key+COUNT+RB. Every time when the CG change is executed, the NW may need to refresh the security key for the source CG via RRC message even if other configuration for the CG is not changed, which causes extra signalling overhead. In order to avoid frequent security key refresh via RRC signalling, the enhancement on security key refresh should be considered for subsequent CG change across CUs.

- Observation 3: In case that the CG change across CUs (i.e. inter-CU case), considering of the ping-pong switch between several CGs (e.g. switch from the CG_1 to CG_2 and then switch back to the CG_1), the security key refresh for candidate CGs should be considered to avoid the security issue due to the reuse of the same key+COUNT+RB.

- Based on the observations above, we proposed:

- Proposal 1: RAN2 consider at least the following aspects to be improved for supporting the subsequent cell group change:

- How to handle the delta configuration for candidate CGs, to keep the stored candidate CG configurations valid after completion of one CG change;

- How to add the source CG as a candidate CG for the subsequent CG change;

- For inter-CU case, how to handle the security key refresh for candidate CGs, to avoid RRC level reconfiguration to refresh the security key after completion of one CG change.

  - 2.2 Applicable scenarios and frameworks

- In this section, we discussed applicable scenarios for NR-DC with selective activation of the cell groups and the potential framework for each scenario. We further analyzed the complexity for each scenario with the proposed framework to resolve the aspects identified in section 2.1.

- For NR-DC with selective activation of the cell groups, we think the following two scenario should be considered:

- Scenario 1: the selective activation of the cell groups within one CU (i.e. Intra-CU inter-DU case)

- Scenario 2: the selective activation of the cell groups across CUs (i.e. Inter-CU case)

- Observation 4: Both the selective activation of the cell groups within one CU (i.e. Intra-CU inter-DU case) and the selective activation of the cell groups across CUs (i.e. Inter-CU case) need to be considered.

- Fig 1. (a) Intra-CU inter-DU CG activation; (b) Inter-CU CG activation

- For scenario 1, since the candidate cell groups are configured within one CU, multiple cell groups can share the same higher layer configuration, i.e. no DRB level reconfiguration is required and security key refresh is not expected. So no enhancement on security key refresh due to ping-pong CG switch, is required.

- Observation 5: For intra-CU inter-DU case, multiple candidate cell groups within one CU can share the same higher layer configuration, so no DRB level reconfiguration is required and security key refresh is not expected.

- The candidate cell group configuration can be limited to the cell group level reconfiguration (i.e. CellGroupConfig), similar to inter-DU intra-CU L1/L2 mobility (the detailed analysis is shown in our accompanied contribution [2]). Thus, a unified framework for both L1/L2 mobility and NR-DC with selective activation of the cell groups can be considered to simplify the overall procedure.

- Proposal 2: For intra-CU inter-DU case, a unified framework is considered for both L1/L2 mobility and NR-DC with selective activation of the cell groups, i.e. the candidate CG configuration is limited to the cell group level reconfiguration (i.e. CellGroupConfig).

- With the cell group level reconfiguration, a list of candidate CGs can be provided via a RRCReconfiguration message, e.g. a CandidateCellGroup AddMod/Release list. It’s also easier to add the source CG as a candidate CG in the list, e.g. by referring to the source cellGroupId. And it’s simpler to support both MCG change and SCG change using the same framework. Anyway the candidate CG configuration can be provided as candidate MCG or candidate SCG in a common way. Furthermore, the candidate CG can be configured as a CG for both MCG and SCG, e.g. MCG specific configuration can be provided, but will only be used in case the CG is activated as MCG. Considering of NW based triggering mechanism, it can be up to NW to determine how to use the candidate CG, e.g. activate the candidate CG as either MCG or SCG.

- Observation 6: For intra-CU inter-DU case, it’s easier to support both subsequent MCG change and subsequent SCG change using the same framework. Anyway the candidate CG configuration can be provided as candidate MCG or candidate SCG in a common way.

- Proposal 3: For intra-CU inter-DU case, both MCG change and SCG change can be supported with a common framework. The candidate CGs can be activated as either MCG or SCG.

- For scenario 2, the candidate CGs belong to different CUs, so the CG change is across CUs. In such case, a full set of RRC reconfiguration for each candidate CG may be required, including cell group level reconfiguration, DRB level reconfiguration and security key refresh.

- Observation 7: For inter-CU case, candidate CGs across CUs require full set of RRC reconfigurtaion, including cell group level reconfiguration, DRB level reconfiguration and security key refresh.

- Thus the conditional reconfiguration similar framework can be considered, e.g. reuse CPA/CPC similar framework for SCG addition/change, and reuse CHO similar framework for MCG change.

- Proposal 4: For inter-CU case, the conditional reconfiguration similar framework is considered for NR-DC with selective activation of the cell groups, i.e. the candidate CG configuration includes a full set of RRC reconfiguration.

- However, it may be more difficult to support both subsequent MCG change and subsequent SCG change simultaneously. Considering that the CHO configuration for MCG change may also include SCG configuration and the CPA/CPC configuration for SCG addition/change may also include MCG configuration, the execution of any one CG change may have some impact on other stored candidate CG configurations. The security key refresh problem should also be considered. Besides, the UE capability coordination among multiple CUs shall be much cumbersome and complicated considering of all possible activated CG combinations. So subsequent SCG change can be prioritized in inter-CU case.

- Observation 8: For inter-CU case, it’s difficult to support both subsequent MCG change and subsequent SCG change considering that one CG change may impact any other candidate CG configurations. And the UE capability coordination among multiple CUs shall be much cumbersome and complicated considering of all possible activated CG combinations.

- Proposal 5: For inter-CU case, SCG change is prioritized.

- In both two scenarios, some enhancements to handle the delta configuration for candidate CGs should be considered. There are following two solutions to be discussed:

- Alt 1: UE store/maintain the baseline or reference configuration (e.g. the configuration where the candidate CG configuration is received).

- Alt 2: UE translates the received delta configuration for candidate CGs into full configuration and store the translated full configuration.

- Each alternative is a possible way to handle the delta configuration to keep the stored candidate CG configurations valid after completion of one CG change. But the Alt. 2 may need larger UE buffer to store full configuration for multiple candidate CGs. So Alt.1 is preferred. The detailed solution for Alt.1 can see our accompanied contribution [2]. Considering that the candidate CG configuration in inter-CU case may include both MCG and SCG part (e.g. the configuration in CPAC container including both MCG and SCG configuration), the handling on delta configuration may be more complex than that for intra-CU case.

- Observation 9: The handling on the delta configuration for candidate CGs needs to be considered in both intra-CU and inter-CU cases. But it may be more complex in inter-DU case considering the candidate CG configuration may include both MCG and SCG part.

- Based on observations above, we summarized these two scenarios in the following table:

- Table 1: Comparison between intra-CU inter-DU case and inter-CU case

| - Scenarios | - Intra-CU inter-DU case | - Inter-CU case |
| --- | --- | --- |
| - Framework | - L1/L2 mobility similar framework | - CPAC similar framework |
| - Candidate CG configuration | - Cell group level reconfiguration, no DRB level reconfiguration, security key refresh is not expected | - Full set RRC reconfiguration, including cell group level reconfiguration, DRB level reconfiguration, and security key refresh |
| - Specific cases | - Support both MCG and SCG change | - Prioritize SCG change |
| - On delta configuration handling | - Need to consider (may easier) | - Need to consider |
| - On security key refresh | - No such issue | - Need to consider |

- In our understanding, it would be nice to simplify the overall procedure and have a unified framework as L1/L2 mobility, to enable a more efficient approach for the selective activation of the cell groups. So it’s preferred to prioritize the scenario of the selective activation of the cell groups within one CU (i.e. Intra-CU inter-DU case).

- Proposal 6: RAN2 prioritize the intra-CU inter-DU case (i.e. the selective activation of the cell groups within one CU).

  - 2.3 Triggering mechanism

- Furthermore, the following two triggering mechanism can be considered for NR-DC with selective activation of the cell groups:

- Alt.1. NW based triggering mechanism

- Alt. 2. UE based triggering mechanism

- In Alt.1, the NW sends the activation signalling (e.g. RRC message) to the UE to trigger the CG activation/switch, e.g. based on the L3 measurement report. In Alt.2, the NW needs to pre-configure the execution condition(s) for each candidate CG. And the UE can autonomously trigger the CG activation/switch when the execution condition(s) is met. To allow the subsequent CG change, each candidate node may need to provide the execution condition(s) for other candidate cell/CGs within the candidate CG configuration, which shall be used for cell/CG evaluation when the corresponding CG is activated. It may require complicated coordination procedure among multiple candidate nodes for candidate cell preparation. Another way is to let the NW update the execution conditions after completion of one CG switch/activation, which requires additional signaling. Anyway the procedure is more complex than NW based triggering mechanism. Thus it’s proposed to consider NW based triggering as the first priority.

- Observation 10: For UE based triggering mechanism, how to provide the execution conditions for the subsequent CG change needs to be considered, which may require complicated coordination among candidate nodes during candidate CG preparation phase.

- Proposal 7: Both NW based triggering mechanism and UE based triggering mechanism can be considered. The NW based triggering mechanism is taken as the first priority.

- Conclusion and proposals

- In this contribution, we discussed NR-DC with selective activation of the cell groups with the following observations and proposals:

- Potential aspects to be improved

- Observation 1: In case that the candidate CG configuration is the delta configuration based on the source CG configuration, the stored candidate CG configuration may become invalid after completion of one CG change.

- Observation 2: The source CG can also be considered as a candidate CG for the subsequent CG change.

- Observation 3: In case that the CG change across CUs (i.e. inter-CU case), considering of the ping-pong switch between several CGs (e.g. switch from the CG_1 to CG_2 and then switch back to the CG_1), the security key refresh for candidate CGs should be considered to avoid the security issue due to the reuse of the same key+COUNT+RB.

- Proposal 1: RAN2 consider at least the following aspects to be improved for supporting the subsequent cell group change:

- How to handle the delta configuration for candidate CGs, to keep the stored candidate CG configurations valid after completion of one CG change;

- How to add the source CG as a candidate CG for the subsequent CG change;

- For inter-CU case, how to handle the security key refresh for candidate CGs, to avoid RRC level reconfiguration to refresh the security key after completion of one CG change.

- Applicable scenarios and frameworks

- Observation 4: Both the selective activation of the cell groups within one CU (i.e. Intra-CU inter-DU case) and the selective activation of the cell groups across CUs (i.e. Inter-CU case) need to be considered.

- Observation 5: For intra-CU inter-DU case, multiple candidate cell groups within one CU can share the same higher layer configuration, so no DRB level reconfiguration is required and security key refresh is not expected.

- Proposal 2: For intra-CU inter-DU case, a unified framework is considered for both L1/L2 mobility and NR-DC with selective activation of the cell groups, i.e. the candidate CG configuration is limited to the cell group level reconfiguration (i.e. CellGroupConfig).

- Observation 6: For intra-CU inter-DU case, it’s easier to support both subsequent MCG change and subsequent SCG change using the same framework. Anyway the candidate CG configuration can be provided as candidate MCG or candidate SCG in a common way.

- Proposal 3: For intra-CU inter-DU case, both MCG change and SCG change can be supported with a common framework. The candidate CGs can be activated as either MCG or SCG.

- Observation 7: For inter-CU case, candidate CGs across CUs require full set of RRC reconfigurtaion, including cell group level reconfiguration, DRB level reconfiguration and security key refresh.

- Proposal 4: For inter-CU case, the conditional reconfiguration similar framework is considered for NR-DC with selective activation of the cell groups, i.e. the candidate CG configuration includes a full set of RRC reconfiguration.

- Observation 8: For inter-CU case, it’s difficult to support both subsequent MCG change and subsequent SCG change considering that one CG change may impact any other candidate CG configurations. And the UE capability coordination among multiple CUs shall be much cumbersome and complicated considering of all possible activated CG combinations.

- Proposal 5: For inter-CU case, SCG change is prioritized.

- Observation 9: The handling on the delta configuration for candidate CGs needs to be considered in both intra-CU and inter-CU cases. But it may be more complex in inter-DU case considering the candidate CG configuration may include both MCG and SCG part.

- Proposal 6: RAN2 prioritize the intra-CU inter-DU case (i.e. the selective activation of the cell groups within one CU).

- Triggering mechanism

- Observation 10: For UE based triggering mechanism, how to provide the execution conditions for the subsequent CG change needs to be considered, which may require complicated coordination among candidate nodes during candidate CG preparation phase.

- Proposal 7: Both NW based triggering mechanism and UE based triggering mechanism can be considered. The NW based triggering mechanism is taken as the first priority.

- Reference

- RP-221799  Revised WID on Further NR mobility enhancements  MediaTek

- R2-2208409  Candidate solutions for L1/L2 mobility  ZTE Corporation, Sanechips