3GPP TSG RAN WG2#119e	R2-2208425

Online: August 17 – August 26, 2022

Agenda Item:	8.7.4

Source:	CMCC

Title:	Discussion on mobility enhancements for connected mode

Document for:	Discussion, Decision

Introduction

In NTN R17, connected mobility enhancements introduce two main solutions, time-based and location-based for CHO. However, the schemes for reducing signaling overhead of handover caused by a large number of UEs handover almost at the same time has less been touched, which is one of the R18 WID scope [1]:

  - Specify NTN-NTN handover enhancement for RRC_CONNECTED UEs in the quasi-earth-fixed cell and earth-moving cell to reduce the signalling overhead. [RAN2, RAN3]

In this contribution, we would like to provide insights into potential enhancements on the signaling overhead reduction for connected mobility.

Discussion

- 2.1 Group handover

Due to the large coverage of satellites, there may be plenty of UEs need to hand over to a new cell in a short period of time as shown in Fig.1. And if a large number of UEs accessing the same new cell(s) almost simultaneously, it may lead to uplink signaling storms and access resources shortage. Therefore, we could consider some solutions such as distributing UEs to access the same new cell(s). Simply, the UE randomly selects a time to access the new cell within the time range configured by the serving cell, or the UE could use truncated UE ID and the parameters provided by the network side to perform a modulo to obtain the UE-specific moment to access the new cell.

Fig.1 UEs handover in NTN system

Proposal 1: We suggest RAN2 to consider some solutions such as distributing UEs to access the same new cell(s) considering uplink signaling storms and access resources shortage due to a large number of UEs accessing the same new cell(s) almost simultaneously.

Furthermore, we could consider that the serving cell provide some common configurations and timing information of one or more candidate cells to UEs. Then UEs divided in the same group according to given rules could use the configurations and timing information to handover to the corresponding target cell. And there are many kinds of UE division principles, for example, distance-based solution(distance between UEs and cell(serving cell/target cell) reference are same or belong to same range), time-based solution(remaining serving time is same or belong to same range, time-based solution may be more suitable to moving cell in which remaining serving time is per UE. ) , etc.

Proposal 2: We suggest RAN2 to consider group HO with UEs divided in the same group according to given rules.

However, legacy handover mechanism requires a large amount of signaling interaction among UE, source gNB, and target gNB. For NTN, the high-speed movement of the satellites(e.g. LEO) will obviously lead to frequent HO in a short time.

And the coverage of the satellites is much larger than the terrestrial network resulting in a very large number of UEs in a single satellite cell, which will make the handover procedure more difficult besides the challenges mentioned above.

For the frequent HO in a short time with a large number of UEs, some signaling and information that are common to all the UEs can be notified to the UEs through broadcast. While, the gain of signaling overhead reduction may be limited through the notification of the common signaling and information via SI. There is still much UE-specific signaling and information that need to be interacted among UE, source gNB, and target gNB, therefore, this solution may be needed to further evaluation.

Observation 1: For NTN system with high propagation delay, high-speed movement of the satellites(e.g. LEO), and potential multiple UEs demanding performing HO , the mobility enhancements solutions are needed to be designed urgently.

Proposal 3: The gain of signaling overhead reduction through the solution that broadcast handover signaling and information common to all the UEs may need to further evaluate due to the limited common signaling and information that can be extracted.

- 2.2 Handover without UE perception

Further, in order to decrease overall signaling overhead, maybe we could consider to reduce one interactive node. As we know, there are three nodes (i.e. source gNB, target gNB and UE) during the whole handover procedure. The information of UE is actually known by the source gNB. Therefore, the UE maybe consider to reduce. Then all the information about UE, including UE context, protocol information etc. could be interacted between source gNB and target gNB directly.

Proposal 4: In order to decrease signaling overhead during the whole HO procedure, we could consider a handover scheme that the UE does not perceive, where all the information about UE, including UE context, protocol configuration, UE variables, constants and timers etc. could be interacted between source gNB and target gNB beforehand.

To ensure seamless handover, the source gNB needs to pre-evaluate the timing to handover the UE to the new gNB, so that all the information of UE can be transmitted to the target gNB in advance. The location information including UE location and the satellite ephemeris is important for the HO timing deduction.

Proposal 5: To ensure seamless handover, the source gNB needs to pre-evaluate the HO timing to transmit all the information of UE to the target gNB in advance.

- 2.3 Handover without PCI changing

On the other hand, if the UE is stationary or moving very slowly, then the UE needs to switch or change to a new cell just because the serving satellite changes due to the high-speed movement of the satellite. At this time, if the service link switching does not cause PCI changing as shown in Fig.2, maybe the UE could consider itself to be in the same cell all the time or for a long period of time at least, which significantly reduce the frequent handover to a great extent, so we could consider to keep same PCI during only service link switch.

Fig.2 Service link switching without PCI changing

Proposal 6: It is proposed to keep same PCI during only service link switch to reduce frequent handover.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on mobility enhancements for connected mode and have the following proposals:

Proposal 1: We suggest RAN2 to consider some solutions such as distributing UEs to access the same new cell(s) considering uplink signaling storms and access resources shortage due to a large number of UEs accessing the same new cell(s) almost simultaneously.

Proposal 2: We suggest RAN2 to consider group HO with UEs divided in the same group according to given rules.

Proposal 3: The gain of signaling overhead reduction through the solution that broadcast handover signaling and information common to all the UEs may need to further evaluate due to the limited common signaling and information that can be extracted.

Proposal 4: In order to decrease signaling overhead during the whole HO procedure, we could consider a handover scheme that the UE does not perceive, where all the information about UE, including UE context, protocol configuration, UE variables, constants and timers etc. could be interacted between source gNB and target gNB beforehand.

Proposal 5: To ensure seamless handover, the source gNB needs to pre-evaluate the HO timing to transmit all the information of UE to the target gNB in advance.

Proposal 6: It is proposed to keep same PCI during only service link switch to reduce frequent handover.

Reference

[1] RP-221819 R18 WID NR_NTN_enh_v08_clean