3GPP TSG-RAN WG2 Meeting #119 electronic	                                        R2-2208441

Online, 17th-26th, Aug, 2022

Agenda Item:	8.11.2

Source:	CMCC

Title:	Initial consideration on multicast reception in RRC_INACTIVE

Document for:	Discussion

Introduction

In RAN#95 E-meeting, Work Item on NR Multicast and Broadcast Services was approved [1]. This Work Item is to further enhance the NR Multicast/Broadcast functions based on Rel-17 MBS. The objectives for Rel-18 include:

- Specify support of multicast reception by UEs in RC_INACTIVE state [RAN2, RAN3] - PTM configuration for UEs receiving multicast in RRC_INACTIVE state [RAN2] - Study the impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE. (Seamless/lossless mobility is not required) [RAN2, RAN3] - Specify Uu signalling enhancements to allow a UE to use shared processing for MBS broadcast and unicast reception, i.e., ‎including UE capability and related assistance information reporting regarding simultaneous unicast reception in RRC_CONNECTED and MBS broadcast reception from the same or different operators [RAN2] - Study and if necessary, specify enhancements to improve the resource efficiency for MBS reception in RAN sharing scenarios [RAN3] Note: collaboration with SA2 is expected in due course for the above objectives.

RAN2#119-e is the first meeting of the Rel-18 NR Multicast and Broadcast Services work item. In this contribution we provide our view on the topic of multicast reception in RRC_INACTIVE, including PTM configuration delivery method, state transition and mobility issues.

Discussion

General principle

In Release 17, MBS was discussed, and two kinds of service and basic transmission schemes were proposed: multicast and broadcast.

For broadcast, it focuses on services with low QoS requirements, and only PTM transmission is supported, the same service and the same specific content data are provided simultaneously to all UEs in a geographical area. A UE can receive a broadcast communication service in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state. In some sense, broadcast is provided in a best-effort way and enables scalability.

When it comes to multicast, the key point becomes different. Multicast aims to services with high QoS requirements. To ensure the QoS requirements, some approaches like dedicated signalling provided configuration, HARQ retransmission, PDCP status report, lossless handover supporting and PTP/PTM dynamic switch are specified. And different from broadcast, 3GPP network could have the user information though join procedure, which is also helpful for QoS assurance.

Observation 1: In Rel-17 discussion, QoS/reliability assurance is a key issue for multicast design.

Since multicast could only be received by UEs in RRC_CONNECTED state, there could be network congestion issues due to a large group UEs receiving multicast services. Therefore, in Rel-17, multicast reception in RRC_IDLE and RRC_INACTIVE was provided by some companies, especially consider the public safety case, in which a large UE number needs to be supported, and this case is also be captured in SA2 TR [2] as a Rel-18 scenario. Due to limited time, it was agreed to postpone the related discussion and focus on RRC_INACTIVE first, if needed. And now it is one important objective in Rel-18 MBS work item.

Considering that without RRC connection with gNB, UE cannot provide any feedback or assistant information for network scheduling, and this may cause some data loss, which does not align with Rel-17 multicast design. In our view Rel-18 MBS is a further enhancement for Rel-17, it should follow Rel-17 design principle as much as possible. Therefore, it’s better for RAN2 to decide the general design principle for Rel-18 multicast reception in RRC_INACTIVE, for example, whether QoS/reliability assurance should be addressed with high priority. Since this may have great effect on PTM configuration delivery and mobility procedure.

Proposal 1: RAN2 is kindly asked to decide the general design principle for Rel-18 multicast reception in RRC_INACTIVE, for example, whether QoS/reliability assurance should be addressed with high priority.

PTM configuration delivery and update scheme

RRC_INACTIVE is a state where a UE remains in CM-CONNECTED and can move within an area configured by NG-RAN (the RNA) without notifying NG-RAN. For a UE in RRC_CONNECTED could be released to RRC_INACTIVE by gNB，therefore，there could be two scenarios for multicast reception：

Scenario1：For A UE in RRC_CONNECTED, it is released to RRC_INACTIVE to start or continue the multicast reception.

Scenario2: For A UE who is in RRC_INACTIVE, it may start multicast reception in RRC_INACTIVE state.

From our point of view, both of the two scenarios should be taken into consideration in the following RAN2 meetings.

Proposal 2: RAN2 should consider both scenarios in the following meetings:

Scenario1：For A UE in RRC_CONNECTED, it is released to RRC_INACTIVE to start or continue the multicast reception.

Scenario2: For A UE who is in RRC_INACTIVE, it may start multicast reception in RRC_INACTIVE state.

In Rel-17 discussion, two kinds of PTM configuration delivery and update methos are defined, which could be the start point for PTM configuration delivery and update for RRC_INACTIVE UEs:

- Via dedicated signalling

- Via SIB and MCCH like solution

Via dedicated signalling

For scenario1, UE is in RRC_CONNECTED state first, and then it moves to RRC_INACTIVE state when it receives RRCRelease message. In this scenario, it’s natural for the network to reconfigure the initial PTM configuration in RRCRelease message, considering that this message could not be avoided, and this aligns with Rel-17 multicast configuration delivery method: providing configuration via dedicated signalling. But there’s still some disadvantages: when PTM configuration for RRC_INACTIVE UE is updated, a large number of UEs may initiate Random Access, which may cause network congestion, and this is contrary to the main purpose of introduction of PTM configuration in RRC_INACTIVE. Besides, this may increase the payload of RRCRelease message.

Observation 2: PTM configuration delivered by dedicated signalling is a way closer to Rel-17 design, while it may increase the payload of RRCRelease message and cause RACH congestion when configuration updated.

Since UE may be receiving multicast when in RRC_CONNECTED (before moving to RRC_INACTIVE), and have PTM configurations for RRC_CONNECTED reception. Therefore, network could only provide a delta configuration or indicate whether the previous configuration could be all or partly used in RRC_CONNECTED, to avoid introducing too much RRC signalling overhead.

Observation 3: Delta configuration or indication may be used to avoid introducing too much RRC signalling overhead.

Besides, scenarios for UE state transition (with multicast reception) should be considered. For RRC_CONNECTED to RRC_INACTIVE, one potential scenario is network congestion like SA6 addressed. While for RRC_INACTIVE back to RRC_CONNECTED, both congestion relieving and UE request (e.g. when QoS requirement could not be achieved or for new configuration when mobility happens) could be taken into consideration.

Via SIB and MCCH like solution

For scenario 2, in Rel-17, UE need to go back to RRC_CONNECTED for multicast configuration and data reception when network indicate session start via group notification. Now that multicast could be received in RRC_INATIVE, solutions without UE states transition could be discussed, for example, Rel-17 SIB+MCCH like solution for broadcast.

SIB+MCCH like solution could be used in both initial configuration provision and configuration update. But UE still need the indication or criterion whether it could receive multicast in RRC_CONNECTED. And it should be noticed that there could be UEs receiving multicast in RRC_CONNECTED inevitably, so that SIB+MCCH like solution seems to introduce some redundancy. And SIB+MCCH way could only provides common figuration but no UE specific configurations and in some sense, hence, it may come with a compromised QoS.

Observation 4: SIB+MCCH like solution may avoid UE state transition but couldn’t provide UE specific configurations and the reliability of configuration could not be assured. What’s more, extra indication or criterion for UE to deduct whether it could receive multicast in RRC_CONNECTED is needed.

The pros and cons of the different two methods are listed in Table 1.

Table1: Pros and cons for different PTM configuration delivery methods

|  | Pros | Cons |
| --- | --- | --- |
| Dedicated signalling | - Reuse RRCRelease message. - Consistent with Rel-17 multicast configuration delivery method. - Reliability assurance for PTM configuration. | - Increased RRCRlease message payload. - Potential RACH congestion. |
| SIB+MCCH like solution | - Without UE state transition and RACH congestion. | - Not consistent with Rel-17 multicast - Compromised QoS. - Extra indication design. |

Since both two methods have pros and cons, and each of them could fits different scenario, both them and its combination could be further discussed.

Proposal 3: For PTM configuration delivery, following solutions could be taken into consideration:

Option 1: Dedicated signalling

Option 2: SIB+MCCH like solution

Option 3: Combination of Option 1 and Option 2

Proposal 4: State transition criterion/scenario should be discussed for all three options.

As to the configurations used in RRC_INACTIVE, Rel-17 configuration could be the baseline, while HARQ and PTP leg related configuration is not needed, considering that there’s no RRC connection between UE and gNB, while introduce HARQ and PTP may introduce extra complexity and workload.

Proposal 5: Rel-17 multicast configuration for RRC_CONNECTED could be the baseline, while HARQ and PTP are not supported for multicast reception in RRC_INACTIVE.

Mobility issue

Similar to Rel-17 discussion on multicast reception UE’s mobility, we need to identify the potential scenarios first. Considering whether multicast and reception in RRC_INACTIVE is supported or provided, mobility in RRC_INACTIVE could be classified into the following cases:

Case 1: UE receive multicast in RRC_INACTIVE moves to a neighbour cell provides multicast for RRC_INACTIVE UEs.

- Same/Different configuration is used in the two cells or not.

Case 2: UE receive multicast in RRC_INACTIVE moves to a neighbour well doesn’t provide multicast for RRC_INACTIVE UEs.

- Neighbour cell supports provide multicast for RRC_INACTIVE UEs but the session is not on-going.

- Neighbour cell supports multicast but not support multicast reception in RRC_INACTIVE.

- Neighbour cell doesn’t support multicast.

Case 3: RRC_INACTIVE UE moves from cell without multicast providing to a neighbour cell providing multicast for RRC_INACTIVE UEs.

Proposal 6: Following are potential mobility scenarios:

Case 1: UE receive multicast in RRC_INACTIVE moves to a neighbour cell provides multicast for RRC_INACTIVE UEs.

- Same/Different configuration is used in the two cells or not.

Case 2: UE receive multicast in RRC_INACTIVE moves to a neighbour well doesn’t provide multicast for RRC_INACTIVE UEs.

- Neighbour cell supports provide multicast for RRC_INACTIVE UEs but the session is not on-going.

- Neighbour cell supports multicast but not support multicast reception in RRC_INACTIVE.

- Neighbour cell doesn’t support multicast.

Case 3: RRC_INACTIVE UE moves from cell without multicast providing to a neighbour cell providing multicast for RRC_INACTIVE UEs.

For case 1, the main issue is whether UE needs to update its PTM configuration for multicast reception in RRC_INACTIVE, no matter dedicated signalling or SIB+MCCH like solution is used. To avoid unnecessary configuration, network broadcasts for neighbour cell information could be considered.

For case 2, no matter which sub-case happens, UE needs to initiate RRC resume procedure for multicast reception via individual delivery method.

For case 3, UE may perform a legacy handover to the target cell, and target cell may release it to RRC_INACTIVE state.

Thus, the first two cases should be addressed.

Proposal 7: Discussion on mobility should focus on the first two cases.

In addition, though seamless/lossless mobility is not required, we still need to minimize the data loss due to mobility. Therefore, cell reselection prioritization like Rel-17 broadcast could be used for reference.

Proposal 8: Cell reselection prioritization could be considered for multicast reception in RRC_INACTIVE.

Conclusions

In this contribution, we have discussed the multicast reception in RRC_INACTIVE state. The observations and proposal are listed below:

Observation 1: In Rel-17 discussion, QoS/reliability assurance is a key issue for multicast design.

Proposal 1: RAN2 is kindly asked to decide the general design principle for Rel-18 multicast reception in RRC_INACTIVE mode, for example, whether QoS/reliability assurance should be addressed with high priority.

Proposal 2: RAN2 should consider both scenarios in the following meetings:

Scenario1：For A UE who is receiving multicast in RRC_CONNECTED, it is released to RRC_INACTIVE and continues multicast reception.

Scenario2: For A UE who is in RRC_INACTIVE, it could start multicast reception in RRC_INACTIVE state.

Observation 2: PTM configuration delivered by dedicated signalling is a way closer to Rel-17 design, while it may increase the payload of RRCRelease message and cause RACH congestion when configuration updated.

Observation 3: Delta configuration or indication may be used to avoid introducing too much RRC signalling overhead.

Observation 4: SIB+MCCH like solution may avoid UE state transition but couldn’t provide UE specific configurations and the reliability of configuration could not be assured. What’s more, extra indication or criterion for UE to deduct whether it could receive multicast in RRC_CONNECTED is needed.

Proposal 3: For PTM configuration delivery, following solutions could be taken into consideration:

Option 1: Dedicated signalling

Option 2: SIB+MCCH like solution

Option 3: Combination of Option 1 and Option 2

Proposal 4: State transition criterion/scenario should be discussed for all three options.

Proposal 5: Rel-17 multicast configuration for RRC_CONNECTED could be the baseline, while HARQ and PTP are not supported for multicast reception in RRC_INACTIVE.

Proposal 6: Following are potential mobility scenarios:

Case 1: UE receive multicast in RRC_INACTIVE moves to a neighbour cell provides multicast for RRC_INACTIVE UEs.

- Same/Different configuration is used in the two cells or not.

Case 2: UE receive multicast in RRC_INACTIVE moves to a neighbour well doesn’t provide multicast for RRC_INACTIVE UEs.

- Neighbour cell supports provide multicast for RRC_INACTIVE UEs but the session is not on-going.

- Neighbour cell supports multicast but not support provide multicast for RRC_INACTIVE UEs.

- Neighbour cell doesn’t support multicast.

Case 3: RRC_INACTIVE UE moves from cell without multicast providing to a neighbour cell providing multicast for RRC_INACTIVE UEs.

Proposal 7: Discussion on mobility should focus on the first two cases.

Proposal 8: Cell reselection prioritization could be considered for multicast reception in RRC_INACTIVE.

References

- RP-213568, New WID: Enhancements of NR Multicast and Broadcast Services, CATT;

- TR 23.700-47 5G multicast-broadcast services; Phase 2 (Release 18)