3GPP TSG-RAN WG2 Meeting #119 electronic   						    	R2-2208467

E-Meeting, 17 – 29 August, 2022

Source: 	Xiaomi

Title:  	Discussion on NR-DC with selective activation of the cell groups

Agenda Item:	8.4.3

Document for:	Discussion and Decision

Introduction

A New WID on Further NR mobility enhancements [1] has been approved. In the WID, one of the objectives is to specify mechanism and procedures of NR-DC with selective activation of the cell groups, shown as below.

2. To specify mechanism and procedures of NR-DC with selective activation of the cell groups (at least for SCG) via L3 enhancements: - To allow subsequent cell group change after changing CG without reconfiguration and re-initiation of CPC/CPA [RAN2, RAN3, RAN4] Note 4: A harmonized RRC modelling approach for objectives 1 and 2 could be considered to minimize the workload in RAN2.

In this contribution, we provide our understandings on selective activation of the cell groups.

Discussion

Intra-node and inter-node mobility for MCG and SCG have been supported in current specification. Hence, selective activation of MCGs or SCGs including intra-node and inter-node can be considered. Generally, only MN can decide to initialize the MCG change and SCG addition procedure. MN and SN can initialize SN change procedure. For different initialization node, MN initiated MCG selective activation and MN/SN initiated SCG selective activation can be considered. For SN initiated intra-SN SCG selective activation, it can be done with/without MN involvement. Based on the above analysis, scenarios of selective activation could be divided as showed in the following table:

| CG | Initialization node | Intra or inter node | Scenarios |
| --- | --- | --- | --- |
| MCG | MN | Intra-MN | MN initiated intra-MN MCG selective activation (NR-DC or NR-standalone) |
|  |  | Inter-MN | MN initiated inter-MN MCG selective activation (NR-DC or NR-standalone) |
| SCG | MN | Intra-SN | MN initiated intra-SN SCG selective activation |
|  |  | Inter-SN | MN initiated inter-SN SCG selective activation |
|  | SN | Intra-SN | SN initiated intra-SN SCG selective activation (with/without MN involvement) |
|  |  | Inter-SN | SN initiated inter-SN SCG selective activation |

The flow charts for selective activation of the cell groups (inter-node) are provided in Annex.

For inter-node or intra-node selective activation, both can use the same configuration and procedure of Uu, but the difference lies in RAN3’s specification. So, whether to support inter-node selective activation, intra-node selective activation or both can be decided by RAN3. RAN2 can send LS to RAN3 to ask the supported scenarios of selective activation in Rel-18.

Proposal 1: RAN2 can consider the following scenarios of selective activation:

- Scenario 1: MN initiated MCG selective activation

- Scenario 2: MN initiated SCG selective activation

- Scenario 3: SN initiated SCG selective activation

Proposal 2: Whether to support inter-node selective activation, intra-node selective activation or both can be decided by RAN3. RAN2 can send LS to RAN3 to ask the supported scenarios of selective activation in Rel-18.

Selective activation of cell groups is performed for MCG or SCG mobility, and existing cell group configuration (e.g. CellGroupConfig) can be used to configure a candidate MCG or SCG. Multiple cell group configurations for selective activation of cell groups can be provided to UE via L3 message (e.g. RRCReconfiguration). One RRCReconfiguration can include multiple cell group configurations for multiple candidate cell groups (MCGs or SCGs), and one cell group configuration is used to configure one candidate cell group.

Proposal 3: Multiple candidate cell group configurations for MCG or SCG are provided to the UE via RRCReconfiguration message.

RAN2 should also discuss how to selectively activate the candidate cell group. And UE apply associated cell group configuration to achieve MCG change or SCG change/addition when activating the candidate cell group. From our understanding, the following solutions can be considered in Rel-18:

- Solution 1: Activation a candidate cell group via RRC signalling

- Solution 2: Activation a candidate cell group based on configured event(s)

For solution 1, network can indicate the activated candidate cell group or cell group configuration via RRC signalling. Upon reception of the RRC signalling including the indication, UE should activate the cell group. For solution 2, event(s) associated with each candidate cell group can be configured in advance for selective activation of cell groups. When associated event(s) are fulfilled, UE can activate the cell group and apply the cell group configuration, like CHO or CPC/CPA.

Proposal 4: The following solutions can be considered for selective activation of cell groups:

- Solution 1: Activation a candidate cell group via RRC signalling

- Solution 2: Activation a candidate cell group based on configured event(s)

To support the objective and allow subsequent cell group change after changing CG without reconfiguration and re-initiation of CPC/CPA, UE should keep the candidate cell group configurations after completing selective activation execution successfully.

Proposal 5: To support objective 2 of selective activation of the cell groups, UE should maintain the candidate cell group configurations after completing selective activation execution successfully.

Network can (re)configure and release cell group configurations of selective activation via RRC signalling. MN and SN can transmit RRC signalling to UE for the addition/modification/removal of candidate cell group configuration, for MN initiated and SN initiated selective activation, respectively. When receiving the RRC signalling from MN or SN, UE update stored configurations of candidate cell groups and remove/add/modify associated candidate cell group configurations.

Proposal 6: Network can (re)configure and release cell group configurations of selective activation via RRC signalling.

Conclusions

According to the analysis given above, we have the following observations and proposals:

Proposal 1: RAN2 can consider the following scenarios of selective activation:

- Scenario 1: MN initiated MCG selective activation

- Scenario 2: MN initiated SCG selective activation

- Scenario 3: SN initiated SCG selective activation

Proposal 2: Whether to support inter-node selective activation, intra-node selective activation or both can be decided by RAN3. RAN2 can send LS to RAN3 to ask the supported scenarios of selective activation in Rel-18.

Proposal 3: Multiple candidate cell group configurations for MCG or SCG are provided to the UE via RRCReconfiguration message.

Proposal 4: The following solutions can be considered for selective activation of cell groups:

- Solution 1: Activation a candidate cell group via RRC signalling

- Solution 2: Activation a candidate cell group based on configured event(s)

Proposal 5: To support objective 2 of selective activation of the cell groups, UE should maintain the candidate cell group configurations after completing selective activation execution successfully.

Proposal 6: Network can (re)configure and release cell group configurations of selective activation via RRC signalling.

Reference

[1] RP-221799, MediaTek Inc., “Revised WID on Further NR mobility enhancements”

- Annex:

- A.1: MN initiated MCG selective activation

Figure 1: MN initiated MCG selective activation (inter-MN, NR-DC)

- A.2: MN initiated SCG selective activation

Figure 2: MN initiated SCG selective activation (inter-SN)

- A.3: SN initiated SCG selective activation

Figure 3: SN initiated SCG selective activation (inter-SN)