3GPP TSG-RAN WG2 Meeting #119 electronic	R2-2208473

Online, August 17th – August 29th, 2022

Source:	CATT

Title:	Clarification on RLC bearer handling for fullConfig

Agenda Item:	5.1.3.1.1

Document for:	Discussion and Decision

Introduction

According to the current RRC specification, when the UE receives fullConfig, the UE shall release the RLC bearer of SRB1. In this contribution, we look into this procedure and explain some potential issue of the RLC state mismatch between the UE and the network. We therefore suggest RAN2 to discuss the issue, as well as potential modifications to the corresponding RRC specification.

Discussion

In accordance to the current specification, the UE will perform RLC re-establishment for SRB1, when the UE:

- performs RRC re-establishment;

- performs RRC resume.

The related procedures are shown below [1]:

- RRC re-establishment

#### 5.3.7.4 Actions related to transmission of RRCReestablishmentRequest message The UE shall set the contents of RRCReestablishmentRequest message as follows: /Omitted/; 1> re-establish PDCP for SRB1; 1> if the UE is acting as L2 U2N Remote UE: /Omitted/ 1> else: 2> re-establish RLC for SRB1; 2> apply the default configuration defined in 9.2.1 for SRB1; /Omitted/

- RRC resume

5.3.8.3 Reception of the RRCRelease by the UE The UE shall: /omitted/ 1> if the RRCRelease includes suspendConfig: 2> reset MAC and release the default MAC Cell Group configuration, if any; 2> apply the received suspendConfig except the received nextHopChainingCount; /omitted/ 2> re-establish RLC entities for SRB1; /omitted/

Observation 1: The RLC entity for SRB1 will be re-established during RRC resume and RRC re-establishment procedure.

During RRC resume or RRC reestablishment procedure, the network may configure fullConfig to the UE. Based on the RRC procedure, the UE will release the RLC bearer configuration for all RBs including SRB1, and the RLC bearer for SRB1 will be reestablished by signaling or as default configuration as following shows:

#### 5.3.5.11 Full configuration The UE shall: 1> release/ clear all current dedicated radio configurations except for the following: - the MCG C-RNTI; - the AS security configurations associated with the master key; - the SRB1/SRB2 configurations and DRB/multicast MRB configurations as configured by radioBearerConfig or radioBearerConfig2. NOTE 1: Radio configuration is not just the resource configuration but includes other configurations like MeasConfig. Radio configuration also includes the RLC bearer configurations as configured by RLC-BearerConfig. In case NR-DC or NE-DC is configured, this also includes the entire NR or E-UTRA SCG configuration which are released according to the MR-DC release procedure as specified in 5.3.5.10. /Omitted/ NOTE 1b: To establish the RLC bearer of SRB(s) after release due to fullConfig, the network can include the srb-Identity within srb-ToAddModList (i.e. the UE applies RLC default configuration) and/or provide rlc-BearerToAddModList of concerned SRB(s) explicitly. /Omitted/

Observation 2: The RLC entity for SRB1 will be re-established during fullConfig procedure.

When RLC is re-established/released and established, all the RLC SDUs and PDUs will be discarded and all the variables will be reset to the initial values. For the case that fullConfig is sent to the UE in RRCResume/RRCReestablishment, the RLC state variables will be initialized twice at the UE side.

When RLC is re-established/released and established, all the RLC SDUs and PDUs will be discarded and all the variables will be reset to the initial values.  This means that the reception window, for the subsequent RLC PDUs of SRB1, i.e., [RX_Next,  RX_Next + AM_Window_Size) will be set to [0, AM_Window_Size). Take the following figures as one example.

- When the UE initiates RRC resume or RRC re-establishment procedure, the RLC state parameters at the UE side is set to the initial values.

Figure 1: The RLC state variables for SRB1 are set to initial value during RRC Resume and RRC re-establishment procedure

When the UE receives RRCResume or RRCReestablishment message, the UE updates the RLC state variables accordingly. The Rx_Next is updated to the SN of the first RLC SDU with SN > current RX_Next for which not all bytes have been received. For gNB, the Tx_Next is updated to the value of the SN to be assigned for the next newly generated AMD PDU.

Figure 2: The RLC state variables are updated when RRCResume/RRCReestablishment message is received

When the UE receives the RRCReconfiguration with fullConfig, the UE firstly updates the RLC state variables. And then, the UE releases RLC entity for SRB1. All the variables at UE RLC are set to the initial values.

Figure 3: The RLC state variables are updated when RRCReconfiguration message w/t fullConfig is received

The POLL bit for RLC PDU with fullConfig will be set. However, the RLC state variables at the UE side are set to initial values since the UE receives fullConfig. Therefore, the conditions for triggering RLC status report are not satisfied. Then, no RLC status report will be sent to the network. The network is not aware whether fullConfig has been received successfully at the UE. Hence, the RLC state at the UE does not match that at the network.

Observation 3: The RLC state mismatch for SRB1 happens when the fullConfig is receives after the UE performs RRC Resume/RRC re-establishment procedure.

In order to solve this issue, one simple solution is not to Release the RLC bearer of SRB1 for RRC resume and RRC re-establishment when fullConfig is receives by the UE.

Hence, we suggest:

Proposal 1: RAN2 discuss on the issue as outlined in the above observations.

Proposal 2: If the issue is confirmed, RAN2 can further discuss correction to the RRC specification so that the RLC configuration of SRB1 is not cleared upon full configuration for RRC resume and re-establishment.

Conclusion

In this document, we analyse the procedure when the UE releases RLC entity for SRB1 in full configuration procedure and make the following observations:

Observation 1: The RLC entity for SRB1 will be re-established during RRC resume and RRC re-establishment procedure.

Observation 2: The RLC entity for SRB1 will be re-established during fullConfig procedure.

Observation 3: The RLC state mismatch for SRB1 happens when the fullConfig is receives after the UE performs RRC Resume/RRC re-establishment procedure.

Based on these, we propose that:

Proposal 1 RAN2 discuss on the issue as outlined in the above observations.

Proposal 2	 If the issue is confirmed, RAN2 can further discuss the correction to the RRC specification so that the RLC configuration of SRB1 is not cleared upon full configuration for RRC resume and re-establishment.

- Reference

- 38.331, V17.1.0, NR; Radio Resource Control (RRC) protocol specification;