3GPP TSG-RAN WG2 #119e                                                                                         R2-2208475

eMeeting, 17th – 26th August, 2022

Agenda Item:	    8.4.4

Source: 	    MediaTek Inc.

Title:		    Discussion and clarification on CHO enhancement scenarios

Document for:	    Discussion and decision

# 1 Introduction

In this contribution, we discuss below objective #3 and objective #4 from mobility enhancement work item [1].

- To specify CHO including target MCG and target SCG [RAN3, RAN2].

Note 5: This is already being targeted for Rel-17, so this objective will be reviewed at RAN#97-e.

- To specify CHO including target MCG and candidate SCGs for CPC/CPA in NR-DC [RAN3, RAN2]

- CHO including target MCG and target SCG is used as the baseline

We would like to clarify the intended scenario for objective #3 and objective #4. In addition, RP#96 has discussed whether objective #3 is already supported in Rel-17 but no conclusion. We also want to discuss whether scenario for objective#3 is already supported from RAN2 perspective.

# 2 Discussion

## 2.1 CHO and CPAC co-existence

RAN2 has discussed co-existence of CHO and CPC since Rel-16. It was concluded that joint CHO and CPC is not supported in Rel-16. In RAN2#113bis, RAN2 discussed different supporting scenario. Basically, it is categorized as whether the UE monitors single trigger condition for CHO or monitoring trigger conditions for CHO and CPC simultaneously (see also Q12 of R2 email discussion summary in R2-2103109 [2]). Based on [2] and our understanding, there could be 3 different CHO + CPAC scenarios as below:

- Scenario 1 - The CHO and CPAC configuration are independent. The UE monitors the triggering conditions for the CHO and CPAC independently.

- Scenario 2 - A CHO configuration that contains an associated CPAC configuration. (Only one trigger condition for CHO based on MCG measurement)

  - Scenario 2a

    - The executed RRC Reconfiguration message for CHO contains target SCG configuration.

    - While executing the CHO RRC Reconfiguration message, the target PSCell is also added/modified/changed via that RRC Reconfiguration

  - Scenario 2b

    - The executed RRC Reconfiguration message for CHO contains CPAC configuration (i.e. candidate SCGs).

    - After executing the CHO RRC Reconfiguration message, the UE also start monitoring the triggering conditions for the associated CPAC configuration

It is our understanding that objective#3 is target for scenario 2a and objective#4 is targeted for scenario 2b. It would be good to confirm RAN2 understanding on supporting scenario first before discussing further details.

Proposal 1: RAN2 understands the obj#3 in R18 mobility WI is targeted for the following scenario:

- The UE monitors the trigger condition for one CHO configuration, where the executed RRC Reconfiguration message for this CHO contains target SCG configuration.

- While executing the CHO RRC Reconfiguration message, the target PSCell is also added/modified/changed via that RRC Reconfiguration.

Proposal 2: RAN2 understands the obj#4 in R18 mobility WI is targeted for the following scenario:

- The UE monitors the trigger condition for one CHO configuration, where the executed RRC Reconfiguration message for this CHO contains candidate SCG configurations.

- After executing the CHO RRC Reconfiguration message, the UE also start monitoring the triggering conditions for the candidate SCG configurations.

For scenario 1, RAN2 has the following working assumption in RAN2#117 (and confirm in RAN2#118)

- 12: (R16/R17 CHO/CPAC coex) If one conditional reconfiguration is executed, the other conditional reconfigurations should be released. Everything else is up to UE implementation.

Our understanding is that scenario 1 is already supported in Rel-17 and it is NOT included in R18 mobility WID.

## 2.2 CHO with target SCG

In RAN2#115, RAN2 discussed whether to support CHO with target SCG configuration. It is concluded that CHO with target SCG configuration is NOT supported in Rel-16 but RAN2 intended to support this in Rel-17. An LS R2-2109172 is sent to RAN3 [3].

R2-2108164	Discussion on CHO with SCG configuration	ZTE Corporation, Sanechips	discussion	Rel-16	NR_Mob_enh-Core

- CHO with SCG configuration is not supported in Rel-16. R2 assumes this will be supported in Rel-17.

R2-2109172	Response LS on CHO with SCG configuration	RAN2	LS out	Rel-16	NR_Mob_enh-Core	To:RAN3

There is a reply LS R2-2204494 [4] from RAN3 which indicated that

.

RAN3 would like to thank RAN2 for the LS on Conditional Handover with SCG configuration scenarios! RAN3 has discussed the matter and decided to enable the needed signalling. The two attached CRs are related to the signalling between the target MN and the target SN.

R2-2206600	Support of CHO with SCG configuration - 36331 [CHOwithDCkept]	CATT, Huawei, ZTE, China Unicom, China Telecommunications, CMCC, Ericsson	CR	Rel-17	36.331	17.0.0	4823	-C	TEI17

R2-2206601	Support of CHO with SCG configuration -  38331 [CHOwithDCkept]	CATT, Huawei, ZTE, China Unicom, China Telecommunications, CMCC, Ericsson	CR	Rel-17	38.331	3183	-17.0.0	C	TEI17

R2-2206602	Support of CHO with SCG configuration - 37340 [CHOwithDCkept]	CATT, Huawei, ZTE, China Unicom, China Telecommunications, Nokia, Nokia Shanghai Bell, CMCC, Ericsson	draftCR	Rel-17	37.340	0329	-	17.0.0	C	TEI17

- [036] 3 CRs agreed

R2-2206456	Introduction UE capability for CHO with SCG configuration		MediaTek Inc	CR	Rel-17	38.331	3179	-	17.0.0	C	TEI17

R2-2206457	Introduction UE capability for CHO with SCG configuration		MediaTek Inc	CR	Rel-17	38.306	0746	-	17.0.0	C	TEI17

- [036] both revised, cover sheet updates to support CR approval, add [CHOwithDCkept], revisions agreed unseen in (R2-2206709, R2-2206710).

RAN2 discussed the incoming RAN3 LS in RAN2#118 and the following 5 CRs are agreed to support CHO with target SCG procedure.

Therefore, both RAN2 and RAN3 have agreed some CRs to support CHO with target SCG procedure. At least, basic flow of objective #3 is already doable in Rel-17 based the agreed CRs. However, there are still some concerns to remove this objective from Rel-18 during RP#96 discussion.

In RP#96, some companies argue that MN is not aware of intra-SN PSCell change and the CHO with target SCG configuration may be out of date after this intra-SN change [5]. We believe that this is an issue need to be resolved in Rel-17. Either intra-SN PSCell change should not be used after CHO with target SCG is configured or additional inter-node signaling is needed. We think that this belongs to RAN3 discussion and no need to have further R18 work on RAN2.

Another argument for keeping objective#3 is that some companies proposed to have additional optimization on SCG failure behavior [6]. In current procedure, there is guard timer for SCG change/addition. If something wrong, the UE will trigger SCG failure procedure to inform network and wait for proper reconfiguration. This baseline failure handler may not be optimal but at least it is workable. We believe that optimization on failure handling is not original intention for this objective and it is not an essential enhancement. We do not see clear motivation to continue the failure handling discussion in RAN2. Our suggestion is to confirm RAN2 procedure already supports this CHO with target SCG flow. No further discussion is needed in Rel-18.

Proposal 3: RAN2 confirms the obj#3 (CHO with target SCG) in R18 mobility WI is already supported in Rel-17 from RAN2 aspects. Additional inter-node signaling (if needed) could be added in Rel-17 based on RAN3 conclusion.

## 2.3 CHO with candidate SCG

For CHO with candidate SCG, if companies confirmed the understanding in P2, we believe that RAN2 impact (or UU interface change) is not so big. One of the major changes will be remove the following configuration limitation in TS 38.331.

CondReconfigToAddMod field descriptions

condRRCReconfig The RRCReconfiguration message to be applied when the condition(s) are fulfilled. The RRCReconfiguration message contained in condRRCReconfig cannot contain the field conditionalReconfiguration or the field daps-Config. For CPA and for CPC, the RRCReconfiguration message contained in condRRCReconfig cannot contain the field scg-State.

We think that the main discussion for objective#4 would be in RAN3 and RAN2 could just wait first. RAN2 can further discuss RAN2 procedure impact based on RAN3 progress.

Proposal 4: For obj#4 (CHO with candidate SCG), RAN2 to wait progress from RAN3.

# 3 Conclusions

Based on the discussion in section 2, we propose the following:

Proposal 1: RAN2 understands the obj#3 in R18 mobility WI is targeted for the following scenario:

- The UE monitors the trigger condition for one CHO configuration, where the executed RRC Reconfiguration message for this CHO contains target SCG configuration.

- While executing the CHO RRC Reconfiguration message, the target PSCell is also added/modified/changed via that RRC Reconfiguration.

Proposal 2: RAN2 understands the obj#4 in R18 mobility WI is targeted for the following scenario:

- The UE monitors the trigger condition for one CHO configuration, where the executed RRC Reconfiguration message for this CHO contains candidate SCG configurations.

- After executing the CHO RRC Reconfiguration message, the UE also start monitoring the triggering conditions for the candidate SCG configurations.

Proposal 3: RAN2 confirms the obj#3 (CHO with target SCG) in R18 mobility WI is already supported in Rel-17 from RAN2 aspects. Additional inter-node signaling (if needed) could be added in Rel-17 based on RAN3 conclusion.

Proposal 4: For obj#4 (CHO with candidate SCG), RAN2 to wait progress from RAN3.

# 4 References

[1] RP-221799, “Revised WID on Further NR mobility enhancements”, MediaTek

[2] R2-2103109, “Summary of [Post113-e][234][eDCCA] CPAC procedures (CATT)”, CATT

[3] R2-2109172, “Response LS on CHO with SCG configuration”, RAN2

[4] R2-2204494/R3-222840, “Response LS on Conditional Handover with SCG configuration scenarios (R3-222840; contact: Nokia)”, RAN3

[5] RP-221433, “Rel-18 MobEnh scope”, Ericsson

[6] RP-221343, “Discussion on Rel-18 WID Further NR mobility enhancements”, Intel