3GPP TSG-RAN WG2 Meeting #119-e	R2-2208486

# Electronic Meeting, 17 – 29 Aug, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | - | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on L2 Remote UE threshold conditions for service continuity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-Core |  |  |  |  |  | Date: |  |  | 2022-08-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For UEs capable of L3 U2N Remote UE (irrespective of its RRC state) and for the IDLE/INACTIVE UEs capable of L2 U2N Remote UE, the UE can decide whether to perform sidelink discovery for relay selection/reselection based on AS criteria. However, for UEs capable of L2 U2N Remote UE, a CONNECTED UE may be configured with RRM measurement to search for candicate L2 U2N Relay UEs for potential direct-to-indirect path switch. In this case, it would be a redundant behavour to check AS criteria for the UE, because the network expects the UE to perform sidelink discovery and report the measurement results when configuring the measurement. In order to simplify the UE behavour for Relay measurement reporting, it can be clarified that a UE capable of L2 U2N Remote UE can perform discovery after configured with measurement object of L2 U2N Relay UE i.e. consider NR Sidelink U2N Remote UE threshold conditions met without checking the Uu condition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 5.8.15.2, clarify that if the UE is configured with NR sidelink measurements of L2 U2N Relay UEs, it considers the threshold conditions to be met (entry). Impact analysis Impacted 5G architecture options: NR SA Impacted functionality SL relay Inter-operability: If the network is implemented according to this CR while the UE is not, a UE is required to check the Uu threshold condition before performing sidelink discovery and measurement reporting on Relay UE(s). This behaviour is redundant but there is no inter-operability issue. If the UE is implemented according to this CR while the network is not, there is no inter-operability issue. If one UE is implemented according to this CR while the other UE is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without the change, the remote UE is required to check the Uu threshold condition before performing sidelink discovery and measurement reporting on Relay UE(s) which is redundant and not needed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.15.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

#### 5.8.15.2	NR Sidelink U2N Remote UE threshold conditions

A UE capable of NR sidelink U2N Remote UE operation shall:

1>	if the threshold conditions specified in this clause were previously not met:

2>	if threshHighRemote is not configured; or the RSRP measurement of the PCell, or the cell on which the UE camps, is below threshHighRemote by hystMaxRemote if configured, or

2> if the UE has no serving cell:

3>	consider the threshold conditions to be met (entry);

2> if the UE is in RRC_CONNECTED and configured with a measObject associated to L2 U2N Relay UE:

3>	consider the threshold conditions to be met (entry);

1>	else:

2>	if the RSRP measurement of the PCell, or the cell on which the UE camps, is above threshHighRemote if configured:

3>	consider the threshold conditions not to be met (leave);

END OF CHANGE