3GPP TSG-RAN WG2 Meeting #119e electronic	R2-2208487

Online, 17th – 29th Aug, 2022

Agenda Item:	6.7.2.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on SRAP entity handling

Document for:	Discussion and Decision

- Introduction

For SRAP handling, the description are not totally aligned across RAN2 specifications, e.g. when/how to establish SRAP entity at the Remote UE and Relay UE, as well as SRB0/SRB1 forwarding related behavior. In this contribution, we would like to discuss the misaligned aspects and propose changes to the corresponding specifications.

- Discussion

2.1 SRAP entity establishment

Remote UE may initiate the RRC connection setup/ re-establishment/ resume procedure via a L2 U2N Relay UE, and send the first SRB0 message (i.e., RRCSetupRequest/ RRCReestablishmentRequest/ RRCResumeRequest) to Relay UE. According to the current description in TS 38.351[2] (see below), the Remote UE shall establish the SRAP entity before delivering the SRB0 message to SRAP sublayer. However, in TS 38.331 it says the UE establishes SRAP entity for SRB1 message transmission or reception with the assumption that the SRAP entity may be not established when transmitting the SRB0 message. In TS 38.300 [3] it says over PC5 hop there is no SRAP layer for BCCH and PCCH and no SRAP header for SRB0 message, but it is not crystal clear whether the first UL SRB0 message needs to pass SRAP layer on PC5 hop.

TS 38.351 ## 5.3 UL Data transfer ### 5.3.1 Transmitting operation of U2N Remote UE #### 5.3.1.2 Egress RLC channel determination For a SRAP Data PDU to be transmitted, the SRAP entity shall: - if the SRAP Data PDU is for SRB0: - Determine the egress PC5 Relay RLC channel in the link with U2N Relay UE corresponding to logicalChannelIdentity for SL-RLC0 as specified in TS 38.331 [3]; - else if there is an entry in sl-SRAP-Config-Remote, whose sl-RemoteUE-RB-Identity matches the SRB identity or DRB identity of the SRAP Data PDU,: - Determine the egress PC5 Relay RLC channel of the link with U2N Relay UE corresponding to sl-Egress-RLC-Channel-PC5 configured for the concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3].

TS 38.331 #### 5.3.7.4 Actions related to transmission of RRCReestablishmentRequest message The UE shall set the contents of RRCReestablishmentRequest message as follows: … 1> if the UE is acting as L2 U2N Remote UE: 2> establish or re-established (e.g. via release and add) SL RLC entity for SRB1; 2> apply the default configuration of SL-RLC1 as defined in 9.2.4 for SRB1; 2> apply the default configuration of PDCP as defined in 9.2.1 for SRB1; 2> establish the SRAP entity and apply the default configuration of SRAP as defined in 9.2.5 for SRB1; … ### 5.3.13 RRC connection resume #### 5.3.13.2 Initiation … 1> if the UE is acting as L2 U2N Remote UE: 2> apply the default configuration of SL-RLC1 as defined in 9.2.4 for SRB1; 2> apply the default PDCP configuration as defined in 9.2.1 for SRB1; 2> establish the SRAP entity and apply the default configuration of SRAP as defined in 9.2.5 for SRB1;

TS 38.300 #### 16.12.2.1 L2 UE-to-Network Relay … For L2 U2N Relay, the SRAP sublayer over PC5 hop is only for the purpose of bearer mapping. The SRAP sublayer is not present over PC5 hop for relaying the L2 U2N Remote UE's message on BCCH and PCCH. For L2 U2N Remote UE's message on SRB0, the SRAP header is not present over PC5 hop, but the SRAP header is present over Uu hop for both DL and UL.

Considering above, it should be clarified first when the PC5 SRAP entity is established and whether the SRB0 messages go through PC5 SRAP entity. In our understanding, SRB0 transmission/reception can work with/without SRAP entity. Without SRAP entity, the PDCP entity of SRB0 just passes the RRC message to the PC5 Relay RLC channel which is specified as SL-RLC0; while with SRAP entity, it is the SRAP entity passes the RRC message to SL-RLC0 transparently like in a TM mode. Therefore this is not a technical issue, but the style how to specify the UE behavior. To minimize the potential changes across RAN2 specifications, we can adopt the way captured in TS 38.351, and update TS 38.331 to make alignment.

Proposal 1: SRAP entity is established by Remote UE before SRB0 transmission, and SRB0 messages go through SRAP entity. (No change to TS 38.351, but TS 38.331 needs to be updated.)

2.2 SRAP entity handling for SRB1

According to the current version of TS 38.331[1], Remote UE shall apply the default configuration of SRAP for RRCReestablishment/ RRCResume message receiving during the RRC connection re-establishment/resume procedure since RRCReestablishment/ RRCResume message is the first RRC message from network. When receiving the RRCReestablishment/ RRCResume message, Remote UE’s SRAP entity shall submit the SRAP PDUs with any local identity to the SRB1 PDCP entity.

TS 38.331 #### 5.3.7.4 Actions related to transmission of RRCReestablishmentRequest message The UE shall set the contents of RRCReestablishmentRequest message as follows: … 1> if the UE is acting as L2 U2N Remote UE: 2> establish or re-established (e.g. via release and add) SL RLC entity for SRB1; 2> apply the default configuration of SL-RLC1 as defined in 9.2.4 for SRB1; 2> apply the default configuration of PDCP as defined in 9.2.1 for SRB1; 2> establish the SRAP entity and apply the default configuration of SRAP as defined in 9.2.5 for SRB1; … ### 5.3.13 RRC connection resume #### 5.3.13.2 Initiation … 1> if the UE is acting as L2 U2N Remote UE: 2> apply the default configuration of SL-RLC1 as defined in 9.2.4 for SRB1; 2> apply the default PDCP configuration as defined in 9.2.1 for SRB1; 2> establish the SRAP entity and apply the default configuration of SRAP as defined in 9.2.5 for SRB1; … ### 9.2.5 Default SRAP configurations Parameters that are used for receiption of Remote UE's RRCResume and RRCReestablishmen messages. Name; Value; Semantics description; Ver. SL SRAP Config. > sl-LocalIdentity; Undefined; SRAP PDUs with any local Identity will be submitted to the SRB1 PDCP entity.. > sl-RemoteUE-RB-Identity; SRB1.

However, this SRAP behavior is not explicitly captured in TS 38.351[3]. And according to the error handling in clause 5.4, the Remote UE’s SRAP entity shall discard the received SRAP Data PDU if the local UE ID or bearer ID is not configured, which means Remote UE shall discard the received SRAP Data PDU (i.e. RRCResume and RRCReestablishmen messages) since at that time there is no local UE ID and bearer ID configured. Another issue needs to be clarified is according to clause 5.2.3 the differentiation between SRB and DRB with the same bearer ID is based on sl-Egress-RLC-Channel-PC5, which is not the case for reception of RRCResume and RRCReestablishmen when default configuration is applied.

TS 38.351 ### 5.2.3 Receiving operation of U2N Remote UE Upon receiving an SRAP Data PDU from lower layer, the receiving part of the SRAP entity shall: - if the SRAP Data PDU is not for SRB0 (not received from SL-RLC0 as specified in TS 38.331 [3]): - remove the SRAP header of this SRAP Data PDU and deliver the SRAP SDU to upper layer corresponding to the BEARER ID field of this SRAP Data PDU (SRB and DRB are differentiated based on sl-RemoteUE-RB-Identity associated with the sl-Egress-RLC-Channel-PC5, and for DRB, the upper layer entity for BEARER ID plus 1); - else: - deliver the SRAP SDU (i.e., same as SRAP PDU for SRB0) to upper layer, i.e., RRC layer entity (TS 38.331 [3]). ## … ## 5.4 Handling of unknown, unforeseen, and erroneous protocol data For U2N Remote UE, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which is not included in sl-SRAP-Config-Remote is received, the SRAP entity shall: - discard the received SRAP Data PDU.

Proposal 2:	In TS 38.351 clause 5.4, capture Remote UE receiving RRCResume and RRCReestablishmen as an exceptional case of error handling, i.e. SRAP entity does not discard the RRC messages.

Proposal 3: In TS 38.351 clause 5.2.3, capture SRAP entity does not rely on sl-RemoteUE-RB-Identity to differentiate SRB or DRB when Remote UE receiving RRCResume and RRCReestablishment.

2.3 SRAP header handling for SRB1 message at Relay UE

According to the current version of TS 38.351[2], there are two alternative modes are supported at Relay UE. One alternative is receiving part passes SRAP PDUs to the transmitting part. The other alternative mode is the receiving part passes SRAP SRAP SDUs to the transmitting part, and the receiving part removes the SRAP header while the transmitting part adds the SRAP header. When using the second alternative, for DL data transmission, the Relay UE’s Uu SRAP as receiving part is supposed to remove the header, and the PC5 SRAP as transmitting part is supposed to add the header. Similarly, for UL data transmission, the PC5 SRAP as receiving part removes the header, and the Uu SRAP as transmitting part adds the header.

Observation 1: When Relay UE relays the DL packets, the receiving part removes the SRAP headers, if the passing mode with removing and adding headers is used.

However, the current description for the SRB0 handling at Relay UE in Figure 4.2.2-2 and Figure 4.2.2-3 is as below and it seems to exclude the alternative with receiving part removing header and transmitting part adding header.

- For the UL SRB0 packets, header is added at Uu interface during transmission procedure

- For the DL SRB0 packets, header is removed at PC5 interface during transmission procedure. .

Therefore, clarification is needed of SRB0 handling at Relay UE to ensure the second alternative is feasible and can be implemented correctly.

Proposal 4: Clarify the handling of DL SRB0 can be the receiving part at the Uu interface to remove the SRAP header.

- Conclusion

Based on above discussion, we have the following observations and proposals.

Proposal 1: SRAP entity is established by Remote UE before SRB0 transmission, and SRB0 messages go through SRAP entity. (No change to TS 38.351, but TS 38.331 needs to be updated.)

Proposal 2:	In TS 38.351 clause 5.4, capture Remote UE receiving RRCResume and RRCReestablishmen as an exceptional case of error handling, i.e. SRAP entity does not discard the RRC messages.

Proposal 3: In TS 38.351 clause 5.2.3, capture SRAP entity does not rely on sl-RemoteUE-RB-Identity to differentiate SRB or DRB when Remote UE receiving RRCResume and RRCReestablishment.

Observation 1: When Relay UE relays the DL packets, the receiving part removes the SRAP headers, if the passing mode with removing and adding headers is used.

Proposal 4: Clarify the handling of DL SRB0 can be the receiving part at the Uu interface to remove the SRAP header.

- Reference

[1] 3GPP TS 38.331: "NR Radio Resource Control (RRC); Protocol Specification". V17.1.0.

[2] 3GPP TS 38.351: "NR; Sidelink Relay Adaptation Protocol (SRAP) Specification". V17.1.0.

[3] 3GPP TS 38.300: "NG Radio Access Network; Overall description". V17.1.0.

Appendix 1. Draft CR for TS 38.331

--------------------------------------------------------------Start of change-----------------------------------------------------------------

#### 5.3.3.2	Initiation

The UE initiates the procedure when upper layers request establishment of an RRC connection while the UE is in RRC_IDLE and it has acquired essential system information, or for sidelink communication as specified in clause 5.3.3.1a.

The UE shall ensure having valid and up to date essential system information as specified in clause 5.2.2.2 before initiating this procedure.

Upon initiation of the procedure, the UE shall:

1>	if the upper layers provide an Access Category and one or more Access Identities upon requesting establishment of an RRC connection:

2>	perform the unified access control procedure as specified in 5.3.14 using the Access Category and Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

1>	if the UE is acting as L2 U2N Remote UE:

2>	apply the specified configuration of SL-RLC0 as specified in 9.1.1.4;

2>	apply the SDAP configuration and PDCP configuration as specified in 9.1.1.2 for SRB0;

2>	establish the SRAP entity;

1>	else:

2>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

2>	apply the default MAC Cell Group configuration as specified in 9.2.2;

2>	apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SIB1;

1>	start timer T300;

1>	initiate transmission of the RRCSetupRequest message in accordance with 5.3.3.3;

-------------------------------------------------------------End of change------------------------------------------------------------------

Appendix 2. Draft CR for TS 38.351

-------------------------------------------------------------Start of change-----------------------------------------------------------------

### 4.2.2	SRAP entities

Figure 4.2.2-1 represents one possible structure for the SRAP sublayer. The figure is based on the radio interface protocol architecture defined in TS 38.300 [2].

Figure 4.2.2-1: SRAP structure overview

On the U2N Relay UE, the SRAP sublayer contains one SRAP entity at Uu interface and a separate collocated SRAP entity at the PC5 interface. On the U2N Remote UE, the SRAP sublayer contains only one SRAP entity at the PC5 interface.

Each SRAP entity has a transmitting part and a receiving part. Across the PC5 interface, the transmitting part of the SRAP entity at the U2N Remote UE has a corresponding receiving part of an SRAP entity at the U2N Relay UE, and vice versa. Across the Uu interface, the transmitting part of the SRAP entity at the U2N Relay UE has a corresponding receiving part of an SRAP entity at the gNB, and vice versa.

Figure 4.2.2-2 and Figure 4.2.2-3 represents the functional view of the SRAP entity for the SRAP sublayer at PC5 interface and at Uu interface respectively.

Figure 4.2.2-2: Example of functional view of SRAP sublayer at PC5 interface

Figure 4.2.2-3: Example of functional view of SRAP sublayer at Uu interface

In the example of Figure 4.2.2-2 and Figure 4.2.2-3, at relay UE:

-	The receiving part on the SRAP entity of Uu interface delivers SRAP Data PDUs to the transmitting part on the collocated SRAP entity of PC5 interface, and the receiving part on the SRAP entity of PC5 interface delivers SRAP Data PDUs to the transmitting part on the collocated SRAP entity of Uu interface, except for data packet for SRB0 (i.e., received from SL-RLC0 as specified in TS 38.331 [3]). As an alternative mode, the receiving part may deliver SRAP SDUs to the transmitting part on the collocated SRAP entity. When passing SRAP SDUs, the receiving part removes the SRAP header and the transmitting part adds the SRAP header with the same SRAP header content as carried on the SRAP Data PDU header prior to removal. Passing SRAP SDUs in this manner is therefore functionally equivalent to passing SRAP Data PDUs, in implementation. The following specification therefore refers to the passing of SRAP data packets in supporting the alternative mode.

-	For UL data packet corresponding to SRB0, the receiving part on the SRAP entity of PC5 interface delivers SRAP SDUs to the transmitting part on the collocated SRAP entity of Uu interface, and the transmitting part on the SRAP entity of Uu interface adds the SRAP header in accordance with clause 5.3.3.

-	For DL data packet corresponding to SRB0, the receiving part on the SRAP entity of Uu interface delivers SRAP Data PDUs to the transmitting part on the collocated SRAP entity of PC5 interface, and the transmitting part on the SRAP entity of PC5 interface may remove the SRAP header in accordance with clause 5.2.2. As an alternative, the receiving part on the SRAP entity of Uu interface may remove the SRAP header accordance with clause 5.2.1 and delivers SRAP SDUs to the transmitting part on the collocated SRAP entity of PC5 interface.

--------------------------------------------------------------Next change-----------------------------------------------------------------

## 5.2	DL Data transfer

### 5.2.1	Receiving operation of U2N Relay UE

Upon receiving an SRAP Data PDU from lower layer, the receiving part of the SRAP entity on the Uu interface of U2N Relay UE shall:

-	if the SRAP Data PDU is for SRB0 (the BEARER ID field is 0, and SRB and DRB are differentiated based on sl-Egress-RLC-Channel-Uu):

-	Removes the SRAP header from the SRAP Data PDU;

NOTE: if the header removing operation for SRB0 is done in transmitting part on PC5 interface in 5.2.2, no action is needed in receiving part on Uu interface.

-	deliver the SRAP data packet to the transmitting part of the collocated SRAP entity on the PC5 interface.

### 5.2.2	Transmitting operation of U2N Relay UE

The transmitting part of the SRAP entity on the PC5 interface of U2N Relay UE receives SRAP data packets from the receiving part of the SRAP entity on the Uu interface of the same U2N Relay UE.

When the transmitting part of the SRAP entity on the PC5 interface has an SRAP Data PDU to transmit, the transmitting part of the SRAP entity on the PC5 interface shall:

-	Determine the egress link in accordance with clause 5.2.2.1;

-	Determine the egress RLC channel in accordance with clause 5.2.2.2;

-	if the SRAP Data PDU is for SRB0 (the BEARER ID field is 0, and SRB and DRB are differentiated based on sl-Egress-RLC-Channel-Uu):

-	Removes the SRAP header from the SRAP Data PDU;

NOTE: if the header removing operation for SRB0 is done in receiving part on Uu interface in 5.2.1, no action is needed in transmitting part on PC5 interface.

-	Submit this SRAP Data PDU to the determined egress RLC channel of the determined egress link.

-------------------------------------------------------------Next of change-----------------------------------------------------------------

### 5.2.3	Receiving operation of U2N Remote UE

Upon receiving an SRAP Data PDU from lower layer, the receiving part of the SRAP entity shall:

-	if the SRAP Data PDU is not for SRB0 (not received from SL-RLC0 as specified in TS 38.331 [3]), and the SRAP entity is not applying default configuration:

-	remove the SRAP header of this SRAP Data PDU and deliver the SRAP SDU to upper layer corresponding to the BEARER ID field of this SRAP Data PDU (SRB and DRB are differentiated based on sl-RemoteUE-RB-Identity associated with the sl-Egress-RLC-Channel-PC5, and for DRB, the upper layer entity for BEARER ID plus 1);

-	if the SRAP entity is applying default configuration:

-	remove the SRAP header of this SRAP Data PDU and deliver the SRAP SDU to PDCP entity of SRB1 by ignoring the UE ID filed and BEARER ID field of the this SRAP Data PDU;

-	else:

-	deliver the SRAP SDU (i.e., same as SRAP PDU for SRB0) to upper layer, i.e., RRC layer entity (TS 38.331 [3]).

--------------------------------------------------------------Next change-----------------------------------------------------------------

## 5.4	Handling of unknown, unforeseen, and erroneous protocol data

For U2N Remote UE, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which is not included in sl-SRAP-Config-Remote is received except that the SRAP Data PDU is received from SL-RLC1 and the default SRAP configuration is applied as specified in TS 38.331 [3], the SRAP entity shall:

-	discard the received SRAP Data PDU.

For U2N Relay UE, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which is not included in sl-SRAP-Config-Relay is received except that the SRAP Data PDU from SL-RLC1 as specified in TS 38.331 [3] is the first SRAP Data PDU received from a U2N Remote UE, or when a SRAP Data PDU that contains a UE ID which does not match the concerned sl-LocalIdentity corresponding to sl-L2Identity-Remote of the ingress link is received by U2N Relay UE, the SRAP entity shall:

-	discard the received SRAP Data PDU.

--------------------------------------------------------------End of change-----------------------------------------------------------------