3GPP TSG-RAN WG2 Meeting #119-e	R2-2208499

# Electronic meeting, 17 – 29 August 2022

Agenda item:		8.11.2

Source:	Intel Corporation

Title:	Multicast reception in RRC_INACTIVE

Document for:		Discussion and Decision

# Introduction

Rel-18 NR MBS enhancement WID RP-221458 [1] has the following RAN2-related objectives:

- Specify support of multicast reception by UEs in RRC_INACTIVE state [RAN2, RAN3] - PTM configuration for UEs receiving multicast in RRC_INACTIVE state [RAN2] - Study the impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE. (Seamless/lossless mobility is not required) [RAN2, RAN3] - Specify Uu signalling enhancements to allow a UE to use shared processing for MBS broadcast and unicast reception, i.e., ‎including UE capability and related assistance information reporting regarding simultaneous unicast reception in RRC_CONNECTED and MBS broadcast reception from the same or different operators [RAN2]

In this contribution, we discuss multicast reception in RRC_INACTIVE.

# Discussion

## PTM configuration

During Rel-17 MBS discussion, there were two solution directions to support multicast reception in RRC_INACTIVE:

- Option 1: Parts of the multicast configuration in RRC_CONNECTED are reused for RRC_INACTIVE, e.g. [2][3].

- Option 2: Multicast reception in RRC_INACTIVE reuses broadcast mechanism, e.g. [4][5][6].

Neither of the above solutions were pursued further in Rel-17, according to the conclusion in RAN2#113bis-e meeting: “RAN2 will prioritize Active Multicast support in RRC Connected mode in Rel-17. If time permits Multicast support for RRC Inactive can be considered later (once connected mode Multicast solution, and Broadcast solution has become more mature).”

The main use case for Option 1 is that multicast reception in RRC_CONNECTED is continued in RRC_INACTIVE when the cell is congested. However, if the QoS requirement for a multicast service can be satisfied in RRC_INACTIVE, then it is questionable why those UEs should receive the multicast service in RRC_CONNECTED first and is only sent to RRC_INACTIVE due to congestion given that there are benefits of UE power saving and network spectrum efficiency when UE stays in RRC_INACTIVE.

Observation 1: If the QoS requirement for a multicast service can be satisfied in RRC_INACTIVE, there is no need that UE firstly receives the multicast in RRC_CONNECTED and is switched to RRC_INACTIVE due to congestion.

When discussing Option 1 vs. Option 2, it is helpful to compare Rel-17 NR Multicast and Broadcast, as shown in Table 1 below:

Table 1: Comparison of Rel-17 NR Multicast and Broadcast

|  | Rel-17 Multicast MRB | Rel-17 Broadcast MRB |
| --- | --- | --- |
| Applicable RRC states | RRC_CONNECTED only | All RRC states |
| Configuration | Dedicated RRC signalling | SIB20+MCCH |
| CFR and BWP | CFR is configured within dedicated unicast BWP. | The bandwidth of CFR can be equal to or larger than initial BWP, which is indicated by system information (SIB20). |
| HARQ feedback | Supported | Not supported |
| SPS | Supported | Not supported |
| RNTIs to schedule MTCH | G-RNTI and G-CS-RNTI | G-RNTI |
| PTP leg | Supported | Not supported |

For multicast reception in RRC_INACTIVE, several multicast features in RRC_CONNECTED are not available: HARQ feedback, SPS / G-CS-RNTI, PTP leg. Multicast reception in RRC_INACTIVE is very similar to broadcast reception in nature as broadcast does not have those features either. In addition, with the removal of those multicast features, DRX operation for multicast in RRC_INACTIVE is essentially the same as DRX operation for broadcast (specified in TS 38.321 clause 5.7a) instead of DRX operation for multicast in RRC_CONNECTED (specified in TS 38.321 clause 5.7b). Therefore, multicast reception in RRC_INACTIVE is very similar to broadcast reception.

Observation 2: For a UE in RRC_INACTIVE, multicast MRB reception is very similar to broadcast MRB reception.

When comparing Option 1 and Option 2, it is clear that for Option 1, standardization efforts are needed to specify which multicast configurations in RRC_CONNECTED are applicable to RRC_INACTIVE, while for option 2, the usage of broadcast configuration for multicast reception in RRC_INACTIVE is straightforward.

Observation 3: Option 2 has less standardization efforts compared with Option 1 to support multicast reception in RRC_INACTIVE.

There are also problems for Option 1 regarding PTM configuration assuming the configuration in dedicated signalling received in RRC_CONNECTED is reused for RRC_INACTIVE. Since PTM configuration is only provided in dedicated signalling, UE needs to go to RRC_CONNETED to receive updated PTM configuration, which increases the system load as well as UE power consumption. There are several cases that PTM configuration for multicast needs to be updated:

- Update of the parameters of any PTM configuration.

- Activation of a multicast session which can be received in RRC_INACTIVE.

- Cell reselection to a new cell.

Observation 4: When using Option 1 to support multicast reception in RRC_INACTIVE, Option 1 has the issue of increased system load and UE power consumption when updated PTM configuration is provided as UE needs to switch to RRC_CONNECTED.

Above discussion shows that Option 2 has various advantages over Option 1, therefore it is proposed to adopt Option 2 for PTM configuration.

Proposal 1: For multicast reception in RRC_INACTIVE, PTM configuration is provided via SIB20 + MCCH, similarly as in broadcast reception.

One terminology aspect is that in current specifications, there is tight coupling between multicast and RRC_CONNECTED, as well as broadcast reception and SIB20 + MCCH mechanism. For example, “multicast MRB” is tied to dedicated RRC signaling in many places in TS 38.331 e.g. clause 5.3.5.6.6 and 5.3.5.6.7, the configuration carried in MCCH has the IE name MBSBroadcastConfiguration. RAN2 needs to discuss how to address the terminology issue e.g. adding general clarifications regarding multicast reception in RRC_INACTIVE.

Proposal 2: RAN2 to discuss the terminology aspects in current specification regarding multicast reception in RRC_INACTIVE.

## Mobility and state transition

In Rel-17 MBS, mobility procedure for broadcast MBS reception allows the UE to start or continue receiving MBS service(s) when changing cells. The gNB may indicate in the MCCH the list of neighbour cells providing the same MBS broadcast service which is provided in the serving cell. This allows the UE, e.g. to request unicast reception of the service before moving to a cell not providing the MBS broadcast service(s) using PTM transmission. Relevant ASN.1 signalling is shown below.

MBSBroadcastConfiguration-r17-IEs ::=		SEQUENCE {

mbs-SessionInfoList-r17        MBS-SessionInfoList-r17,

mbs-NeighbourCellList-r17      MBS-NeighbourCellList-r17OPTIONAL,   -- Need S

…

}

MBS-SessionInfoList-r17 ::=   SEQUENCE (SIZE (0..maxNrofMBS-Session-r17)) OF MBS-SessionInfo-r17

MBS-SessionInfo-r17 ::=	          SEQUENCE    {

mbs-SessionId-r17              TMGI-r17,

…

mtch-NeighbourCell-r17        BIT STRING (SIZE(maxNeighCell-MBS-r17))      OPTIONAL,-- NEED R

…

}

MBS-NeighbourCellList-r17 ::=       SEQUENCE (SIZE (0..maxNeighCell-MBS-r17)) OF PCI-ARFCN-r17

The above procedure for broadcast service can also be applicable to multicast services that can be received in RRC_INACTIVE. Since multicast service that can be received in RRC_INACTIVE is signalled in MCCH, the IE mtch-NeighbourCell-r17 can indicate neighbour cells providing the same MBS multicast service (that can be received in RRC_INACTIVE) which is provided in the serving cell.

Proposal 3: Rel-17 broadcast procedure for indicating neighbour cells providing the same MBS service is reused for multicast service that can be received in RRC_INACTIVE.

In SA2 TR 23.700-47, Key Issue #1 Multicast MBS data reception in RRC Inactive state has the following note:

NOTE 2:	RAN WG will determine how the switching for the UEs belonging to MBS session from CM-CONNECTED state to CM-CONNECTED with RRC Inactive state (and vice versa) is performed by the RAN node.

If UE switches between RRC_CONNECTED and RRC_INACTIVE, there is no impact regarding the reception of multicast services that can be received in RRC_INACTIVE, just as broadcast service that can be received in any RRC state.

Proposal 4: Reception of multicast services that can be received in RRC_INACTIVE is not impacted when UE switches between RRC_CONNECTED and RRC_INACTIVE.

# Conclusion

In this contribution, we discuss multicast reception in RRC_INACTIVE. We have the following observations regarding the comparison of options for PTM configuration:

Observation 1: If the QoS requirement for a multicast service can be satisfied in RRC_INACTIVE, there is no need that UE firstly receives the multicast in RRC_CONNECTED and is switched to RRC_INACTIVE due to congestion.

Observation 2: For a UE in RRC_INACTIVE, multicast MRB reception is very similar to broadcast MRB reception.

Observation 3: Option 2 has less standardization efforts compared with Option 1 to support multicast reception in RRC_INACTIVE.

Observation 4: When using Option 1 to support multicast reception in RRC_INACTIVE, Option 1 has the issue of increased system load and UE power consumption when updated PTM configuration is provided as UE needs to switch to RRC_CONNECTED.

We propose the following:

Proposal 1: For multicast reception in RRC_INACTIVE, PTM configuration is provided via SIB20 + MCCH, similarly as in broadcast reception.

Proposal 2: RAN2 to discuss the terminology aspects in current specification regarding multicast reception in RRC_INACTIVE.

Proposal 3: Rel-17 broadcast procedure for indicating neighbour cells providing the same MBS service is reused for multicast service that can be received in RRC_INACTIVE.

Proposal 4: Reception of multicast services that can be received in RRC_INACTIVE is not impacted when UE switches between RRC_CONNECTED and RRC_INACTIVE.

# References

[1] RP-221458, CATT et al, "Revised WID: Enhancements of NR Multicast and Broadcast Services"

[2] R2-2103775, Ericsson et al, "Multicast in Idle and Inactive"

[3] R2-2103907, Huawei et al, "Multicast session reception in RRC INACTIVE"

[4] R2-2102838, Intel, "Discussion on SA2 LS and Multicast Session Activation"

[5] R2-2102716, CATT et al, "Discussion on Multicast in Idle and Inactive Mode"

[6] R2-2102938, Samsung, "NR MBS operation in Idle/Inactive mode"