3GPP TSG-RAN WG2 Meeting #119electronic          	          R2-2208507

Online, August 17 – 29, 2022

Agenda Item:	6.0.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on positioning SRS transmission capability

Document for:	Discussion and decision

Introduction

In this contribution, we discuss about the corrections on positioning SRS transmission capability.

Discussion

According to the latest RAN1 UE features list for Rel-17 NR[1], FG27-15 indicates the positioning SRS transmission capability in RRC_INACTIVE state for initial BWP, which is optional with capability signaling. Besides, there is another FG 27-15a to indicate semi-persistent SRS capability for initial BWP. The FG 27-15a is optional with the prerequisite of FG 27-15. From RAN1 understanding, a UE supporting FG 27-15 may not support FG 27-15a.

However, in current 38.331, the features introduced for FG 27-15 and FG 27-15a are included as mandatory fields within the IE srs-PosResourcesRRC-Inactive-r17. That means FG 27-15 and FG 27-15a have to be signalled together. A UE supporting FG27-15 must indicate support of FG 27-15a. This is not aligned with the intention of RAN1.

From our understanding, FG 27-15 is a basic feature, and FG 27-15a is the additional part of the feature that may not be supported by all UEs. According to the guidelines for UE capability definitions in previous LS [2], RAN2 agrees the principle to define baseline feature bits that covers the minimum parts of a feature and use additional bits for parts that can be used together with the baseline feature.

Observation 1: The principle is to define baseline feature bits that covers the minimum parts of a feature and use additional bits for parts that can be used together with the baseline feature.

Based on above, we propose to add a new optional capability field in BandNR to indicate the features of FG 27-15a. The legacy fields of maxNumOfSemiPersistentSRSposResources-r17 and maxNumOfSemiPersistentSRSposResourcesPerSlot-r17 can be dummied, instead added as mandatory fields in the new capability. The corresponding CRs are in [3][4].

Proposal 1: Add a new optional capability field for FG 27-15a in BandNR and dummy the legacy fields for FG 27-15a within srs-PosResourcesRRC-Inactive-r17.

Besides, in current 38.331, the capability fields for FG 27-15b and FG 27-15c are all defined as optional. To make the feature group workable and usable by the network, it should be clarified in 38.306 that the fields for FG 27-15c should be reported together if supported by UE, and the fields for FG 27-15b should be reported together except for the ones especially indicated as optional according to the NOTEs. The corresponding CR is in [3].

Proposal 2: Clarify that the fields for FG 27-15c/FG 27-15b should be reported together if supported by UE, unless the ones especially indicated as optional in the notes.

Conclusion

In this contribution, we provide our analysis on the capability signalling design for positioning SRS transmission capability. We have the following observation and proposals.

Observation 1: The principle for UE capability definition is to define baseline feature bits that covers the minimum parts of a feature and use additional bits for parts that can be used together with the baseline feature.

Proposal 1: Add a new optional capability field for FG 27-15a in BandNR and dummy the legacy fields for FG 27-15a within srs-PosResourcesRRC-Inactive-r17.

Proposal 2: Clarify that the fields for FG 27-15c/FG 27-15b should be reported together if supported by UE, unless the ones especially indicated as optional in the notes.

References

- R1-2205607, Updated RAN1 UE features list for Rel-17 NR after RAN1 #109-e Week2

- R2-2002378, Guidelines for UE capability definitions

- R2-2208508, Correction on positioning SRS transmission capability, TS 38.306, V17.1.0

- R2-2208509, Correction on positioning SRS transmission capability, TS 38.331, V17.1.0