3GPP TSG-RAN WG2 Meeting #119 electronic	R2-2208529

E-Meeting, 17th – 26th Aug., 2022

Agenda item:	8.4.2.2

Source:	Samsung

Title:	Considerations on the L1/L2 Inter-Cell Mobility

Document for:	Discussion & Decision

# Introduction

According to WID on Further NR mobility enhancements [1], the detail objectives to support L1/L2 inter-cell mobility is provided as below:

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction: - Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3] - Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1] - L1 enhancements for inter-cell beam management, including L1 measurement and reporting, and beam indication [RAN1, RAN2] - Note 1: Early RAN2 involvement is necessary, including the possibility of further clarifying the interaction between this bullet with the previous bullet - Timing Advance management [RAN1, RAN2] - CU-DU interface signaling to support L1/L2 mobility, if needed [RAN3] Note 2: FR2 specific enhancements are not precluded, if any. Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios: - Standalone, CA and NR-DC case with serving cell change within one CG - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected) - Both intra-frequency and inter-frequency - Both FR1 and FR2 - Source and target cells may be synchronized or non-synchronized

In this contribution, we identify the main issues to support L1/L2 inter-cell mobility, and provide the candidate solutions to support L1/L2 inter-cell mobility.

# Discussion

In [2], the main scenarios which meet the above objectives and the target performance enhancements in terms of latency reduction during the L1/L2 inter-cell mobility are provided. In this contribution, we want to identify which feature should be enhanced and the candidate solutions are suggested. Below sections are the required enhancements to support L1/L2 inter-cell mobility from our understanding.

## L1 measurement and report

In Rel-17 feMIMO WI, inter-cell beam management (ICBM) using unified TCI framework is operated with the enhanced L1 measurement and report procedure which enables to measure/report the SSB/CSI-RS resources associated with PCIs different from the PCI of a serving cell. CSI resources to be measured are configured and list of references to SSB resources used for CSI measurement/report in a CSI-RS resource set is provided.

- CSI-SSB-ResourceSet includes the additional PCI info for CSI resource set.

- The number of additional PCI configured for CSI-RS is at most 7 (i.e. servingAadditionalPCIList-r17).

CSI-SSB-ResourceSet ::=             SEQUENCE {

csi-SSB-ResourceSetId               CSI-SSB-ResourceSetId,

csi-SSB-ResourceList                SEQUENCE (SIZE(1..maxNrofCSI-SSB-ResourcePerSet)) OF SSB-Index,

...,

[[

servingAdditionalPCIList-r17           SEQUENCE (SIZE(1..maxNrofCSI-SSB-ResourcePerSet)) OF  ServingAdditionalPCIIndex-r17    OPTIONAL  -- Need R

]]

}

ServingAdditionalPCIIndex-r17  ::=  INTEGER(0..maxNrofAdditionalPCI-r17)

CSI report is configured in ServingCellConfig and following entities are configured:

- Report type (periodic, semi-persistent for PUCCH, semi-persistent for PUSCH, aperiodic)

- Report quantity

- Etc.

We think above enhanced L1 measurement and report structure can be directly applied for the L1/L2 inter-cell mobility because the main functionality would be measure/report the L1 measurement for the cells with PCI different from the serving cell. However, we think RAN2 first needs to consider how to guarantee robustness of L1 measurement to prevent ping-pong during the L1/L2 inter-cell mobility. Different from the legacy HO procedure (operated with the L3 measurement), L1/L2 inter-cell mobility (i.e. HO) operated with the L1 measurement so robustness of channel quality toward the cells with PCI different from the serving cell is not guaranteed.

We think following candidate solutions would be considered as an enhancement of L1 measurement and report procedure for L1/L2 inter-cell mobility.

- Option 1: Network implementation solution based on L1 measurement report from UE

- Keep the Rel-17 L1 measurement/report as is.

- NW determines the L1/L2 inter-cell mobility considering at least one or more factors below:

- The reported L1 measurement of the target cell is huge enough during the static time (e.g. timer).

- The number of reported L1 measurement(s) (e.g. counter) of the target cell better than the serving cell’s beam is exceed the static value.

- NW derives the cell quality using the own method based on the report L1 measurement, and apply the same logic of a) and b) above.

- Option 2: UE indicates the additional information in L1 measurement report

- UE additionally reports least one or more factors below:

- Counter value: How many CSI resources of the target cell’s beam are better than the serving cell’s beam if the reported L1 measurement is better than the serving cell’s beam.

- Timer value: How long CSI resources of the target cell’s beam are better than the serving cell’s beam if the reported L1 measurement is better than the serving cell’s beam.

- Indicator: whether to meet the pre-configured robustness of L1 measurement (e.g. using counter or time metric)

- Option 3: Introduce the new event-based L1 measurement report procedure

- New trigger for L1 report, for example:

- N beams of neighbor cell are above threshold, N can be configured. If N not configured, best beam of neighbor cell is above threshold

- N beams of neighbor cell are better than serving cell’s best beam by a threshold, N can be configured. If N not configured, best beam of neighbor cell is better than serving cell’s best beam.

- Filtering can be applied by L1 measurements. Filtering parameters can be configured.

Proposal 1: RAN2 detemine the enhancement of L1 measurement/report for L1/L2 inter-cell mobility considering how to guarantee robustness of L1 measurement to prevent ping-pong during the inter-cell mobility.

## Configuration and maintenance for multiple candidate cells

Similar with the conditional HO, we think the pre-configured candidate cells can be applied to L1/L2 inter-cell mobility. The expected high-level framework would be as below:

- Pre-configuration of the cells with PCI different from the serving cell

- L1/L2 signaling for inter-cell mobility based on L1 measurement

- Apply the target cell’s pre-configured configuration.

- (Optionally) random access to the target cell for UL timing advance

Therefore, how to model/signal these candidate neighbor cell’s configuration is required. General principle for modeling of pre-configuration is that the number of candidate cells are aligned with the number of PCIs which are referred by unified TCI state. Following candidate modeling would be considered.

- Option 1: Pool of candidate neighbor cell’s configuration is provided in CG-level

- Some reasonable number of the candidate cell configurations is provided in cell group configuration (e.g. 7 candidate cells), and this pool is used for all serving cells (i.e. same pool is applied to all serving cells)

- Huge enough number of candidate cell’s configuration is provided in CG-Config (e.g. 16 candidate cells), and each serving cell indicate the linking information in the ServingCellConfig.

- Option 2: Pool of candidate neighbor cell’s configuration is provided in serving cell level

- Each serving cell can provide at most 7 additional cells (servingAadditionalPCIList-r17)

- This approach is consistent with the modeling of Rel-17 TCI state framework (i.e. each serving cell can provide at most 7 additional PCIs for reference of TCI state)

- However, it requires too many cell configurations in case of CA and DC

Proposal 2: RAN2 detemine the pre-configuration modeling of multiple candidate cells for L1/L2 inter-cell mobility.

## L1/L2 signalling for inter-cell mobility

As a dynamic switch mechanism among candidate serving cells, L1/L2 signaling for inter-cell mobility is required. The basic assumption is that similar signaling mechanism of L1/L2 inter-cell beam management (i.e. unified TCI state activation/deactivation MAC CE and DCI indication) can be used.

- MAC CE is used to indicate the L2 mobility (i.e. indicate a TCI state for L2 mobility); or

- MAC CE is used to indicate the candidate beams (activation/deactivation) and DCI indicates the L1 mobility among the candidate beams

At least, additional information is needed to differentiate L1/L2 inter-cell mobility from the inter-cell beam management. For example following information is required in the (new or legacy) MAC CE.

- Indicates whether the inter-cell mobility (i.e. HO) is indicated or not.

- Indicates whether RACH procedure is required or not.

- Confirmation signaling (e.g. MAC CE) regarding the MAC CE signaling for L1/L2 inter-cell mobility.

Proposal 3: Additional information is needed to differentiate L1/L2 inter-cell mobility from the inter-cell beam management.

# Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Proposal 1: RAN2 detemine the enhancement of L1 measurement/report for L1/L2 inter-cell mobility considering how to guarantee robustness of L1 measurement to prevent ping-pong during the inter-cell mobility.

Proposal 2: RAN2 detemine the pre-configuration modeling of multiple candidate cells for L1/L2 inter-cell mobility.

Proposal 3: Additional information is needed to differentiate L1/L2 inter-cell mobility from the inter-cell beam management.

- Indicates whether the inter-cell mobility (i.e. HO) is indicated or not.

- Indicates whether RACH procedure is required or not.

- Confirmation signaling (e.g. MAC CE) regarding the MAC CE signaling for L1/L2 inter-cell mobility.

# References

RP-213565	New WID on Further NR mobility enhancements	MediaTek.

R2-2208528	Scenario and Target Performance Enhancements for L1/L2 mobility	Samsung.