3GPP TSG-RAN WG2 Meeting #119-e	R2-2208539

# 17-26 August 2022

# Online

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0541 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 38.300 on SHR and RACH optimization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The stage 2 description for Successful Handover report is missing. - The listed RA content in RACH optimization is incomplete since RA report has been enhanced to include 2 step RA information Also the reference on how UE stores in RA report is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce stage 2 description for Successful Handover report - Add in 15.5.3 RACH optimization use case that RA content includes 2 step RA information as specified in 38331. - Inter-operability analysis - There is no inter-operability issue forseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Stage 2 description for Successful Handover report is missing and RA report content is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.5.2.x, 15.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>> START OF CHANGE <<<

15.5.2.x Successful Handover Report

For detection of a sub-optimal successful handover, the UE stores Successful Handover Report based on configuration by network if received, and makes the Successful Handover Report available to the network specified in TS 38.331[12].

The UE stores the latest Successful Handover Report until the Successful Handover Report is fetched by the network or for 48 hours after the Successful Handover Report is stored.

The UE only indicates Successful Handover Report’s availability and only provides the Successful Handover Report to the network if the current RPLMN is a PLMN that was present in the UE's EPLMN List or was the RPLMN at the time the successful handover was detected.

Upon reception of a Successful Handover Report, the receiving node is able to analyse whether its mobility configuration needs adjustment. Such adjustments may result in changes of mobility configurations, such as changes of RLM configurations or changes of mobility thresholds between the source and the target. In addition, target NG RAN node, in the performed handover, may further optimize the dedicated RACH-beam resources based on the beam measurements reported upon successful handovers.

A Successful Handover Report may also be sent to the node last serving the UE when the NG-RAN node fetches the Successful Handover Report from UE by triggering:

-	The Failure Indication procedure over Xn;

-	The Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

>>> NEXT OF CHANGE <<<

15.5.3	Support for RACH Optimization

RACH optimization is supported by UE reported information made available at the NG RAN node as specified in TS 38.331[12] and by PRACH parameters exchange between NG RAN nodes.

The contents of the RACH information report comprise of the following:

-	Contention detection indication per RACH attempt;

-	Indexes of the SSBs and number of RACH preambles sent on each tried SSB listed in chronological order of attempts;

-	Indication whether the selected SSB is above or below the rsrp-ThresholdSSB threshold per RACH attempt.

-  2 step RACH information as specified in subclause 5.7.10.4 of TS 38.331[12]

>>> END OF CHANGE <<<