3GPP TSG-RAN WG2 Meeting #119 Electronic	R2-2208560

Elbonia, 17 – 26 August 2022

Agenda item:	6.10.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	On issues for Timing Advance Report MAC CE

WID/SID:	NR_NTN_solutions-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

Though the timing advance report MAC CE was heavily discussed in RAN2-118e meeting, there are two open issues left to be further discussed in RAN2-119e.

R2-2206612 [offline-104] UP corrections – fourth round Interdigital discussion Rel-17 NR_NTN_solutions-Core Proposal 1: RAN2 to further discuss if any additional mechanism is needed to address the possibility of outdated UE TA info at the NW in RAN2#119e. - RAN2 to further discuss if any additional mechanism is needed to address the possibility of outdated UE TA info at the NW in RAN2#119e Proposal 2: RAN2 to further discuss support for dedicated SR configuration for TAR MAC CE in RAN2#119e. - RAN2 to further discuss support for dedicated SR configuration for TAR MAC CE in RAN2#119e

In this contribution, we would like to discuss the issue caused by outdated TA information at the network and then provide some options to address it. Furthermore, we also provide our view on SR configuration which should be used for SR triggered by TA MAC CE.

# 2	Discussion

## 2.1	Outdated UE TA information at network

The issue of outdated UE TA information at network was raised at RAN2-118e meeting. The issue may happen when the Timing Advance report was triggered and transmitted to NW via UL HARQ mode B, but it was not successfully received by the NW.

According to current MAC specification, the trigger of TA MAC CE will be cancelled when the MAC CE is transmitted in a MAC PDU no matter in UL HARQ mode A or B. However, for UL HARQ mode B, the gNB may not decode the MAC PDU correctly since HARQ state B best supports no UL retransmission and/or blind UL retransmission (where the retransmission is not based on UL decoding result). Different from data transmission where the RLC retransmission mechanism may be triggered for transmission robustness, there is no upper layer retransmission for MAC CE. If the trigger event has been cancelled, there is no chance for TA report MAC CE to be retransmitted which means NW will fail to get the latest TA. Furthermore, a new TA report may not be triggered again since the variation between latest TA information and the last reported information is less than offsetThresholdTA, though the last reported information was not successful received at all. The case will cause the UE TA information at network become outdated while UE has no chance to update it in time.

The outdated TA information status may last for a long time in NW until another TA report can be triggered when the TA variation (compared with the unsuccessful TA delivery) is equal or larger than TA offset.

38.321 V17.1.0: A Timing Advance report (TAR) shall be triggered if any of the following events occur: … - if the variation between current information about Timing Advance and the last reported information about Timing Advance is equal to or larger than offsetThresholdTA, if configured. … All triggered Timing Advance reports shall be cancelled when a MAC PDU is transmitted and this PDU includes the corresponding Timing Advance Report MAC CE.

Observation 1: The unsuccessful delivery on UE TA information may cause issue of outdated UE TA in NW, which may last for a long time.

As NW cannot know whether the UE TA report MAC CE failed in UL transmission hence don’t know whether the UE’s TA information is outdated. NW may continue scheduling the UE based on the outdated TA and scheduling UE with unsuitable K_offset configuration and K2. However, the scheduling with wrong (K_offset + K2) may cause UE cannot perform UL transmission if UE’s actual TA is larger than the NW scheduled (K_offset+K2), which will result in NW scheduling failure.

The scenario can be illustrated in below Figure1 where UE cannot transmit PUSCH in slot X+2. The UE cannot transmit PUSCH since UE’s actual/self-estimated TA is larger than (K2+K_offset) and UE cannot transmit signal before it receives the corresponding DL scheduling information (due to loss of causality – the UE cannot be expected to transmit information prior to obtaining instructions to transmit said information).

Figure 1: PUSCH transmission failure due to (K2+K_offset) less than UE specific TA

Observation 2: NW is not aware whether the UE’s TA is outdated and whether the TA MAC CE transmission is failed, which may cause NW schedule UE with unsuitable K_offset and K2.

Observation 3: Outdated UE TA information in NW may cause UE cannot perform PUSCH transmission which will result in NW UL scheduling failure.

To address the issue of outdated UE TA information in NW, there may need some enhancements to mitigate the impact to NW scheduling.

- Option1: NW trigger PDCCH order to request UE TA report

- Option2: UE indicates to NW if it cannot perform UL transmission due to unsuitable uplink transmission time offset

For Option1, since NW is not aware whether the UL TA report transmission is failed or even whether the UE’s TA report is triggered, NW have to guess whether a TA Report MAC CE update is needed hence triggering PDCCH order to request a new TA report. The typical scenario is that, when gNB detects UL transmissions could not be decoded for a HARQ process, it may guess the TA Report MAC CE may be failed and then blindly adjust K_offset or use higher K2 and request a new TA report. However, there are many factors which may cause the UL transmission failure (e.g., DL radio channel condition which may cause UE to miss the UL grant, or UL radio channel conditions which may cause UL transmission failure, or UE’s power head room is too limited to support the scheduled PRB/MCS etc.). gNB cannot exactly tell the UL transmission failure is caused by failed TA Report MAC CE transmission hence it can only blindly adjust K_offset/K2 and request UE TA report via PDCCH order, which may cause PDCCH resource waste. In other words, the NW triggered PDCCH order to request UE TA report cannot efficiently address the outdated TA issue.

Observation 4: NW triggered PDCCH order to request UE TA report cannot efficiently address the outdated TA issue since it may waste PDCCH resource.

Instead, UE exactly knows whether the uplink transmission time offset (e.g., K_offset + K2) is larger than the UE actual TA hence decides the UL PUSCH transmission can be performed or not.

- If the uplink transmission time offset (e.g. K_offset + K2) is larger than or equal to the UE self-estimated TA, UE can perform UL transmission as legacy.

- Otherwise (if the uplink transmission time offset is less than the UE self-estimated TA), the UE should not perform UL transmission as explained in Figure1. In this case, UE may need to indicate to NW its UL transmission failure is caused by unsuitable uplink transmission time offset in scheduling (e.g. K_offset+K2). Hence NW can adjust K_offset and K2 accordingly.

Proposal 1: To mitigate the impact caused by outdated TA information in NW, UE may indicate to NW its UL transmission failure is caused by unsuitable uplink transmission time offset in scheduling (e.g., K_offset+K2)

In our understanding, the K_offset is only applied as additional delay for transmission that are scheduled by NW. For PUCCH transmission, UE will follow the UE-specific TA for SR transmission which is not impacted by K_offset. Hence the UE can send the SR to NW even the K_offset and UE UL TA is outdated in NW.

Observation 5: The SR can be sent by UE even outdated UE TA issue happened in NW.

Based on that, if a dedicated SR configuration for the purpose of PUSCH transmission failure caused by unsuitable uplink transmission time offset (K_offset + K2) can be defined by NW, UE may trigger a dedicated SR to inform NW. Then NW can adjust K_offset and K2 accordingly.

Proposal 2: UE should trigger SR to inform NW that the K_offset and K2 configured for UL grant are not suitable.

Proposal 3: To address outdated TA issue in NW, a dedicated SR configuration should be introduced for the purpose of PUSCH transmission failure caused by unsuitable uplink transmission time offset.

Another way to avoid the outdated TA information in NW is to improve the TA MAC CE report transmission robustness.

For example, the issue can be mitigated if the TA MAC CE is transmitted in UL HARQ mode A which may improve the transmission robustness since the scheduling of UL retransmission grant is based on UL decoding result. However, RAN2 previously agreed the new LCP restriction is not applied to at least existing TA MAC CE.

Agreement: - No new LCP restrictions are introduced for existing UL MAC CEs (if new MAC CEs will be introduced, we can revisit this)

Furthermore, if the MAC CE has to be transmitted via HARQ mode A, it may bring extra restriction to NW implementation to always need to configure/schedule a HARQ mode A process even NW has no idea when the TA MAC CE will be triggered. Otherwise, the UL TA MAC CE report delay will be extremely extended.

Observation 6: Apply new LCP restriction to limit UL TA MAC CE report via UL HARQ mode A may bring extra restriction to NW implementation.

Another possible way-forward to mitigate the issue is to introduce a DL MAC CE for reception confirmation. Even the UL TA MAC CE is transmitted in HARQ mode B, UE may send the MAC CE multiple times before it receives the DL MAC CE for reception confirmation.

Proposal 4: RAN2 to discuss whether to introduce a DL MAC CE to confirm the reception of UL TA MAC CE or to apply the new LCP restriction to limit the transmission of UL TA MAC CE through HARQ mode A.

## 2.2	SR configuration for TA report MAC CE

RAN2 has reached below agreements on SR configuration for TA report MAC CE in RAN2-118e meeting.

- If a dedicated SR configuration for TAR MAC CE is not introduced and timingAdvanceSR is configured with value enabled, UE selects among any available SR configuration - If a dedicated SR configuration for TAR MAC CE is supported, UE shall only use it when timingadvanceSR is enabled.

The focus is whether a dedicated SR configuration should be introduced for TAR MAC CE. There are four options proposed in the discussion as indicated in [1].

- Option1: Support a dedicated SR configuration for TAR MAC CE only

- Option2: Network may optionally provide a dedicated SR configuration. If timingadvanceSR is enabled and dedicated configuration is provided UE uses it. Else, select among available configurations

- Option3: Dedicated SR configuration is not supported.

- Option4: Network may optionally provide a dedicated SR configuration. If timingadvanceSR is enabled and dedicated configuration is provided UE uses it. Else (if timingadvanceSR is enabled and dedicated configuration is not provided), UE triggers RACH.

Configuring or not configuring a dedicated SR configuration for TA MAC CE has different pros and cons. For example, a dedicated SR configuration for TA MAC CE report can facilitate the gNB to prioritize the TA MAC CE scheduling among different logical channels. This is helpful to have an in-time TA reporting to NW. On the other hand, if there is no dedicated SR configuration for TA MAC CE report, based on RAN2 agreement, any SR configuration may be used for an SR triggered by TA MAC CE report.  Since TA MAC CE has higher priority over data from LCHs and most of the other MAC CE in LCP procedure, TA MAC CE can be sent at any UL grant which may not bring extra reporting delay.

In our view, either Option#1 (i.e., support dedicated SR configuration) or Option#3 (i.e., not support dedicated SR configuration) can work. However, considering the dedicated SR configuration is beneficial to address the outdated TA information issue as discussed in section2.1, we prefer to support dedicated SR configuration for TA MAC CE report.

Proposal 5: Dedicated SR configuration is supported for TA MAC CE report.

# 3	Conclusion

This document has made the following observations:

Observation 1: The unsuccessful delivery on UE TA information may cause issue of outdated UE TA in NW, which may last for a long time.

Observation 2: NW is not aware whether the UE’s TA is outdated and whether the TA MAC CE transmission is failed, which may cause NW schedule UE with unsuitable K_offset and K2.

Observation 3: Outdated UE TA information in NW may cause UE cannot perform PUSCH transmission which will result in NW UL scheduling failure.

Observation 4: NW triggered PDCCH order to request UE TA report cannot efficiently address the outdated TA issue since it may waste PDCCH resource.

Observation 5: The SR can be sent by UE even outdated UE TA issue happened in NW.

Observation 6: Apply new LCP restriction to limit UL TA MAC CE report via UL HARQ mode A may bring extra restriction to NW implementation.

And proposed the following:

Proposal 1: To mitigate the impact caused by outdated TA information in NW, UE may indicate to NW its UL transmission failure is caused by unsuitable uplink transmission time offset in scheduling (e.g., K_offset+K2)

Proposal 2: UE should trigger SR to inform NW that the K_offset and K2 configured for UL grant are not suitable.

Proposal 3: To address outdated TA issue in NW, a dedicated SR configuration should be introduced for the purpose of PUSCH transmission failure caused by unsuitable uplink transmission time offset.

Proposal 4: RAN2 to discuss whether to introduce a DL MAC CE to confirm the reception of UL TA MAC CE or to apply the new LCP restriction to limit the transmission of UL TA MAC CE through HARQ mode A.

Proposal 5: Dedicated SR configuration is supported for TA MAC CE report.

# Reference

[1] R2-2206612 "Report of [AT118-e][104][NTN] UP Corrections: Phase 3", Interdigital