3GPP TSG RAN2 Meeting #119-e	R2-2208587

# Electronic, 17th – 29th May, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.323 | CR | Draft | rev |  | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on UDC packet |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UDC-Core |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current PDCP specification, the definition of “UDC packet, i.e. UDC header and UDC data block” is not clear when PDCP PDU is generated, given that "UDC header and UDC data block" is used instead of "Data" in PDU format in Section B.2.1. From the functional point of view, NR UDC is almost identical as LTE UDC and there is no big difference but PDCP specifications capture differently. In LTE PDCP specification (TS 36.323), it is clearly specified that “UDC header and UDC Data Block if UDC is configured.” whereas NR PDCP does not have such a sentence. This difference makes a confusion that NR UDC has different behavior on UDC data block. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To clarify “UDC header and UDC data block” as Data in Section 6.3.3. Impact analysis Impacted functionality: UDC Inter-operability: No inter-operability issue is foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclarity remains in PDCP specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

### 6.3.3	Data

Length: Variable

This field includes one of the followings:

-	Uncompressed PDCP SDU (user plane data, or control plane data);

-	Compressed PDCP SDU (user plane data only);

-	UDC header and UDC Data Block if UDC is configured.

NOTE:	All fields other than PDCP PDU header and MAC-I belong to Data field.‎

<End of modification>