3GPP TSG-RAN WG2 Meeting # 119-e	R2-2208596

# e-Meeting, 17 – 29 August 2022

Agenda item:	6.6.1

Source:	Google

Title:	Discussion on RRC IEs in the RAN3 specification

Document for:	Discussion & Decision

- Introduction

In this paper, we discuss the issues raised in the RAN3 LS “LS on transferring SDT configuration and SRS positioning Inactive configuration from DU to CU”. A draft reply LS is attached in the appendix.

- Discussion

In RAN3#116-e, it was identified that “SDT-MACPHY-Config” that is referred to in the RAN3 specification 38.473 does not actually exist in TS 38.331 as shown below.

//38.473 v17.1.0 9.3.1.26 DU to CU RRC Information This IE contains the RRC Information that are sent from the gNB-DU to the gNB-CU. IE/Group Name; Presence; Range; IE type and reference; Semantics description. CellGroupConfig; M; OCTET STRING; CellGroupConfig, as defined in TS 38.331 [8].. …. SDT-MACPHY-Config; O; OCTET STRING; SDT-MACPHY-Config, as defined in TS 38.331 [8]..

During the RAN3#116-e discussion, it was argued that whether the SDT-CG-Config-r17 or the SDT-MAC-PHY-CG-Config-r17 in 38.331 (see the ASN.1 structure below) should be the one to be referred to in the semantics in the RAN3 specification. As the SRS Positioning INACTIVE configuration also needs to be generated by gNB-DU, a similar question is raised for the SRS-PosRRC-Inactive-r17 and the SRS-PosRRC-InactiveConfig-r17 in the RAN3 LS.

SuspendConfig ::=                   SEQUENCE {

fullI-RNTI                          I-RNTI-Value,

shortI-RNTI                         ShortI-RNTI-Value,

ran-PagingCycle                     PagingCycle,

ran-NotificationAreaInfo            RAN-NotificationAreaInfo                                            OPTIONAL,   -- Need M

t380                                PeriodicRNAU-TimerValue                                             OPTIONAL,   -- Need R

nextHopChainingCount                NextHopChainingCount,

...,

[[

sl-UEIdentityRemote-r17             RNTI-Value                                                          OPTIONAL, -- Cond L2RemoteUE

sdt-Config-r17                      SetupRelease { SDT-Config-r17 }                                     OPTIONAL,   -- Need M

srs-PosRRC-Inactive-r17             SetupRelease { SRS-PosRRC-Inactive-r17 }                            OPTIONAL,   -- Need M

ran-ExtendedPagingCycle-r17         ExtendedPagingCycle-r17                                             OPTIONAL    -- Cond RANPaging

]]

}

<Unrelated part omitted>

SDT-Config-r17 ::=                  SEQUENCE {

sdt-DRB-List-r17                    SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity                         OPTIONAL,   -- Need M

sdt-SRB2-Indication-r17             ENUMERATED {allowed}                                                OPTIONAL,   -- Need R

sdt-MAC-PHY-CG-Config-r17           SetupRelease {SDT-CG-Config-r17}                                    OPTIONAL,   -- Need M

sdt-DRB-ContinueROHC-r17            ENUMERATED { cell, rna }                                            OPTIONAL    -- Need S

}

SDT-CG-Config-r17 ::= OCTET STRING (CONTAINING SDT-MAC-PHY-CG-Config-r17)

SDT-MAC-PHY-CG-Config-r17 ::=       SEQUENCE {

-- CG-SDT specific configuration

cg-SDT-ConfigLCH-RestrictionToAddModList-r17 SEQUENCE (SIZE(1..maxLC-ID)) OF  CG-SDT-ConfigLCH-Restriction-r17 OPTIONAL,   -- Need N

cg-SDT-ConfigLCH-RestrictionToReleaseList-r17 SEQUENCE (SIZE(1..maxLC-ID)) OF  LogicalChannelIdentity  OPTIONAL,   -- Need N

cg-SDT-ConfigInitialBWP-NUL-r17       SetupRelease {BWP-UplinkDedicatedSDT-r17}                     OPTIONAL,   -- Need M

cg-SDT-ConfigInitialBWP-SUL-r17       SetupRelease {BWP-UplinkDedicatedSDT-r17}                     OPTIONAL,   -- Need M

cg-SDT-ConfigInitialBWP-DL-r17        BWP-DownlinkDedicatedSDT-r17                                  OPTIONAL,   -- Need M

cg-SDT-TimeAlignmentTimer-r17           TimeAlignmentTimer                                              OPTIONAL,   -- Need M

cg-SDT-RSRP-ThresholdSSB-r17            RSRP-Range                                                      OPTIONAL,   -- Need M

cg-SDT-TA-ValidationConfig-r17          SetupRelease { CG-SDT-TA-ValidationConfig-r17 }                 OPTIONAL,   -- Need M

cg-SDT-CS-RNTI-r17                      RNTI-Value                                                      OPTIONAL,   -- Need M

...

}

SRS-PosRRC-Inactive-r17 ::= OCTET STRING (CONTAINING SRS-PosRRC-InactiveConfig-r17)

SRS-PosRRC-InactiveConfig-r17 ::=       SEQUENCE {

srs-PosConfigNUL-r17                SRS-PosConfig-r17                                                   OPTIONAL,    -- Need R

srs-PosConfigSUL-r17                SRS-PosConfig-r17                                                   OPTIONAL,    -- Need R

bwp-NUL-r17                         BWP                                                                 OPTIONAL,    -- Need S

bwp-SUL-r17                         BWP                                                                 OPTIONAL,    -- Need S

inactivePosSRS-TimeAlignmentTimer-r17  TimeAlignmentTimer                                                  OPTIONAL,    -- Need M

inactivePosSRS-RSRP-changeThreshold-r17RSRP-ChangeThreshold-r17                                            OPTIONAL     -- Need M

}

Generally speacking, an IE is defined as an “OCTET STRING” because the IE can be received by RAN node 1 (e.g., CU or the source base station) from RAN node 2 (e.g., DU or the target base station).  RAN node 1 directly includes the received IE in the OCTET STRING.  When the ASN.1 encoder of RAN node 1 processes the parent IE or the message including the IE, the encoder bypasses the OCTET STRING without any encoding processing.  Therefore, using “OCTET STRING” for an IE has a merit of saving a RAN node’s effort on ASN.1 processing.

Take the CellGroupConfig as an example. As shown below, the CellGroupConfig is defined as an OCTET STRING in the RRCReconfiguration-IEs. When the CU receives a DU-encoded CellGroupConfig from the DU, the CU just includes the DU-encoded CellGroupConfig in the “OCTET STRING” in the RRCReconfiguration-IEs. Thus, the CU does not need to decode the DU-encoded CellGroupConfig and then encode the decoded CellGroupConfig to a CU-encoded CellGroupConfig.

RRCReconfiguration-IEs ::=              SEQUENCE {

radioBearerConfig                   RadioBearerConfig                                                      OPTIONAL, -- Need M

secondaryCellGroup                  OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Cond SCG

measConfig                          MeasConfig                                                             OPTIONAL, -- Need M

lateNonCriticalExtension            OCTET STRING                                                           OPTIONAL,

nonCriticalExtension                RRCReconfiguration-v1530-IEs                                           OPTIONAL

}

RRCReconfiguration-v1530-IEs ::=        SEQUENCE {

masterCellGroup                     OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Need M

fullConfig                          ENUMERATED {true}                                                      OPTIONAL, -- Cond FullConfig

dedicatedNAS-MessageList            SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message                     OPTIONAL, -- Cond nonHO

masterKeyUpdate                     MasterKeyUpdate                                                        OPTIONAL, -- Cond MasterKeyChange

dedicatedSIB1-Delivery              OCTET STRING (CONTAINING SIB1)                                         OPTIONAL, -- Need N

dedicatedSystemInformationDelivery  OCTET STRING (CONTAINING SystemInformation)                            OPTIONAL, -- Need N

otherConfig                         OtherConfig                                                            OPTIONAL, -- Need M

nonCriticalExtension                RRCReconfiguration-v1540-IEs                                           OPTIONAL

}

The SDT-MAC-PHY-CG-Config-r17 is also defined as an OCTET STRING in the SDT-CG-Config-r17, similar to the CellGroupConfig case. With the same reasoning as described for the CellGroupConfig case, the SDT-MAC-PHY-CG-Config-r17 should be referred to in the RAN3 specification instead of the SDT-CG-Config-r17.  If the SDT-CG-Config-r17 is generated by the DU, the CU must decode the DU-generated SDT-CG-Config-r17 and then generate a SDT-CG-Config-r17 because the SDT-CG-Config-r17 is not defined as an OCTET STRING.

Similarly, the SRS-PosRRC-InactiveConfig-r17 is defined as an OCTET STRING in the SRS-PosRRC-Inactive-r17. With the same reasoning as described for the CellGroupConfig case, the SRS-PosRRC-InactiveConfig-r17 should be referred to in the RAN3 specification instead of the SRS-PosRRC-Inactive-r17.

Proposal: RAN2 to reply RAN3 that the SDT-MAC-PHY-CG-Config-r17 and SRS-PosRRC-InactiveConfig-r17 are the ones to be referred to in the RAN3 specification.

- Appendix: Draft Reply LS to RAN3

3GPP TSG RAN WG2#119-e	R2-22XXXXX

e-Meeting, 17 - 29 August, 2022

# Title:	[Draft] Reply LS on transferring SDT configuration and SRS positioning Inactive configuration from DU to CU

# Response to:	-

# Release:	Release 17

# Work Item:	NR_SmallData_INACTIVE-Core, NR_pos_enh

Source:	Google (to be RAN2)

To:	RAN3

Cc:

Contact Person:

Name:	-

Tel. Number:	-

E-mail Address:	-

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

RAN2 thanks RAN3 for the LS on transferring SDT configuration and SRS positioning Inactive configuration from DU to CU and would like to provide the following information for RAN3 consideration.

SDT-MAC-PHY-CG-Config-r17 and SRS-PosRRC-InactiveConfig-r17 are the ones to be referred to in the RAN3 specification.

2. Actions:

To RAN3 group.

ACTION: 	RAN2 respectfully asks RAN3 to take decision in the above aspects and provide feedback.

3. Date of Next RAN WG3 Meetings:

RAN2 #119bis-e                   10 – 19 October 2022	Electronic Meeting

RAN2 Meeting #120	14 – 18 November 2022 	Canada